BLOCK I APOLLO GUIDANCE COMPUTER REPLICA SCHEMATICS

Rev. A0

FOREWORD

The AGC schematics that you now have stored in your hard drive are, in their present form, the brainchild of John Pultorak. Before getting into the details of the changes/corrections I’ve made I’d like to honor him for his incredible job on reviving a long forgotten (but ultimately important) piece of space history. His work has put the foundations upon which we can create bigger and better things such as a physical Block II AGC or a complete working hardware CM panel. I have some ideas I’m working on with Ron Burkey. Hopefully others will follow.

Thanks again John for making this happen!!!

January 2, 2008

Dimitris Vitoris

Athens, Greece

GENERAL REMARKS

- EAGLE does not support direct connect of nets originating from different busses. Even though I tried applying the ALIAS method, mentioned in the documentation, I couldn’t make it to work.

- In many cases in John’s schematics, the pin numbers of device do not correspond to reality. This is auto generated/corrected by EAGLE but leads to pin number mismatch between my schematics and John’s. For example in TIME PULSE GENERATOR #1 device U43D has in John’s schematics the pin numbers 12,11,13  , in his PDF’s has 1,2,3 while the correct ones are 11,12,13.

- Some devices do not exist in EAGLE’s parts library. Whenever possible, pin compatible devices with similar function are used. In the rest of the cases generic DIL packages are used. All the substitutes have the original part’s name value to avoid confusion.

- In these Rev. A0 schematics the system block naming scheme has changed. So:

CTL     =    Driver Module

PROC =    Logic Module

IO        =    Interface Module

MEM   =    Memory module

CHANGELOG

DRIVER MODULE :

- In AGC MONITOR #3 Power up reset R22 (47K). Name used elsewhere. Renamed R87.

- In AGC MONITOR #5 Bus 121_SEQ_MONITOR_BUS not implemented. Direct connect due to EAGLE being unable to connect nets from different busses.

- In AGC MONITOR #6 TPG_DECODED_STATE_1 directly connected with U20B due to EAGLE being unable to connect nets from different busses (BUS 923 and BUS STBY).

- In AGC CLOCK #1 instead of CD4001 (not in EAGLE library) 74LS33N is used. Same function but not exactly pin compatible so used gates U30C and U30B in the place of U30A and U30B.

- In AGC CLOCK #2 all 74LS00N gates and 2.2K resistors doesn’t have a name. Named A18A, A18B, A18C and R88, R89. Also near U201 capacitor C1 name already exists so renamed both capacitors CA and CB. Potentiometer 10K has no name. Named R86.

- In TIME PULSE GENERATOR #1 bus 921_TPG_INPUT not implemented. Direct connect due to EAGLE being unable to connect nets from different busses. Busses TPG_Q0 through TPG_Q3 not going anywhere. Those are the same as TPG_COUNT_1 through TPG_COUNT_4. Not implemented.

- In SEQUENCE GENERATOR #1 U51A pin 2 and U48D pin 12 are connected directly to U70A pin12 of CONTROL PULSE MATRIX C #1 as net 94 of control bus (not possible to merge nets 94 and 96 of CONTROL BUS).

- In SEQUENCE GENERATOR #2 U50A pin 5 and U50B pin 9 are directly connected to U22C and U22D due to EAGLE being unable to connect nets from different busses.

- In SEQUENCE GENERATOR #3 U104 is 74LS161A not in EAGLE library. Replaced with pin compatible 74160N. Also U8B pin 9 and U105 pins 2,5,6,9 directly connected to U21A, U20C, U20D, U20E, U20F respectively due to EAGLE being unable to connect nets from different busses. Between U57C and U77A there is bus SQ_BRST not going anywhere. Not implemented.

- In CONTROL PULSE MATRIX A #1 added Vcc (+5v) to 27C128 eproms U86 through U92 to pins 28 (Vcc) and 1 (Vpp).

LOGIC MODULE:

- In J100 connector, pin 15 of U77B is directly connected to pin 19 of J101. Also pins 18 of U79A and pin 7 of U79B are directly connected to pin 4 (201B)/pin 12 (U73B) and pin 9 (U201C)/pin 13 (U73B) respectively due to EAGLE being unable to connect nets from different busses.

- MONITORING BUS 824 has two names in Pultorak's schematics. "U_ MONITOR_BUS" and "ADD_OUT_BUS". In these schematics it's 824_ADD_OUT_BUS.

- The AU_EOC signal from ALU #1 isn't connected to an indicator. An extra indicator is placed in LM ASSEMBLY INDICATORS #3 using U35D, LED154 and R200.

- In ALU #5 U73B pin12 is directly connected to U201B pin4 / U79A pin18 due to EAGLE being unable to connect nets from different busses (8_BUSY_BUS net3 and 1_CONTROL_BUS net18).

- In ALU #5 U73B pin13 is directly connected to U201C pin9 / U79B pin7 due to EAGLE being unable to connect nets from different busses (8_BUSY_BUS net4 and 1_CONTROL_BUS net16).

MEMORY MODULE:

- In J102MM, U70B pin7 directly connected to J106MM, pin5 to J105MM pin22 and pin3 to J105MM pin21. (busses U_PARALM , U_BUSSY7 and U_BUSSY5 not implemented)

- In the MEMORY MODULE INDICATOR assembly the current limiting resistor naming is changed. R84 & R85 are also used in ADDRESS SUBSYSTEM  #4 (SWITCH DEBOUNCE CLOCK) and were skipped. Also R128 is used in ERASEABLE / FIXED MEMORY #2 so after R127, R148 was used.

- In ADDRESS SUBSYSTEM #3 instead of 74LS193 (U63, 64, 65, 66) used pin compatible 74LS192 (original part not in EAGLE library).

- In ERASEABLE / FIXED MEMORY #1 instead of 2016 ram cells used pin compatible 2015 (original part not in EAGLE library). Also in eproms U99 and U100 added Vcc (+5v) to pins 28 (Vcc) and 1 (Vpp).

- In MEMORY BUFFER #1 pin4 of U200B is directly connected to pin3 of U32A (MEMORY BUFFER #2) due to EAGLE being unable to connect nets from different busses.

- In PARITY GENERATION AND TEST #1 R127 was renamed R147 (R127 used in MEMORY MODULE INDICATOR assembly).

INTERFACE MODULE:

- In J103IM, U101 pin8 is directly connected to 1421_OUT1_BUS net8, pin5 is directly connected to bus BUSY2 and pin3 is directly connected to bus BUSY1 due to EAGLE being unable to connect nets from different busses.

- In DISPLAY #4 U49 is a 74LS161A not in EAGLE library. Replaced with pin compatible 74160N.

- In DISPLAY #5 all TIP107 transistors are replaced with TIP136 pin compatible ones (original part not in EAGLE library). LED display driver line 14 was removed because it wasn't going anywhere. There are 22 driver lines but only 21 7-seg LED displays to drive.

- In DISPLAY #6,#7,#8,#9 all TIP102 transistors are replaced with TIP122 pin compatible ones (original part not in EAGLE library). Also for U45, U46, U47, U48 (all 74LS47) a generic DIL-16 IC package was used (original part not in EAGLE library).

- In DISPLAY #10 the 7-seg LED displays used are HD-H101 CA (common anode). Not sure what John used.

- All nets from busses 1363A, 1362A, 1364A and 1365A are directly connected to the corresponding lines at DISPLAY #6, DISPLAY #7, DISPLAY #9 and DISPLAY #8 respectively.

- In DISPLAY #11 all 330Ω resistors are named R55. Renamed R55A, B, C, D, E, F.

- In DISPLAY #12 all TIP102 transistors are replaced with TIP122 pin compatible ones (original part not in EAGLE library).

CLOSING THOUGHTS

During this process of translating John’s schematics to EAGLE format, I managed to understand the AGC architecture and functions better than reading any book or manual. It’s not an easy mental task I assure you but it’s well worth it. It’s amazing what its designers did in the early 60’s. Today we take for granted technologies like Direct Memory Access or Real Time Priority Based Scheduling and Execution but back then it was bleeding edge tech. Kudos to them for achieving what they did while pushing digital computer technology light years ahead in the process…