AGC REVISION A1 CHANGELOG

SYSTEMWIDE CHANGES

- MONITORING SUBSYSTEM IS NOW A SEPERATE ASSEMBLY CONSISTING OF 3 SUBASSEMBLIES        (MCAMM, MCALM, MCADM). ALSO THE DSKY HAS BEEN SEPERATED FROM THE INTERFACE MODULE.

- THE BUSSES IN ALL MODULES THAT CONNECT COMPONENTS TO THE SWITCHES IN THE MCA    SUBASSEMBLIES ARE NAMED MCA_SWITCH_BUS.

- THE CURRENT LIMITING RESISTORS HAVE CHANGED DEPENDING ON THE LED COLOR:

WHITE      = 56 OHM

GREEN     = 120 OHM

RED           = 120 OHM

YELLOW  = 124 OHM

4.   ADDED THE VCC - GND CONNECTIONS TO ALL IC's PLUS 0.1uF BYPASS CAPACITORS.

INDIVIDUAL MODULE CHANGES

DRIVER MODULE:

- REMOVED POTENSIOMETER R1 10K. CLOCK FIXED TO 1MHz.

- REMOVED POTENSIOMETER R86 10K. CLOCK FIXED TO 1MHz.

- IN 1720_SCALER_OUTPUT ONLY PINS 10, 13, 17 ARE USED (100Hz, 12.5Hz, 0.78125Hz)

THE REST HAVE BEEN DELETED.

4.   IN 1_CONTROL_BUS PINS 62, 63, 64, 65, 67, 68, 69, 77, 78, 79 ARE NOT USED (SIGNALS

NOT IMPLEMENTED) AND HAVE BEEN DELETED.

5.   NODE OT_CLK2 IN OUTPUT REG 1 (OUT1) IS NODE CLK2. CORRECTED.

LOGIC MODULE:

CONNECTED 1_CONTROL_BUS NODE 25 TO U42A PIN 2.

MEMORY MODULE:

1.    IN ADDRESS SUBSYSTEM (ADR #1) NODE 613 (INTERNAL WRITE BUS) WAS NAMED BOTH

613_INTERNAL_WRITE_BUS & 613_2_INTERNAL_WRITE_BUS CAUSING A MISMATCH. 	CORRECTED.

2.    RESISTOR R1 (1K) HAD NO VALUE. CORRECTED.

INTERFACE MODULE / DSKY:

1.    ADDED PRO KEY (SW19 & R1 1K) CONNECTED TO CON18 PIN 19.

2.    ADDED THE FOLLOWING CAUTION & STATUS INDICATORS:

TEMP

AUTO

GIMBAL LOCK

HOLD

FREE

RESTART

NO ATT

TRACKER

3.    CHANGED THE +/- LOGIC IN DISPLAY (DSP) #11. REMOVED U25x HEX INVERTERS,

REPLACED 74LS02 WITH 74LS32 AND RECONFIGURED THE CONNECTIONS SO THAT WHEN

+ IS PRESSED ALL LEDS ARE LIT WHILE WHEN - IS PRESSED ONLY THE HORIZONTAL

LEDS ARE LIT.

4.     CHANGED THE VALUE OF R55x CURRENT LIMITING RESISTORS FROM 1 Ohm TO 40 Ohm.

DIMITRIS VITORIS

9/27/2009