# D.	USING A SEQUENCE DIGITISER

## D.1.	DIGISEQ

DIGISEQ reads DNA sequences from Sanger dideoxy gels using a sonic digitiser and stores the sequences in disk files. The files can then be sent to the VAX using KERMIT for further analysis.

DIGISEQ works with the Graf/Bar digitiser. Other digitiser makes may, or may not, work. Place the digitiser on top of a light box, with the DNA menu secured at the bottom-right corner, and the autoradiograph in the centre of the box. At all times you should ensure that there is no obstruction between the pen tip and the microphones on the digitiser box.

Make sure the power to the digitiser is on and that it is connected to the PC. Turn on the PC and select the correct directory:

cd digiseq

Instructions appear on the screen. The series of steps is as follows:

Start the program by typing DS

Press any key.

Select the directory where your gel readings are to be stored. Change the data format to UWGCG format. You may use Staden format if you intend to use the Staden DB system later on.

When you are satisfied press return when BEGIN is highlighted.

Holding the pen correctly mark the CANCEL spot, then the ORIGIN at the head of the digitiser box.

Mark the bottom left point of the menu box, then the top right. Check the computer screen for messages.

Press the pen on START. The computer screen should prompt you to enter the heading text for the GCG format file. Press return to finish.

Position a ruler at the level you will read the first band. Mark the centre of each lane reading from left to right.

You are now ready to enter your data. Progress upwards, pressing the pen point in the CENTRE of each lane, in order of bands.

Mark the end of the sequence by pressing the pen in the STOP area of the menu.

To delete a previously entered base, press the pen in the DELETE area of the menu box. If you are unsure of the identity of the next base, enter one of the uncertainty codes by pressing the pen on one of the codes in the menu box.

## D.2.	Interpretation of sequence gels

### D.2.1.	Band distribution

In addition to the problems associated with poor gel quality there are some general points worth mentioning:

In a given section of sequence, bands should be spaced regularly.

What appears to be a larger than normal gap between bands is often a fainter than normal band.

What appears to be a smaller than normal gap between bands can be a spurious band that can be ignored.

### D.2.2.	Variable band intensity

Much of the variation in band intensity is sequence specific.

#### 1	D.2.2.1The C rules: -

Single C bands are generally weak.

In a run of two or more consecutive C bands the first is very weak and the second the strongest. This can sometimes make the first C in a run particularly difficult to see.

#### 2	D.2.2.2.The A rules: -

The first A band in a run of consecutive A's is stronger than the others. This can make it difficult to see exactly how many A bands are in a run. In particular the second A in a band can be partially obscured by the stronger first A band.

#### 3	D.2.2.3.Other rules: -

Single C's are occasionally not as weak when preceded by a G.

G bands are weaker when preceded by a T band.

The TGCC sequence can cause an artefact band in the C track -somewhere between the levels of the T and G band.

The GCA sequence can cause artefact bands in the T and C tracks, which occur at the level of the A band in the GCA sequence.

## D.3.	KERMIT file transfers

The program KERMIT can be used to send files between the PC and the VAX. Here is an example session.

On the PC run kermit by entering

C>KERMIT
KERMIT>C

You are now connected to the VAX. Logon in the usual way: press return until the PAD> prompt appears. Next you must start up KERMIT from the VAX end:

$ CKERMIT

Now inform the VAX Kermit that it should expect to receive a file from the PC:

C-KERMIT>Rec

With KERMIT active on both the VAX and the PC you can hop between the two computers and give instructions from one side or the other. To hop from the VAX to the PC enter Alt-X. To hop to the VAX enter C as before.

Now we will send the file DEMO.GEL from the PC.

C-KERMIT>Alt-X
KERMIT>Send DEMO.GEL

Now hop back to the VAX. Look at the directory to see the transferred file. The next thing to do is to EXIT kermit, log off the VAX, hop back to the PC and EXIT kermit there:

C-KERMIT>Dir

C-KERMIT>Exit

$ lo

Alt-X

KERMIT>Exit

C>

## D.4.	EMUTEK file transfers

The EMUTEK program provides terminal emulation, supporting Tektronix graphics on a PC (See Chapter 1-3). Emutek also supports file transfer, in combination with a VAX-based kermit.

On the PC run Emutek by entering

C>emu-tek

You should get a banner message. Press return until you get a response. Depending on how your PC is connected to the VAX, you will either get a PAD prompt, or a login prompt. Logon in the usual way

Move to the directory you wish to store the PC files in. eg:

$ sd .gelsin

Next you must start up KERMIT on the VAX:

$ Kermit

Now inform the VAX Kermit that it should expect to receive instructions from Emutek:

Kermit-32> server

Bring up the UPLOAD menu from emutek by pressing Alt-U.

We want the item upload Kermit checksum, so press the K key.

The next window asks you to enter two items:

Source File name: c:\digiseq\mu18.seq
Destination File name: mu18.seq (defaults cause problems!)

When complete you will see [TRANSFER COMPLETED - PRESS A KEY].

If you get a TRANSFER HALTED message - check that your file names have been entered correctly.

Repeat entering K and file names until all files have been transferred.

MU10.SEQ MU18.SEQ MU18B.SEQ MU2.SEQ MU23.SEQ MU26.SEQ MU26B.SEQ MU27.SEQ MU32.SEQ MU5.SEQ MU6.SEQ MU9.SEQ

Enter F to Finish host server mode, return, then esc,return.

Kermit-32>exit

Look at the directory to see the transferred files.

To send files in the other direction: ALT-C for the emu-tek CAPTURE menu. Enter G for Get from Kermit server.

To EXIT Emutek: log off the VAX, enter Alt-X, then select Y.