SEMAPHORE DIRECTIVES FOR RSX-11M EXECUTIVE

Version 01.2		May 18,1977

Last modification	April 16,1980

Harry Atherton

University of Cincinnati Medical Center

Department of Pediatrics, Room 6253

231 Bethesda Ave.

Cincinnati, Ohio 45267

Phone (513) 872-5341

The files in this UIC implement binary semaphore opera-

tions  in  the RSX-11M executive.  This software has been in

use at our installation for over two years, and  it  is  be-

lieved  to be bug free.  If you have any problems, comments,

or suggestions I would like to hear them.  No responsibility

is assumed for the use or reliability of this software.

SEMAPHORE DIRECTIVES FOR RSX-11M EXECUTIVE            PAGE 2

The following  files  implement  the  binary  semaphore

P-operation and V-operation executive directives:

DRDSP.MOD   -- Adds the directives to the dispatch table.

DREIF.MOD   -- Adds call to cleanup routine on task exit.

The above are for RSX-11M V3.2 (BL26).

DRSEM.MAC   -- Directive processing and  exit  cleanup  rou-

tines.

SEMDIR.MAC  -- Macros that can be  added  to  RSXMAC.SML  to

provide directive calls SEMP$S AND SEMV$S.

SEMCAL.MAC  -- FORTRAN callable subroutines  provide  direc-

tive  calls  SEMP  and  SEMV.   Object module

should be added to SYSLIB.OLB or FOROTS.OLB.

SEMTEST.FTN -- Small FORTRAN program to test and demonstrate

use of semaphores.  The object module of SEM-

CAL.MAC must be available to the task  build-

er.

DRDSP.OLD   --

DREIF.OLD   -- If you are generating RSX-11M V3.1 (BL22) use

these files in place of the first two files.

To add the semaphore  directives  a  system  generation

must  be  done.   To  prepare  for  the  SYSGEN  copy  files

DRDSP.MOD and DREIF.MOD to [11,40] on the  executive  source

disk.   These  two files are for RSX-11M V3.2(BL26).  If you

are generating RSX-11M V3.1 (BL22) use the  files  DRDSP.OLD

and  DREIF.OLD  instead.   Also copy DRSEM.MAC to [11,10] on

the executive source disk.

After applying any DEC supplied patches, these  modifi-

cations  are  made.   The SLP correction files DRDSP.MOD and

DREIF.MOD use ASCII string locators to insert the  additions

to  the  program  logic.  (The comments are still handled by

absolute line numbers since their location is  not  critical

to  proper  program operation).  These files must be applied

AFTER the DEC patches.  (There is a correction to  DRDSP.MAC

on  the  Autopatch  kit).   Note that these files are called

.MOD instead of .COR to avoid confusing them  with  the  DEC

files.

Also watch out for file version numbers on DRDSP.MAC if

multiple SYSGENS are done on the same disk.  The DEC correc-

tion files specify version 1 for input  and  version  2  for

output.   If  you  are working with a virgin disk everything

will work right since DRDSP.MOD  does  not  specify  version

numbers.   If you have gone through all of this before there

will be a DRDSP.MAC;3 left over.   In  that  case  DRDSP.MOD

will  use  it  for  input and create a version 4 which would

have the modifications applied twice.  Make sure that  there

is not already a DRDSP.MAC;3 on the disk before you start.

DRDSP.OLD and DREIF.OLD still use absolute line numbers

SEMAPHORE DIRECTIVES FOR RSX-11M EXECUTIVE            PAGE 3

for  insertion.   I am not aware of any DEC changes to these

files in V3.1.  It is  essential  that  these  modifications

only be applied to the original version of the .MAC files.

To have the module DRSEM assembled with the other  exe-

cutive  modules  answer  Y to the question:  "Do you wish to

edit any of the  Executive  files  ?"  Then  edit  the  file

[11,24]RSXASM.CMD.   Locate  the  line containing DRSED.  It

should look like:

[11,24]DRSED,listing=[1,1]EXEMC/ML,[11,10]RSXMC,LB:DRSED

Add a line exactly like it, except replace DRSED with  DRSEM

(it occurrs three times in the line).  The module DRSEM will

be included in the executive object module library automati-

cally  by SYSGEN.  This is also a good time to edit the exe-

cutive build command file.  It is file [11,10]RSXBLD.CMD  on

the  baseline  disk.  Locate the line containing DRSED.  Add

the following line after it:

[1,24]RSX11M/LB:DRSEM

The SYSGEN can proceed normally from this point.

When the SYSGEN  is  finished  you  can  add  the  file

SEMDIR.MAC  to  [1,1]RSXMAC.SML  so  the  macros  SEMP$S and

SEMV$S will be available.  Also the file  SEMCAL.MAC  should

be assembled and the object module placed in [1,1]SYSLIB.OLB

or [1,1]FOROTS.OLB so the FORTRAN subroutines SEMP and  SEMV

are available.

To demonstrate the functioning of the semaphores,  com-

pile and build the program SEMTEST.FTN.  By running multiple

copies from several terminals you can see how the  semaphore

functions  allow  only  one program at a time to execute the

"critical  region"   between   the   P-operation   and   the

V-operation.

The following pages describe the  executive  directives

in the format of the Executive Reference Manual.  Additional

information about semaphores is given in the  paper  "Imple-

mentation and Application of Semaphores in RSX-11M" present-

ed at the Fall  1979  DECUS.   This  should  appear  in  the

Proceedings  of the Digital Equipment Computer Users Society

USA 1979 Fall.  Reprints are available from the author.

SEMP$S (only $S form supplied)

Semaphore P-operation

The Semaphore P-operation instructs  the  executive  to

place  the issuing task on the specified semaphore list.  If

there is already a task on the list the issuing task is sus-

pended.   If  no  other task is on the list the issuing task

continues execution.

FORTRAN call:

CALL SEMP(number[,ids])

number = Semaphore number

ids    = Directive status

Macro call:

SEMP$S number[,error]

number = Semaphore number

error  = Error routine address

DSW Return Codes:

IS.SPD -- Successful Completion (task was suspended)

IS.SUC -- Successful completion (task was not  suspend-

ed)

IE.UPN -- Insufficient dynamic memory

IE.ILU --  Semaphore  number  outside  allowable  range

(range  is 1-20;  upper limit is alterable at

system   generation   time;     see    module

DRSEM.MAC)

IE.ADP -- Part of DPB is out of the issuing task's  ad-

dress space

IE.SDP -- DIC or DPB size is invalid

Notes:

1.  If the task is suspended the suspend status bit  is

set;   see description of directive SPND$ for addi-

tional notes.

2.  Since the suspend bit is used the task may  be  re-

sumed  by  the  MCR command or the executive direc-

tive.  If this is done proper operation of the sem-

aphore will be impossible.

3.  There are two successful completion codes, IS.SPD =

+2 and IS.SUC = +1.  If you are interested in know-

ing whether or not a program is actually  suspended

when it calls SEMP$S, the status returned will tell

you.

SEMV$S (only $S form supplied)

Semaphore V-operation

The Semaphore V-operation instructs  the  executive  to

remove the issuing task from the head of the specified sema-

phore list.  If any other tasks are on the list the  one  at

the head is resumed.  If the issuing task is not at the head

of the specified list no action is  taken  and  status  code

IE.ULN is returned.

FORTRAN call:

CALL SEMV(number[,ids])

number = Semaphore number

ids    = Directive status

Macro call:

SEMV$S number[,error]

number = Semaphore number

error  = Error routine address

DSW Return Codes:

IS.SUC -- Successful completion

IE.ULN -- Task was not at head of  the  specified  list

(Note 1)

IE.ACT -- Next task on list was not active (Note 2)

IE.ITS -- Next task on list was not suspended (Note 2)

IE.ADP -- Part of DPB is out of issuing task's  address

space

IE.SDP -- DIC or DPB size is invalid

Notes:

1.  This will result if the task calls  SEMV$S  without

having previously called SEMP$S with the same sema-

phore number.

2.  This may be caused if a  program  using  the  sema-

phores  is  resumed by the MCR command or executive

directive.

3.  When a task exits the executive cleans up the sema-

phore  lists.  If the exiting task has failed to do

any required SEMV$S directives, they are  simulated

by the executive.