For over the past decade, those who had RT-11 source

kits saw the following comment in the sources for RESORC:

;>>>    add multiple CSR display in the future

I decided that the future has come, and made some modifications.

Here's how I felt that secondary (and tertiary and whatever comes

next) CSR's should be displayed:

.SHOW DEV

Device    Status                   CSR     Vector(s)

------    ------                   ---     ---------

DY      Installed               177170   264

VM      Installed               177572   000

DZ      Not installed           000000

DW      Not installed           000000

SP      Installed               000000   110

DX      Not installed           177170   264

XC      Not installed           173300   210 214

RK      Not installed           177400   220

DL      Installed               174400   160

DM      Not installed           177440   210

NQ      Installed               174440   120

NC      Not installed           000000

NU      Not installed           174510   120

MT      Not installed           172520   224

MM      Not installed           172440   224

MS      Installed               172522   224 300

CSR2  172526

LP      Not installed           177514   200

SL      Installed               000000   000

UB     -Not installed           170200   000

PI     -Not installed           000000   000

EQ      102226                  174440   120

NL      Installed               000000   000

LD      077236                  000000   000

DU      Resident                172150   154 150 144 140

CSR2  160400

CSR3  160700

CSR4  160344

MU      Installed               174500   260 340 344 350

CSR2  160444

CSR3  160404

CSR4  160454

Note that multiple vectors still appear on the "main" line.  I

suppose that it's conceivable that a multi-CSR handler would have

multiple vectors per CSR, and it wasn't clear to me how to tell

which vectors belong to which CSR.  If anyone has any good ideas

on this subject, I'm willing to listen!

And here's the SRCCOM differences between the distributed RT-11 5.7

sources and my modifications:

.DIFF DU2:RESDEV.MAC,LD0:RESDEV.MAC

1) DU2:RESDEV.MAC

2) LD0:RESDEV.MAC

**********

1)4	;

****

2)4	; 014	14-Mar-1999	TDS	Add multi-CSR displays

2)	;

**********

1)10		MOV	$HDCSR(R4),R3	;Get the CSR value

1)		BNE	1$		;Found one

1)		CMP	#^RHAN,(R4)	;Is this a pre-V5.3 handler

1)		BNE	1$		;Branch if so

1)		MOV	$DDCSR(R4),R3	;Try a display CSR

1)	1$:

1)	;>>>	add multiple CSR display in the future

1)		MOV	$HDCQE(R4),DIRTY ;save possible HDCQE value

****

2)10	;	CLR	DIDVEC		;We haven't gone through vectors yet

2)		MOV	R2,-(SP)	;Save R2

2)		MOV	R5,-(SP)	;Save R5

2)		MOV	#CSRLST+4,R5	;CSR's to list here

2)		CLR	-(R5)		;Clear the first

2)		CLR	-(R5)		; two values

2)		MOV	R4,R2		;Place to start looking for CSR's

2)		ADD	#<$HDCSR+2>,R2	; is the HDCSR word

2)		MOV	-(R2),R3	;Test it

2)		BEQ	1$		;if nothing there, try display CSR's

2)		MOV	R3,(R5)+	;if something there, save it

2)		MOV	-(R2),R3	; and skip first display CSR's

2)		CMP	R3,#170200	;Is this the UB handler?

2)		BEQ	3$

2)	1$:	CMP	#^RHAN,(R4)	;Is this a pre-V5.3 handler

2)		BNE	3$		;Branch if so

2)	2$:	MOV	-(R2),(R5)+	;Get the display CSR value

2)		BEQ	3$		;if a zero, then we're done

2)		CMP	R5,#CSRLST+10.	;do we already have 4 CSR's?

2)		BNE	2$		;if not, try another

2)	3$:	MOV	#CSRLST,R5

2)

2)		.DSABL	LSB

2)		.ENABL	LSB

2)

2)	LSTVE2: MOV	(R5)+,R3	;Get the next CSR to display

2)		MOV	$HDCQE(R4),DIRTY ;save possible HDCQE value

**********

1)10	10$:

1)		RETURN

****

2)10	10$:	MOV	(R5)+,R3	;is there another CSR to display?

2)		BEQ	20$		;branch if not

2)		.PRINT	#BUFFER		;print the previous line

2)		MOV	R5,R0		;figure out which CSR we're showing

2)		SUB	#<CSRLST>,R0	; be subtracting off the list base

2)		ASR	R0		;divide by two

2)		FORMAT  #BUFFER,<                            CSR[U1]>,<R0> ;CSRn

2)		FORMAT  ,<  [B]>,<R3>	;and the actual CSR

2)		BR	10$		;try for another

2)	20$:	MOV	(SP)+,R5	;restore R5

2)		MOV	(SP)+,R2	;restore R2

2)		RETURN

**********

?SRCCOM-W-Files are different

.diff du2:resrot.mac ld0:resrot.mac

1) DU2:RESROT.MAC

2) LD0:RESROT.MAC

**********

1)21	;END OF AREA TO KEEP TOGETHER

****

2)21	CSRLST::.BLKW	5.		;4 CSR's, trailing zero word.

2)	;END OF AREA TO KEEP TOGETHER

**********

?SRCCOM-W-Files are different

Those who are interested in the resulting patched RT-11 5.7 binary

can fetch it from

ftp://sunsite.unc.edu/pub/academic/computer-science/history/pdp-11/rt/resorc/

Similar patches could be made to earlier RT-11 releases, but

obviously everyone's who is actively using RT has upgraded to

RT-11 5.7 by now.

--

Tim Shoppa                        Email: shoppa@trailing-edge.com

Trailing Edge Technology          WWW:   http://www.trailing-edge.com/

7328 Bradley Blvd                 Voice: 301-767-5917

Bethesda, MD, USA 20817           Fax:   301-767-5927