Skipping unavailable article...........................................................................................Article 4414 (285 more) in rec.woodworking:

From: rkc@xn.ll.mit.edu

Subject: Re: Installing Hardwood Floors

Message-ID: <1991Sep12.134751.13311@xn.ll.mit.edu>

Date: 12 Sep 91 13:47:51 GMT

Organization: MIT Lincoln Laboratory

Lines: 31

I am in the process of installing hardwood floors.  I have a small cape, and

we did some remodeling that removed a wall downstairs and converted the

upstairs from attic to livable space.  The total square feet of hardwood to be

installed/replaced is about 250.  I was quoted the following prices:

Installing/Piecing in:

Materials/Eq. Rental    Contractor A  Contractor B

$700             $1800          $2000

Sanding/Finishing:

Materials/Eq. Rental

$350             $1150          $1500

So you can see that you can save significant amounts of money by doing it

--MORE--(49%)yourself.  But you should realize that this work is HARD.  The hammer you use

to install the floor is about 5 lbs, and swinging that for 10 hours a day over

2-3 days can get to be VERY tiring.  While fitting edge pieces and installing

in closets (both are handwork jobs) can take quite a long time, wide open

areas are relatively easy to install.  I'm told that some people use pnematic

nailers to speed the work and ease the strain on the arms and back; these were

not available in my area, and I'm not so certain I'd want to use them for

safety reasons.

I am going to finish the installing myself, and then have a professional sand an d

finish the wood, because the sanding job makes a mess, and we are living in

the house while this work is going on.  Thus, it makes sense for me to hire

someone to finish up quickly so we can go about our "normal" lives.

-Rob "Living in a constuction zone in Mass."

End of article 4414 (of 4701)--what next? [npq] Article 4415 (284 more) in rec.woodworking:

From: jims@amadeus.WR.TEK.COM (Jim Sullins)

Subject: HELP! Bandsaw Information Needed

Keywords: bandsaw

Message-ID: <7169@wrgate.WR.TEK.COM>

Date: 12 Sep 91 05:00:19 GMT

Sender: news@wrgate.WR.TEK.COM

Reply-To: jims@amadeus.WR.TEK.COM (Jim Sullins)

Distribution: usa, na

Organization: Tektronix, Inc., Beaverton,  OR.

Lines: 39

HELP! I got an OLD (Early 60s vintage?) Sears bandsaw at a garage sale

a while back and I am trying to improve its cutting characteristics,

etc. The model number of the saw is 103.24280. I believe it is considered

to be a 12" saw (12 inches from blade to case).

Well, not too surprisingly, Sears no longer stocks parts for this particular

model of saw, and unfortunately does not provide any cross-reference to a

newer (still supported model) with compatible parts. So I hope that by

--MORE--(40%)Article 4416 (283 more) in rec.woodworking:

From: moody+@cs.cmu.edu (James Moody)

Subject: AMT summary

Message-ID: <1991Sep12.135835.98331@cs.cmu.edu>

Date: 12 Sep 91 13:58:35 GMT

Distribution: na

Organization: School of Computer Science, Carnegie Mellon

Lines: 81

Nntp-Posting-Host: ius4.ius.cs.cmu.edu

Originator: moody@IUS4.IUS.CS.CMU.edu

Yesterday I posted a request for opinions on AMT products.  Here's the

responses I received so far (I received a couple requests to post my

results).  Names and addresses removed for brevity:

--------------------

I have their scroll saw and their cross vise.  The scroll saw is ok;

it vibrates more than I'd like, but I was expecting about the quality

I got.  The vise is more problematic; the vibration of the drill press

tends to move the cross-arms...

In general, I'd rate them as a good Taiwanese clone importer.  Their

--MORE--(25%)Article 4417 (282 more) in rec.woodworking:

From: wilson@schaefer.math.wisc.edu (Bob Wilson)

Newsgroups: rec.scuba,rec.woodworking

Subject: Re: scuba tanks as spray gun source

Message-ID: <1991Sep12.134811.14574@schaefer.math.wisc.edu>

Date: 12 Sep 91 13:48:11 GMT

References: <1991Sep6.151933.13229@cbfsb.att.com> <12716014524010@mathom.cisco.c om> <91252.101642U08485@uicvm.uic.edu> <BUDD.91Sep10131621@drynix.dfrf.nasa.gov>  <SOS.91Sep11135651@spsd4360a.erim.org>

Reply-To: wilson@math.wisc.edu (Bob Wilson)

Organization: Univ. of Wisconsin Dept. of Mathematics

Lines: 26

>not a good idea unless you have the scube gear already.  It used

>carefully 80 cubic feet of air will go quite aways.  On the surface,

>a person will take up to 1 1/2 hours to consume that much.

>--

>

>Tim (or Garrett) Sos  sos@erim.org or sos@amadeus.erim.org

>  --  all standard disclamers apply --

>Environmental Research Institute of Michigan, Signal Processing Department

>ERIM, P.O. Box 134001, Ann Arbor, MI 48113-40001,  (313)994-1200 Ext. 2848

--MORE--(53%)Article 4418 (281 more) in rec.woodworking:

From: ashen@hpnmdla.sr.hp.com (Claude Ashen)

Subject: Re: Power and *cost* (was: Re: table saw motor)

Message-ID: <4890168@hpnmdla.sr.hp.com>

Date: 11 Sep 91 15:36:25 GMT

References: <1991Aug29.160913.11674@cbnewse.cb.att.com>

Organization: HP Network Measurements Div, Santa Rosa, CA

Lines: 19

In rec.woodworking, mlord@bnr-rsc.UUCP (Mark Lord) writes:

In article <15450@ulysses.att.com> smb@ulysses.att.com (Steven Bellovin) wri tes:

*   <In article <marshall.684199522@windslc>, marshall@windslc.seri.gov (Marshal l Buhl) writes:

*   <> >Anybody have any idea why Sears would say not to run the table saw motor

*   <> >on 240V? This intrigues me because I am going to add a 240V circuit to m y

*

*   Don't forget to also change the power switch from SPST to SPDT, so that it

*   disconnect BOTH live wires when in the off position.

--MORE--(74%)Article 4419 (280 more) in rec.woodworking:

From: stanj@hpnmdla.sr.hp.com (Stan Jaffe)

Subject: Turning Wooden Writing Pens

Message-ID: <4890169@hpnmdla.sr.hp.com>

Date: 11 Sep 91 20:14:21 GMT

Organization: HP Network Measurements Div, Santa Rosa, CA

Lines: 44

A little while ago someone was looking for information on turning writing

pens from wood.  Last night I had the chance to visit a local wood turner

who turns bowls and pens.  Here's some information on turning pens:

1.  The pen blank is roughly 7/8" square by ~2" long (grain running

lengthwise?).  There is a 7mm hole bored lengthwise with a thin

brass tube inserted.  The tube is glued to the blank.

2.  The wood/brass combination is chucked between centers on a special

mandrel.

3.  Wood is turned to desired shape and size using standard spindle shaping

techniques.

4.  The piece is finished on the lathe.  Deft finish (modified lacquer) is

--MORE--(48%)    applied with a rag, the the lathe is turned on and the finish dried

through friction heating between the wood and the rag.  Final finishing

is accomplished with a paste wax.

So, would you like to pursue this but the above instructions leave much to

be desired?  Well, I have good news for yews!  Full instructions are

available ($4), pen hardware ($3.25/pen for small quantities, less for more),

and special lathe mandrel ($20) from:

Craft Supplies         1-801-373-0917

1287 E.  1120 S.

Provo, Utah   84601

They carry a full line of woodturning tools and supplies.

DISCLAIMER: I have no affiliation with Craft Supplies.  I have never done

business with them.  In fact, I had never heard of them until

last night.  Please DO NOT ask me any questions regarding

wooden pens or wood turning in general, because I have

exhausted all my knowledge of the subject within this

posting!  ;^)

GOOD LUCK!

--MORE--(91%)

Stan Jaffe    A.K.A.   stanj@sr.hp.com

HP Santa Rosa, Calif.

"Not responsible for any opinions but my own"

------------------------------------------------------------------

End of article 4419 (of 4701)--what next? [npq] Article 4420 (279 more) in rec.woodworking:

From: isjrc@dcatla.uucp (John R. Caruso)

Subject: Re: Keller dovetail jigs

Message-ID: <4531.28cf15e3@dcatla.uucp>

Date: 12 Sep 91 07:23:15 GMT

References: <1991Sep9.195912.27660@infonode.ingr.com>

Organization: Digital Communications Associates, Alpharetta, GA

Lines: 15

In article <1991Sep9.195912.27660@infonode.ingr.com>, bernie@barney.b11.ingr.COM  (Bernie Holl) writes:

> Does anyoneknow of sources for the Keller dovetail jigs?  I have found

> them in a "Woodworkers of New mexico" catalog but not anywhere else.

> Thanks

>

> Bernie Holl  bernie@barney.b11.ingr.com

I think the Trendlines catalog has that item. Trendlines has an 800

number which I don't have at the moment. See the FAQ list or call

1-800-555-1212 and ask for the Trendlines listing.

--MORE--(99%)Article 4421 (278 more) in rec.woodworking:

From: isjrc@dcatla.uucp (John R. Caruso)

Subject: Re: Installing a Hardwood Floor

Message-ID: <4532.28cf185a@dcatla.uucp>

Date: 12 Sep 91 07:33:45 GMT

References: <91252.151337DM3WFINR@MIAMIU.BITNET>

Organization: Digital Communications Associates, Alpharetta, GA

Lines: 31

In article <91252.151337DM3WFINR@MIAMIU.BITNET>, DM3WFINR@MIAMIU.BITNET writes:

> I will be moving next year...having a house built for myself (i.e., I am

> not doing any of the construction).  But, I did think of installing hardwood

> floors because it seemed like a way for me to save some money, and

> contribute to the construction.  Has anyone installed hardwood floors and

> is it worth the effort/time to save some bucks?

>

> David.

I have done a hallway and a room about 10 by 12. The cost of the materials

alone, 3/4 T & G red oak, came to about $4/sq. ft.. That's about what

you pay for a damn good carpet but the hardwood floor will probably

outlive even you.

--MORE--(50%)

It's an awful job if you do it by hand with cut nails. Any other nail will

probably bend. I did the hallway by hand and said I'd never do it again.

A few years later, I did the room and rented a flooring nailer. Since

I have a compressor it wasn't a problem; in retrospect I would have

rented a compressor if needed because the job is a colossal bore. It's

like refinishing a gymnasium floor with an artist's brush. You lay down

one row at a time and when you get to the end of the row you're amazed at

how little ground you've covered, and on and on it goes. The pneumatic

nailer makes the job infinitely easier. When finished installing the floor

make sure you sand it preferably with a drum sander which is much easier

to control. Get a price on your materials locally and then get a quote

from an installer. I think I've described the effort involved and you

can decide if that plus the savings is worth it. I couldn't afford to have

most things done for me on my house so it was worth it.

jRc

End of article 4421 (of 4701)--what next? [npq] Article 4422 (277 more) in rec.woodworking:

From: budd@elxsi.dfrf.nasa.gov (Gerry Budd)

Newsgroups: rec.scuba,rec.woodworking

Subject: Re: scuba tanks as spray gun source

Message-ID: <BUDD.91Sep12083105@drynix.dfrf.nasa.gov>

Date: 12 Sep 91 15:31:05 GMT

References: <1991Sep6.151933.13229@cbfsb.att.com>

<12716014524010@mathom.cisco.com> <91252.101642U08485@uicvm.uic.edu>

<BUDD.91Sep10131621@drynix.dfrf.nasa.gov>

<SOS.91Sep11135651@spsd4360a.erim.org>

Sender: news@skipper.dfrf.nasa.gov

Organization: NASA Dryden, Edwards, Cal.

Lines: 28

In-reply-to: sos@spsd4360a.erim.org's message of 11 Sep 91 18:56:51 GMT

In article <SOS.91Sep11135651@spsd4360a.erim.org> sos@spsd4360a.erim.org (Tim So s) writes:

If you own the scuba tank, all you need is a $10 fitting and the

secondary regulator ($30-60).  The air costs $2-4/tank.  If this

lasts 2-3 months of "small" spray jobs I would be happy.  This is

not a good idea unless you have the scube gear already.  It used

carefully 80 cubic feet of air will go quite aways.  On the surface,

--MORE--(53%)Article 4423 (276 more) in rec.woodworking:

From: br+@CS.CMU.EDU (Bill Ross)

Subject: Thickness Planer Follow-on

Message-ID: <1991Sep12.113918.44751@cs.cmu.edu>

Date: 12 Sep 91 11:39:18 GMT

Reply-To: br@cs.cmu.edu

Organization: CMU Vision and Autonomous Systems Center

Lines: 30

Nntp-Posting-Host: ius4.ius.cs.cmu.edu

Since I recently posted a summary of my thickness planer findings,

I thought I would tell you folks about how it turned out.

I ended up ordering the Delta 12" from Tools on Sale.  The price was

$389 and it showed up 8 days after I placed my order.  No real

problems except that the cosmolene that was supposed to protect the

blades had melted/liquified and left sticky black oily stuff all

over everything...

After a good cleaning I ran a bunch of pine scrap through it until

I thought it had spun all the residue off the cutter head.  Then I ran

several 6"*3" pieces of oak through it.  The oak went smoothly with

--MORE--(57%)    no bogging down even on deeper cuts.  As some people have mentioned

previously, the Delta does seem to snipe a little more than some

other planers if you take deep cuts.  The sniping was more noticable

in the soft wood than in the oak though.

Nice things:  The height gauge seems to be of usable accuracy.  The

infeed and outfeed tables are nice and big and support the wood well.

The machine is less noisy than typical small planers.   It seems to

have plenty of power.

Biggest problem so far:  The infeed and outfeed tables don't attach

to the planer very securely and are a little flimsy.  I would like to

mount the planer on a couple of oak rails to make it more stable to

use but I will have to rig up some sort of support for the infeed and

outfeed too...

Bill

End of article 4423 (of 4701)--what next? [npq] Skipping unavailable article..............................................................................................................................Article 4552 (149 more) in rec.woodworking:

From: hsc@stroid.sybase.com (Howard Cohen)

Subject: Re: HELP! Bandsaw Information Needed

Message-ID: <15236@sybase.sybase.com>

Date: 20 Sep 91 16:16:46 GMT

References: <64204@rphroy.ph.gmr.com>

Sender: news@Sybase.COM

Organization: Sybase, Inc.

Lines: 24

In article <64204@rphroy.ph.gmr.com> rhaar@albert.oname writes:

>I wouldn't count out Sears. Your comments do apply to the saleslime

>at many Sear retail stores. But Sears also has separate Service Centers.

>Maybe the one near me is unusual, but I have had consistently good

>success with them.

I concur.  I too have used a Sears service center. They were crowded on a

Saturday, but I soon came to understand that it was because they offer a

valuable service and they do a good job of it.  I had a Sears hand-held

circular saw that had a trashed gear on the inside.  I brought the serial

number and model number with me, and the clerk found an exploded diagram.

She let me point to the part I needed, and that way I was sure I was

ordering what I needed.  She filled out an order right there, and I had my

--MORE--(68%)Article 4553 (148 more) in rec.woodworking:

From: wainscot@oreo.berkeley.edu (Brian Wainscott)

Subject: Need advice: looking for a slow grinder.

Message-ID: <1991Sep22.034249.3950@agate.berkeley.edu>

Date: 22 Sep 91 03:42:49 GMT

Sender: usenet@agate.berkeley.edu (USENET Administrator)

Distribution: na

Organization: U.C. Berkeley Math. Department.

Lines: 35

Originator: wainscot@oreo.berkeley.edu

I'm looking for a slow grinder for sharpening my chisels on (only for the

first stage, when I've really abused one of them).  I need advice.  This

is what I've found so far:

Delta has a nice looking wet/dry grinder (normal small wheel with a

10"x2" 70rpm wet wheel) for $159 (price at TOS).  This is more than I

want to spend.

AMT has a very similar looking beast, price $97+$14 shipping.

I don't know anything about AMT (American Machine and Tool, PA).

I was not impressed with anything else I saw in their catalogue, so I am

suspicious about this also.  It looks ok, but I just don't seem to be able

--MORE--(47%)to bring myself to buy it.  Any and all information about these folks

and the quality of their stuff would be appreciated.

Finally, Krenov (in his book The Fine Art of Cabinetmaking) recommends

a simple hand driven grinder.  I believe this is becuase the slow grinders

have only recently become available.  In any case, Garret Wade has a

hand driven 6" grinder (geared with 13.5 to 1 ratio) for about $55 (last

years catalogue).  This is much more in my price range, but a motor would

be nice -- I'm not sure I'm coordinated enough to turn the wheel with one

hand while sharpening the chisel with the other!

Advice, comments and suggestions all most welcome.  Thank you.

(BTW, I already have a 180 grit waterstone, which is ok but not really

adequate for what I want to do.  I've heard people like the coarse diamond

stone, but I think I really want a grinder.)

--

Brian Wainscott     \=========================================================

Graduate Student      \ "I want to get so close to Him, that its no big change

U.C. Berkeley           \ on the day that Jesus calls my name." - Wayne Watson

wainscot@math.berkeley.edu\===================================================

End of article 4553 (of 4701)--what next? [npq] Article 4554 (147 more) in rec.woodworking:

From: sbhattac@sales.GBA.NYU.EDU (Shankar Bhattacharyya)

Subject: Re: circ -> pseudo table saw : good or bad idea?

Message-ID: <16506@sales.GBA.NYU.EDU>

Date: 22 Sep 91 13:00:54 GMT

References: <14845.28da1296@ecs.umass.edu>

Organization: NYU Stern School of Business

Lines: 52

In article <14845.28da1296@ecs.umass.edu> marsden@ecs.umass.edu writes:

>My question:  I've heard of "workstation" tables that will allow you

>to attach hand held tools for added versatility, eg circular saw becomes

>a poor man's table saw, drill -> drill press, and such.  I'm sorry this

>sounds like such a hack, but I'd be interested in hearing if anyone

>has opinions/knowledge of such things, and where to get them.  I'm

>concerned about safety, and cost/quality trade-offs.  I plan on getting

>a router table, but thought I'd find out if a more versatile router

>plus circular saw table might be a reasonable option.

I have. As a very broke student I went through the same reasoning, and

bought a Hirsch table, designed for just this purpose. It sounded like a

good idea. It turned out it was not.

--MORE--(41%)Article 4555 (146 more) in rec.woodworking:

From: hsc@stroid.sybase.com (Howard Cohen)

Subject: Re: Making frame&panel doors by hand

Message-ID: <15244@sybase.sybase.com>

Date: 20 Sep 91 17:25:28 GMT

References: <9935@cognos.UUCP>

Sender: news@Sybase.COM

Organization: Sybase, Inc.

Lines: 17

In article <9935@cognos.UUCP> richardb@cognos.UUCP () writes:

>

>Is there some hand tool I'm not aware of? What

>did they do in the 'old' days of frame&panel construction?

I guess it depends on which culture you think of when you say the

"good old days".  I have one of those very thin japanese pull-saws, and I

have been extremley impressed with it so far.  It really is easy to use and

makes a wonderful cut. I'd bet you can easily cut a groove in the end-grain

that would be suitable for the frame in a frame and panel cabinet.  You might

clean up any excess with a chisel, and you're in business (so to speak.)

Howard

--MORE--(79%)-------------------------------------------------------------------------------

Howard "I wood if I could" Cohen

hsc@sybase.com  {pacbell,pyramid,sun,{uunet,ucbvax}!mtxinu}!sybase!hsc

Sybase, Inc.    6475 Christie Avenue, Emeryville, CA 94608, 510-596-3406

End of article 4555 (of 4701)--what next? [npq] Article 4556 (145 more) in rec.woodworking:

From: ncramer@bbn.com (Nichael Cramer)

Subject: Query about teeth on Japanese Saws [was: Making frame&panel doors by hand]

Message-ID: <66446@bbn.BBN.COM>

Date: 22 Sep 91 17:31:57 GMT

References: <9935@cognos.UUCP> <15244@sybase.sybase.com>

Sender: news@bbn.com

Reply-To: ncramer@labs-n.bbn.com (Nichael Cramer)

Lines: 21

hsc@stroid.sybase.com (Howard Cohen) writes:

> ...  I have one of those very thin japanese pull-saws, and I

>have been extremley impressed with it so far.  It really is easy to use and

>makes a wonderful cut....

My experience with Japanese saws is totally limited to FW articles.  While

I've been very intrigued by what seems to be their obvious advantage (i.e.

cut on the draw ==> narrower blade ==> narrower kerf) I've been rather put

off by the apparent problems of maintaining the teeth (i.e. long, narrow

and "multi-faceted" ==> easy to break/bend and difficult/complicated to

sharpen).

--MORE--(71%)So my question is: Is there anything inherent in the nature of how the saw

works that required the teeth to be this way?  I guess what I'm imagining

is a hybrid of a thin japanese-like blade filed (re-filed?) with sturdier,

less-complicated western style teeth.  Do such beasts exist, or am I

missing something simple here?

Thanks

NICHAEL

nichael@bbn.com  --  "Dr Pepper -- it's not just for breakfast anymore."

End of article 4556 (of 4701)--what next? [npq] Article 4557 (144 more) in rec.woodworking:

From: whit@milton.u.washington.edu (John Whitmore)

Subject: Re: Query about teeth on Japanese Saws [was: Making frame&panel doors by hand]

Message-ID: <1991Sep22.231528.12817@milton.u.washington.edu>

Date: 22 Sep 91 23:15:28 GMT

References: <9935@cognos.UUCP> <15244@sybase.sybase.com> <66446@bbn.BBN.COM>

Organization: University of Washington, Seattle

Lines: 15

In article <66446@bbn.BBN.COM> ncramer@labs-n.bbn.com (Nichael Cramer) writes:

... [about Japanese hand saws]

>I've been very intrigued by what seems to be their obvious advantage (i.e.

>cut on the draw ==> narrower blade ==> narrower kerf) I've been rather put

>off by the apparent problems of maintaining the teeth (i.e. long, narrow

>and "multi-faceted" ==> easy to break/bend and difficult/complicated to

>sharpen).

I usually put coping saw blades in my coping saw in the

'cut on the draw' orientation.  Except for the difficulty of holding

to a line with such narrow blades, I find that they work quite well.

Blades are inexpensive and replaceable, of course.

--MORE--(98%)

John Whitmore

End of article 4557 (of 4701)--what next? [npq] Article 4558 (143 more) in rec.woodworking:

From: peterson@ucunix.san.uc.edu (John M. Peterson)

Subject: Re: treated lumber

Message-ID: <1991Sep23.120043.2722@ucunix.san.uc.edu>

Date: 23 Sep 91 12:00:43 GMT

Sender: peterson@ucunix.san.uc.edu (John M. Peterson)

Organization: Univ. of Cincinnati

Lines: 20

>  Are there any problems associated with routing salt treated lumber?

> (other than those associated with non-salt treated lumber)????

> I had heard that the bits wear down quicker because of the chemicals.

This depends on the kind of treatment, regular pressure treated, or

fire proofed (resistant).

PT presents no particular problem to tooling, the problem is you.

The dust and shaving are VERY toxic if inhaled or ingusted so:

wear a good resporater, cleanup all waste and dispose of carefully,

and wash all clothing          when finished (these are the industries recs).

Otherwise the stuff is fairly benign.

--MORE--(81%)

As forthe FP that I have seen, a steel saw blade was rendered useless after

riping an 8' 2X4.  Carbide is better but still wares faster with this

stuff.  I don't know anything about its toxity, does anyone else?

End of article 4558 (of 4701)--what next? [npq] Article 4559 (142 more) in rec.woodworking:

From: isjrc@dcatla.uucp (John R. Caruso)

Subject: Re: Special purpose Euro-hinges wanted

Message-ID: <4645.28dda3b7@dcatla.uucp>

Date: 23 Sep 91 08:20:07 GMT

References: <2875@atlas.cs.nps.navy.mil>

Organization: Digital Communications Associates, Alpharetta, GA

Lines: 25

In article <2875@atlas.cs.nps.navy.mil>, erickson@atlas.cs.nps.navy.mil (David E rickson) writes:

> I need some cabinet door hinges for European frameless cabinets with full-

> overlay doors that solve the following problem:

>

>   Is there a hinge which swings the door completely away from the

>   front of the cabinet (when opened more than 90 degrees)?

Grass America makes a concealed hinge that works for

inset as well as overlay. It does not require a pocket

in the door. I bought them locally at a cabinet supply

company for 3.50 each. I've seen them in the Trendline

catalog for 12.95/pair so the price is extremely variable.

--MORE--(73%)

When opened the door retracts from the frame approximately

1/2 to 3/4. I bought them for my kithen cabinet remodeling

job. My only criticism is that they only provided for

X or Y plane adjustment in each half of the hinge. It would

have been nice to have both adjustment planes in each half.

If you need more info, email me and I will try to get the

mfrs address.

jRc

End of article 4559 (of 4701)--what next? [npq] 4557

Article 4557 (142 more) in rec.woodworking:

From: whit@milton.u.washington.edu (John Whitmore)

Subject: Re: Query about teeth on Japanese Saws [was: Making frame&panel doors by hand]

Message-ID: <1991Sep22.231528.12817@milton.u.washington.edu>

Date: 22 Sep 91 23:15:28 GMT

References: <9935@cognos.UUCP> <15244@sybase.sybase.com> <66446@bbn.BBN.COM>

Organization: University of Washington, Seattle

Lines: 15

In article <66446@bbn.BBN.COM> ncramer@labs-n.bbn.com (Nichael Cramer) writes:

... [about Japanese hand saws]

>I've been very intrigued by what seems to be their obvious advantage (i.e.

>cut on the draw ==> narrower blade ==> narrower kerf) I've been rather put

>off by the apparent problems of maintaining the teeth (i.e. long, narrow

>and "multi-faceted" ==> easy to break/bend and difficult/complicated to

>sharpen).

I usually put coping saw blades in my coping saw in the

'cut on the draw' orientation.  Except for the difficulty of holding

to a line with such narrow blades, I find that they work quite well.

Blades are inexpensive and replaceable, of course.

--MORE--(98%)

John Whitmore

End of article 4557 (of 4701)--what next? [npq] 4556

Article 4556 (142 more) in rec.woodworking:

From: ncramer@bbn.com (Nichael Cramer)

Subject: Query about teeth on Japanese Saws [was: Making frame&panel doors by hand]

Message-ID: <66446@bbn.BBN.COM>

Date: 22 Sep 91 17:31:57 GMT

References: <9935@cognos.UUCP> <15244@sybase.sybase.com>

Sender: news@bbn.com

Reply-To: ncramer@labs-n.bbn.com (Nichael Cramer)

Lines: 21

hsc@stroid.sybase.com (Howard Cohen) writes:

> ...  I have one of those very thin japanese pull-saws, and I

>have been extremley impressed with it so far.  It really is easy to use and

>makes a wonderful cut....

My experience with Japanese saws is totally limited to FW articles.  While

I've been very intrigued by what seems to be their obvious advantage (i.e.

cut on the draw ==> narrower blade ==> narrower kerf) I've been rather put

off by the apparent problems of maintaining the teeth (i.e. long, narrow

and "multi-faceted" ==> easy to break/bend and difficult/complicated to

sharpen).

--MORE--(71%)So my question is: Is there anything inherent in the nature of how the saw

works that required the teeth to be this way?  I guess what I'm imagining

is a hybrid of a thin japanese-like blade filed (re-filed?) with sturdier,

less-complicated western style teeth.  Do such beasts exist, or am I

missing something simple here?

Thanks

NICHAEL

nichael@bbn.com  --  "Dr Pepper -- it's not just for breakfast anymore."

End of article 4556 (of 4701)--what next? [npq] 4558

Article 4558 (142 more) in rec.woodworking:

From: peterson@ucunix.san.uc.edu (John M. Peterson)

Subject: Re: treated lumber

Message-ID: <1991Sep23.120043.2722@ucunix.san.uc.edu>

Date: 23 Sep 91 12:00:43 GMT

Sender: peterson@ucunix.san.uc.edu (John M. Peterson)

Organization: Univ. of Cincinnati

Lines: 20

>  Are there any problems associated with routing salt treated lumber?

> (other than those associated with non-salt treated lumber)????

> I had heard that the bits wear down quicker because of the chemicals.

This depends on the kind of treatment, regular pressure treated, or

fire proofed (resistant).

PT presents no particular problem to tooling, the problem is you.

The dust and shaving are VERY toxic if inhaled or ingusted so:

wear a good resporater, cleanup all waste and dispose of carefully,

and wash all clothing          when finished (these are the industries recs).

Otherwise the stuff is fairly benign.

--MORE--(81%)

As forthe FP that I have seen, a steel saw blade was rendered useless after

riping an 8' 2X4.  Carbide is better but still wares faster with this

stuff.  I don't know anything about its toxity, does anyone else?

End of article 4558 (of 4701)--what next? [npq] Article 4560 (141 more) in rec.woodworking:

From: isjrc@dcatla.uucp (John R. Caruso)

Subject: Re: circ -> pseudo table saw : good or bad idea?

Message-ID: <4646.28dda7f9@dcatla.uucp>

Date: 23 Sep 91 08:38:17 GMT

References: <14845.28da1296@ecs.umass.edu>

Organization: Digital Communications Associates, Alpharetta, GA

Lines: 18

In article <14845.28da1296@ecs.umass.edu>, marsden@ecs.umass.edu writes:

> I'm a weekend woodworker on a tight budget, so gaze longingly at the

> It's frustrating watching Norm Abrams 'cause I know I'm a long way from

> having the variety of equipment he has (call it envy, I guess  *sigh*).

>

Gee, I thought I was the only one! I'll NEVER be able to aford Nom's

tools. I'd like him to enter my world with a 30 year old Rockwell

table saw, a taiwan drill press, and an AMT jointer. Here, 90

degree cuts are a major achievement. I saw him once putting up a

casing around a window with a pneumatic nailer. The guy

watching said "I guess you don't use hammers anymore." Norm replied

--MORE--(91%)Article 4561 (140 more) in rec.woodworking:

From: news@walter.bellcore.com (All the News - Period)

Subject: PC 690 Router question (was Re: Yet Another Router Question...)

Keywords: Router table mount

Message-ID: <1991Sep23.144254.2745@walter.bellcore.com>

Date: 23 Sep 91 14:42:54 GMT

References: <20231@exodus.Eng.Sun.COM> <1991Sep20.153054.11784@mlb.semi.harris.c om>

Reply-To: marc@bellcore.com (Marc F Pucci)

Organization: Bellcore, Morristown, NJ

Lines: 9

Nntp-Posting-Host: breeze

I too am looking for a good router.  From the earlier discussion

it sounds like the PC 690 has a changeable base (plunge and standard?).

Does it make sense to get both bases and use the standard in a router

table and the plunge for hand work?  Is it easy to switch between the two?

Marc Pucci

marc@bellcore.com

End of article 4561 (of 4701)--what next? [npq] Article 4562 (139 more) in rec.woodworking:

From: bryan@intvax2.cs.sandia.gov (Jon R Bryan)

Subject: Re: What to do, how to spend it.

Message-ID: <1991Sep23.141635.5104@intvax2.cs.sandia.gov>

Date: 23 Sep 91 14:16:35 GMT

References: <4890171@hpnmdla.sr.hp.com>

Organization: Sandia National Labs, Org. 1411, Albq., NM

Lines: 17

From article <4890171@hpnmdla.sr.hp.com>, by stanj@hpnmdla.sr.hp.com (Stan Jaffe ):

> In rec.woodworking, chrisi@lloyd.Camex.COM (Chris Ischay) writes:

>

>     My only complaint is that my thumbs sometimes get very hot.

>

>     Chris Ischay

>

> I have not actually tried this, but someone once suggested using those

> little rubber finger cots that clerical workers use when handling paper.

I read a suggestion somewhere (Fine Woodworking?) that magnetic strips

work well for this, too.  Finger cots would be a hassle to keep track

of.  A magnet strip you could leave stuck to the scraper.

--MORE--(88%)

--

Jon R. Bryan    (505) 844-2015  <=>     bryan@istg.cs.sandia.gov

Sandia National Laboratories, Intelligent Machine Principles Division

End of article 4562 (of 4701)--what next? [npq] Article 4563 (138 more) in rec.woodworking:

From: kco@sppy00.UUCP (O'NEIL KEVIN)

Subject: query on some kind of bendable wood

Keywords: wood, bendable

Message-ID: <1394@sppy00.UUCP>

Date: 23 Sep 91 12:40:48 GMT

Distribution: usa

Organization: Online Computer Library Center, Dublin, Ohio.

Lines: 14

I am in the process of making a drum (a tapan actually).  I have a shell

but I need to make "tone rings" (that go inside the shell) and "hoops" that

hold the skins on each end.  I'm looking for wood that is shapable or

bendable to make these items.  A friend told me of something called

"bendable birch" but I haven't been able to locate any locally.  I'd

very much appreciate any suggestions anyone has on a substitute or sources.

Please E-mail responses.

Thanks.

--

Kevin C. O'Neil  Phone: (614) 764-6271        Reverse cannot befall that

OCLC Inc.        Internet: kco@rsch.oclc.org  fine prosperity whose

6565 Frantz Rd.  UUCP:...!osu-cis!sppy00!kco  sources are interior.

--MORE--(94%)Article 4564 (137 more) in rec.woodworking:

From: ig@caliban.uucp (Iain Bason)

Subject: Motor power

Message-ID: <1991Sep20.160046.274@caliban.uucp>

Date: 20 Sep 91 16:00:46 GMT

Organization: none

Lines: 15

The recent discussion about the power limits of saws that run off

110 volts was very interesting.  But it just occurred to me that the

same limits ought to apply to routers, yet reputable companies

claim to sell 3 h.p. routers.  Does anyone know how much power they

actually produce?

--

Iain Bason

..uunet!caliban!ig

End of article 4564 (of 4701)--what next? [npq] Article 4565 (136 more) in rec.woodworking:

From: twl10@ccc.amdahl.com (Tom Littauer)

Subject: Re: circ -> pseudo table saw : good or bad idea?

Summary: Normie can be envious too

Message-ID: <85nF029H00Wn01@JUTS.ccc.amdahl.com>

Date: 23 Sep 91 16:00:02 GMT

References: <14845.28da1296@ecs.umass.edu>

Reply-To: twl10@JUTS.uts.amdahl.com (Tom Littauer)

Organization: Amdahl Corporation, Sunnyvale CA

Lines: 9

In article <14845.28da1296@ecs.umass.edu> marsden@ecs.umass.edu writes:

>

>It's frustrating watching Norm Abrams 'cause I know I'm a long way from

>having the variety of equipment he has (call it envy, I guess  *sigh*).

If it makes you feel any better, there *ARE* shops that make Normie's

look like ours. I was in a fulltime furniture-maker's shop taking a class

and was awed. 12" jointer. 30" planer. 40" wide belt sander. Table

saw w/ sliding table 48" wide. and on and on...

End of article 4565 (of 4701)--what next? [npq] Article 4566 (135 more) in rec.woodworking:

From: wilson@schaefer.math.wisc.edu (Bob Wilson)

Subject: Re: What to do, how to spend it.

Message-ID: <1991Sep23.155155.8977@schaefer.math.wisc.edu>

Date: 23 Sep 91 15:51:55 GMT

References: <4890171@hpnmdla.sr.hp.com> <1991Sep23.141635.5104@intvax2.cs.sandia .gov>

Reply-To: wilson@math.wisc.edu@schaefer.UUCP (Bob Wilson)

Organization: Univ. of Wisconsin Dept. of Mathematics

Lines: 23

In article <1991Sep23.141635.5104@intvax2.cs.sandia.gov> bryan@intvax2.cs.sandia .gov (Jon R Bryan) writes:

>From article <4890171@hpnmdla.sr.hp.com>, by stanj@hpnmdla.sr.hp.com (Stan Jaff e):

>> In rec.woodworking, chrisi@lloyd.Camex.COM (Chris Ischay) writes:

>>

>>     My only complaint is that my thumbs sometimes get very hot.

>>

>>     Chris Ischay

>>

>> I have not actually tried this, but someone once suggested using those

>> little rubber finger cots that clerical workers use when handling paper.

--MORE--(64%)Article 4567 (134 more) in rec.woodworking:

From: schnable@cbnewsc.cb.att.com (andrew.schnable)

Subject: Re: PC 690 Router question (was Re: Yet Another Router Question...)

Message-ID: <1991Sep23.162348.20807@cbnewsc.cb.att.com>

Date: 23 Sep 91 16:23:48 GMT

References: <1991Sep23.144254.2745@walter.bellcore.com>

Organization: AT&T Bell Laboratories

Lines: 15

From article <1991Sep23.144254.2745@walter.bellcore.com>, by news@walter.bellcor e.com (All the News - Period):

>

> I too am looking for a good router.  From the earlier discussion

> it sounds like the PC 690 has a changeable base (plunge and standard?).

>

> Does it make sense to get both bases and use the standard in a router

> table and the plunge for hand work?  Is it easy to switch between the two?

I think so - it's what I did.  I bought the router with the standard base

base and used it in my router table and out of the table for a year.  When

I scraped a little more cash together, I bought the plunge base and

now leave my normal base attached to the table. Works great - yes it is easy

to shift between the bases.

--MORE--(98%)Article 4568 (133 more) in rec.woodworking:

From: frank@marvin.contex.com (Frank Perdicaro)

Subject: What tools to buy

Keywords: my thoughts

Message-ID: <2111@contex.contex.com>

Date: 23 Sep 91 16:53:57 GMT

Sender: news@contex.contex.com

Lines: 68

I cannot resist throwing my $0.02 in about how to spend $.  First let

me say that my volume woodworking has consisted of framing houses, and

that I still do many practical woodworking projects.  I have also made

beds, tables, cabinets, etc with solid woods and fine finishes.  These

projects are slow but rewarding.  Also, I find that I get most of my

lumber from other people's trash: a great many people throw out old

furniture.  I have gotten around 50 slabs of laminated hardwoods

from trashed old dressers in Lowell, MA alone.  All the trusses in

my garage roof are made of trash lumber ( and 5000 screws ).

As for tools, I got a good circular saw first.  I made some good,

heavy stacking saw horses, and bought a good clamping straightedge.

I have used it on all my furniture.  It is not quite as fast as my

table saw, but due to experience, I find I have better control

--MORE--(29%)with the circular.  For cut off duties, I bought a 10" cutoff

saw and the Delta 80 tooth, copper-plugged cut off blade.  I made a table

for it that includes an extension on one side, an outlet, and

*importantly* a good light.  There is also a slot for the tape

measure and speed square.  I love it.  For ripping of odd stock

and some repetative cuts, I bought the <$300 aluminium tabled

Delta saw.  Junk.  I have to be very patient with it.  It was

improoved quite a bit by the addition of Shop Hands.  For drilling,

I have a Makita 6012HD rechargable.  It is great for making pilot

holes and puhsing screws.  I use my Milwakee 1/2" Holeshooter

for other holes, and in combination with a Blu-Mol holesaw set,

I find I can drill up to about 4" freehand.  I do not like to

sand but was given a Milwakee half sheet random and a Sears

4*21 belt.

I have some hand tools also.  I find a good mix of hammers important.

I got or made a light finish nailing, 22 oz framer, 36 oz pounder,

20 oz rubber, 16 oz wood and a ~50 oz wood.  Clamps are important,

bar and C.  Also, I got for free 10 hand planes;  I use these

infrequently, but converted one to a ski sharpener.  Various files

and chisels come in handy.  Measuring tools are important.  I have

eight squares and eight measuring tapes of various description.

Good planning, measuring and marking makes the cutting fast and

--MORE--(62%)easy, and the assembly a joy.

A Bench is important, but can be very expensive.  Mine is made out of

discarded 4" t&g retaining wall stuff.  I use scrap iron for ballast.

Mass of bench is quite high, and Cg quite low.  I spent the $ on a

pair of ok vices from Enco.

It will sound trite, but the best tools I have are my safety

equipment.  I went and bought Uvex clear goggels -- like ski

goggels, but for the shop.  Very comfortable and full coverage.

I also got some Remington ear muffs designed for shooting.  They

are comfortable when worn for hours.  To breath, I bought an

asbestos-rated, four strap, dual cartridge mask.  I also picked up

a lab coat and a good broom.  I can come away from a few hours of

woodworking clean, breathing eaisly and able to hear well, but with

dusty shoes.  The broom is used to push the clutter into one pile.

I have had at various times a RAS and a joiner.  I would next like a

drill press.  I would not change the order in which I got tools.

I still find the circular saw most versatile.  The cut off saw

compliments the circular saw perfectly.  I think I have all the

benefits of RAS & a TS without the size and cost.  Good safety

equipment makes the wearing not just acceptable, but pleasant.

--MORE--(92%)It becomes habit forming.  I blew your budget, but these are my

thoughts.

--

Frank E Perdicaro, Systems Admin, etc.          Xyvision Design Systems

LEGALIZE Guns, drugs and cash...today.               101 Edgewater Drive

inhouse:  frank@marvin, X5572                           Wakefield MA

outhouse: frank@contex.com, 617-224-5572                018801285

End of article 4568 (of 4701)--what next? [npq] Article 4569 (132 more) in rec.woodworking:

From: smb@ulysses.att.com (Steven Bellovin)

Subject: Re: circ -> pseudo table saw : good or bad idea?

Message-ID: <15529@ulysses.att.com>

Date: 23 Sep 91 16:57:50 GMT

References: <14845.28da1296@ecs.umass.edu> <4646.28dda7f9@dcatla.uucp>

Sender: netnews@ulysses.att.com

Lines: 14

In article <4646.28dda7f9@dcatla.uucp>, isjrc@dcatla.uucp (John R. Caruso) write s:

} Gee, I thought I was the only one! I'll NEVER be able to aford Nom's

} tools. I'd like him to enter my world with a 30 year old Rockwell

} table saw, a taiwan drill press, and an AMT jointer. Here, 90

} degree cuts are a major achievement. I saw him once putting up a

} casing around a window with a pneumatic nailer. The guy

} watching said "I guess you don't use hammers anymore." Norm replied

} "Naaah, nobody uses a hammer anymore."

Umm -- I beg to differ.  I very strongly suspect that that Norm could

do a far better job with my tools than I could with his...  (Btw, remember

that during the first season, he was using things like a Sears radial

arm saw and a Shopsmith table saw.  Not necessarily garbage -- but

--MORE--(97%)Article 4570 (131 more) in rec.woodworking:

From: fskanes@kean.ucs.mun.ca

Subject: Request: Log Cabin Plans

Message-ID: <1991Sep23.151456.1@kean.ucs.mun.ca>

Date: 23 Sep 91 17:44:56 GMT

Sender: usenet@morgan.ucs.mun.ca (NNTP server account)

Organization: Memorial University. St.John's Nfld, Canada

Lines: 4

Anybody know where i can find out about log cabin biulding?

I am wondering if there are plans that I can get electronically?

Any help would be appreciated. Post or

E-mail....fskanes@kean.ucs.mun.ca

End of article 4570 (of 4701)--what next? [npq] Article 4571 (130 more) in rec.woodworking:

From: evh@genie.slhs.udel.edu (Troy Saville)

Subject: Re: norm..blah blah blah

Message-ID: <1991Sep23.175405.8834@genie.slhs.udel.edu>

Date: 23 Sep 91 17:54:05 GMT

References: <14845.28da1296@ecs.umass.edu> <4646.28dda7f9@dcatla.uucp>

Organization: UDel, School of Life & Health Sciences

Lines: 22

In article <4646.28dda7f9@dcatla.uucp> isjrc@dcatla.uucp (John R. Caruso) writes :

>In article <14845.28da1296@ecs.umass.edu>, marsden@ecs.umass.edu writes:

>> I'm a weekend woodworker on a tight budget, so gaze longingly at the

>> It's frustrating watching Norm Abrams 'cause I know I'm a long way from

>> having the variety of equipment he has (call it envy, I guess  *sigh*).

>

>Gee, I thought I was the only one! I'll NEVER be able to aford Nom's

>tools. I'd like him to enter my world with a 30 year old Rockwell

>table saw, a taiwan drill press, and an AMT jointer. Here, 90

>degree cuts are a major achievement. I saw him once putting up a

>casing around a window with a pneumatic nailer. The guy

>watching said "I guess you don't use hammers anymore." Norm replied

>"Naaah, nobody uses a hammer anymore." I don;t know who he is supposed

--MORE--(75%)>to appeal to but it isn't me.

I thought woodworking/carpentry was Norm's job. So its understandable

why he has all those tools and 3 walls of pipe clamps. I wouldn't

doubt it if he writes all those tools off as a business expense.

--

- Maryland State Frisbee Championships,Seneca Creek State Park, Sept. 21/22, 91'

evh@genie.slhs.udel.edu    - 92' PDGA points tournament, AWESOME course

End of article 4571 (of 4701)--what next? [npq] Article 4572 (129 more) in rec.woodworking:

From: sos@spsd4360a.erim.org (Tim Sos)

Subject: Re: Ripping on a RAS (was Safety)

Message-ID: <SOS.91Sep23132002@spsd4360a.erim.org>

Date: 23 Sep 91 20:20:02 GMT

References: <15230011@hpspdla.spd.HP.COM> <15124@sybase.sybase.com>

<1264@gboro.glassboro.edu>

<1991Sep18.214002.14520@murdoch.acc.Virginia.EDU> <4870@happym.WA.COM>

Sender: jgarb@newsspool.erim.org (Joe Garbarino)

Organization: Environmental Research Institute of Michigan, Ann Arbor, Michigan

Lines: 23

In-reply-to: josh@happym.WA.COM's message of 20 Sep 91 20:04:12 GMT

Yet another horror story - well a warning anyway...

I have noticed that there are time when a RAS is used in what seems to

be correctly and it will still launch boards into orbit.

When ripping square items bigger than you can crosscut (like >1 foot

by >1 foot), there is a very good chance that the sqaure of wood

between the fence and the blade will be pulled back thru the saw (as

it climbs over the top of the corner).  This is expecally true of thin

materials (like 1/8" masonite).  I used to work at a sign shop and never

--MORE--(66%)had any major problems until I had to cut a bunch of these square

signs -- Well, the first square masonite was pulled thru the saw,

just missed me and hit the wall at high speed.  The square sheet was

left with a very interesting circular "blade" track.

The moral of the story is that there are some sizes of cuts that can

bring out the worst in a table saw...

--

Tim (or Garrett) Sos  sos@erim.org or sos@amadeus.erim.org

--  all standard disclamers apply --

Environmental Research Institute of Michigan, Signal Processing Department

ERIM, P.O. Box 134001, Ann Arbor, MI 48113-40001,  (313)994-1200 Ext. 2848

End of article 4572 (of 4701)--what next? [npq] Article 4573 (128 more) in rec.woodworking:

From: wainscot@rosarita.berkeley.edu (Brian Wainscott)

Subject: Building a grinder

Message-ID: <1991Sep23.184441.19781@agate.berkeley.edu>

Date: 23 Sep 91 18:44:41 GMT

Sender: usenet@agate.berkeley.edu (USENET Administrator)

Distribution: na

Organization: U.C. Berkeley Math. Department.

Lines: 25

Originator: wainscot@rosarita.berkeley.edu

I'm still in search of a slow grinder, and am thinking of building my

own.  (I think I can get the motor for free, and the rest shouldn't

be complicated.)  So, do any of you folks know a good source for a

grinding wheel?  The Delta slow grinder has a 2" wide wheel, and all

the grinding supply places around here have only 1" wheels.  So far

I've found (local) 6"x1" Aluminum Oxide 60 grit wheels for 'prox $15

("general purpose" gray wheel) and $30 for a white wheel.

Can anyone tell my what the difference between these is, which I should

use, and where I might be able to get a 6" or 10" wheel that is wider?

As long as I'm building a grinder, there is no reason why it should have

--MORE--(68%)only one stone on it.  For that matter I could put a felt wheel, and/or

a leather covered "stropping" wheel on it also.

Question:  If you were building something like this, what would YOU do?

Thanks for all the help!

--

Brian Wainscott     \=========================================================

Graduate Student      \ "I want to get so close to Him, that its no big change

U.C. Berkeley           \ on the day that Jesus calls my name." - Wayne Watson

wainscot@math.berkeley.edu\===================================================

End of article 4573 (of 4701)--what next? [npq] Article 4574 (127 more) in rec.woodworking:

From: sesv@cbnewsc.cb.att.com (steven.e.sommars)

Subject: Safety Guidelines from Power Tool Institute

Keywords: safety

Message-ID: <1991Sep23.184548.24744@cbnewsc.cb.att.com>

Date: 23 Sep 91 18:45:48 GMT

Distribution: na

Organization: AT&T Bell Laboratories

Lines: 18

At a recent woodworking show I picked up a safety brochure from the

Power Tool Institute.  The specific brochure was titled ``Safety is

Specific'' (SIS-585) It has twenty pages covering circular saws,

routers, radial arm saws, lathes, drills, planers, etc.

I phoned PTI today.  They will send out single samples

of the brochure on *written request*, but there is a cost for

bulk orders (e.g., if a club wanted to order 100 copies).

Power Tool Institute, Inc.

P.O.Box 818

Yachats, OR 97498-0818

Phone numbers for other inquiries:

503-547-3185

--MORE--(88%)        503-547-3539 (fax)

I think the brochure is good, but would like to hear other opinions.

--Steve Sommars (no connection with PTI)

End of article 4574 (of 4701)--what next? [npq] Article 4575 (126 more) in rec.woodworking:

From: chrisi@lloyd.Camex.COM (Chris Ischay)

Subject: Re: Wood Carving Tools

Message-ID: <2386@camex.COM>

Date: 23 Sep 91 18:41:59 GMT

References: <1590001@hpfcdq.fc.hp.com>

Sender: news@Camex.COM

Organization: Camex Inc., Boston MA

Lines: 17

In article <1590001@hpfcdq.fc.hp.com> banks@hpfcdq.fc.hp.com (Mike Banks)

writes:

>Where can I get a good set of wood carving tools?

>What companies make them?

>Do any of the mail order catalogs carry them?

Garrett Wade and Woodcraft are two catalogs that have lots of tools,

books, videos etc. for carvers.  Both have 800 numbers available from

800-555-1212.  Sorry, I don't know what they are offhand.

Be forewarned.  These catalogs do not cater to the low-budget-minded.

In fact, some of my tools are less expensive than the GW catalog itself!

--MORE--(90%)But beautiful stuff.  Also wonderful for finishing products.

Chris Ischay

chrisi@bos.camex.com

End of article 4575 (of 4701)--what next? [npq] Article 4576 (125 more) in rec.woodworking:

From: burkett@convex.com (Phillip Burkett)

Subject: Re: Need advice: looking for a slow grinder.

Message-ID: <burkett.685649638@aspen>

Date: 23 Sep 91 18:13:58 GMT

References: <1991Sep22.034249.3950@agate.berkeley.edu>

Sender: usenet@convex.com (news access account)

Distribution: na

Organization: CONVEX Computer Corporation, Richardson, Tx., USA

Lines: 4

Nntp-Posting-Host: aspen.convex.com

"slow grinder"

(:/  I think I dated someone like that once.

sorry couldn't resist

End of article 4576 (of 4701)--what next? [npq] Article 4577 (124 more) in rec.woodworking:

From: psh@wg.estec.nl (Per Hemsoe)

Subject: Plans for Shaker furniture !

Keywords: Shaker, furniture plans

Message-ID: <1991Sep23.153414.380@wg.estec.nl>

Date: 23 Sep 91 15:34:14 GMT

Sender: news@wg.estec.nl (news admin)

Reply-To: psh@wg.estec.nl

Organization: ESTEC/WG, Noordwijk, The Netherlands

Lines: 14

Originator: psh@

Nntp-Posting-Host: estwg9

My wife and I have, through reading of various American magazines on housing

and especially furniture, got very interested in SHAKER style furniture, and

have decided to add building of this type of furniture to the my favourite

hobby (you guessed it right, woodworking, mainly building eskimo kayaks from

wood and canvas). We have although not been sucessful in finding any plans

and/or litterature of/on this type of furniture here in Holland, so if anybody

could help me with some titles of books with plans and/or descriptions of

Shaker furniture I will be very grateful. I could then go to the local

--MORE--(94%)bookstore and ask them to order it for me.

Thanks, Per.

End of article 4577 (of 4701)--what next? [npq] Article 4578 (123 more) in rec.woodworking:

From: chrisi@lloyd.Camex.COM (Chris Ischay)

Subject: Best of both worlds, was Query about teeth on Japanese Saws

Message-ID: <2387@camex.COM>

Date: 23 Sep 91 19:35:55 GMT

References: <15244@sybase.sybase.com> <66446@bbn.BBN.COM> <1991Sep22.231528.1281 7@milton.u.washington.edu>

Sender: news@Camex.COM

Organization: Camex Inc., Boston MA

Lines: 31

In article <1991Sep22.231528.12817@milton.u.washington.edu>

whit@milton.u.washington.edu (John Whitmore) writes:

|>In article <66446@bbn.BBN.COM> ncramer@labs-n.bbn.com (Nichael Cramer)

writes:

|>>I've been very intrigued by what seems to be their obvious advantage (i.e.

|>>cut on the draw ==> narrower blade ==> narrower kerf) I've been rather put

|>>off by the apparent problems of maintaining the teeth (i.e. long, narrow

|>>and "multi-faceted" ==> easy to break/bend and difficult/complicated to

|>>sharpen).

--MORE--(48%)|>      I usually put coping saw blades in my coping saw in the

|>'cut on the draw' orientation.  Except for the difficulty of holding

|>to a line with such narrow blades, I find that they work quite well.

|>Blades are inexpensive and replaceable, of course.

I wound up doing nearly the same thing, to solve a problem.  I had noticed

that every cut I made with my bowsaw was a little crooked.  It didn't

bother me enough to attend to it until I had a *really* long rip cut to

make.  That's when I found out one handle on the bowsaw was bent.  Of

course there's two handles on this thing so I just turned it around and

started pulling instead of pushing.  Bingo.  I invented the Japanese

bowsaw!  I'm not crazy about the way pull-cutting fuzzes up the cut line,

but it's otherwise a great way to do things.  I find I can get steadier

motion on the pull.  I don't know how complicated japanese teeth are to set

and file, but bowsaws are pretty forgiving.  And like a coping saw, you can

turn the blade 90 degrees and rip a 6' board.

Chris Ischay

chrisi@bos.camex.com

End of article 4578 (of 4701)--what next? [npq] Article 4579 (122 more) in rec.woodworking:

From: dolata@lead.uazaic.arizona.edu (Dolata)

Subject: Re: norm..blah blah blah

Summary: Norm as Turner???

Message-ID: <1991Sep23.131054.1596@arizona.edu>

Date: 23 Sep 91 20:10:53 GMT

References: <14845.28da1296@ecs.umass.edu> <4646.28dda7f9@dcatla.uucp> <1991Sep2 3.175405.8834@genie.slhs.udel.edu>

Sender: dolata@lead.aichem.arizona.edu

Distribution: world,local

Organization: University of Arizona AI Chemistry Lab, Tucson, AZ

Lines: 8

Nntp-Posting-Host: lead.aichem.arizona.edu

This Saturday our local PBS showed Norm building a Shaker inspired drop

leaf table.

Any other turners out there?   What did you think of Norm's lathe

technique?

Dan

End of article 4579 (of 4701)--what next? [npq] Article 4580 (121 more) in rec.woodworking:

From: dolata@lead.uazaic.arizona.edu (Dolata)

Subject: Re: Wood Carving Tools

Summary: Make em...

Message-ID: <1991Sep23.132839.1597@arizona.edu>

Date: 23 Sep 91 20:28:38 GMT

References: <1590001@hpfcdq.fc.hp.com>

Sender: dolata@lead.aichem.arizona.edu

Distribution: world,local

Organization: University of Arizona AI Chemistry Lab, Tucson, AZ

Lines: 33

Nntp-Posting-Host: lead.aichem.arizona.edu

Carving tools are soooo expensive I didn't want to take up the craft, for

fear it would be a never ending sink for money.   Then I met a woodworker

who showed me how easy and cheap it is to make a lot of your own tools.

I had a forge made out of a 50 gallon drum bottom and some tubing welded

on.  I lined it with clay,  and put it near my shop.  I use the outlet

from my shop vac for the air source.   I got a 600 pound

block of scrap iron for an anvil, and bought a little anvil for fine shaping.

A few hammers, pliers and tongs from the local swap meet, and for less than

--MORE--(48%)$75, viola,  I'm set!

I use old files.  I anneal them,  grind or hammer out the teeth,  and then

shape them with a combination of forging and grinding techniques.  The forging

defines the sweep,  the grinding refines it and is how I make fishtail

gouges, etc...    I can make all sorts of gouges, chisels, bent gouges and

larger vieners with no trouble.

I can't make small vieners, V tools, and macaroni tools. My spoon gouges

aren't very good, yet.   I need a swage block for that.

There are several books out there that actually tell you how to make your

own carving tools!   I am at work,  so I don't know the names or authors.

But if you are dextrous enough to learn carving,  you can easily learn

how to make simple tools.

This is the only way I can afford a large enough set of tools.   And, you

can make tools for the specific job your are doing.  Just

a little while ago I decided I needed a 1/8" wide, spear pointed bent chisel

for finishing some details in the folds of a leek I was carving...   It

took me about 3 hours,  but I now have a set of three.  Total cost,  about

$2 - 3 for the coal and about $1 for the dead files,  and three small

bits of wood for the turned handles.

End of article 4580 (of 4701)--what next? [npq] Article 4581 (120 more) in rec.woodworking:

From: jim@jagubox.gsfc.nasa.gov (Jim Jagielski)

Subject: Norm's Tools (Was: Re: norm..blah blah blah)

Message-ID: <257@jagubox.gsfc.nasa.gov>

Date: 24 Sep 91 10:51:13 GMT

References: <14845.28da1296@ecs.umass.edu> <4646.28dda7f9@dcatla.uucp> <1991Sep2 3.175405.8834@genie.slhs.udel.edu>

Sender: usenet@nsisrv.gsfc.nasa.gov (Usenet)

Reply-To: jim@jagubox.gsfc.nasa.gov (Jim Jagielski)

Organization: NASA/Goddard Space Flight Center

Lines: 39

Nntp-Posting-Host: jagubox.gsfc.nasa.gov

evh@genie.slhs.udel.edu (Troy Saville) writes:

>In article <4646.28dda7f9@dcatla.uucp> isjrc@dcatla.uucp (John R. Caruso) write s:

>>In article <14845.28da1296@ecs.umass.edu>, marsden@ecs.umass.edu writes:

>>> I'm a weekend woodworker on a tight budget, so gaze longingly at the

>>> It's frustrating watching Norm Abrams 'cause I know I'm a long way from

>>> having the variety of equipment he has (call it envy, I guess  *sigh*).

>>

>>Gee, I thought I was the only one! I'll NEVER be able to aford Nom's

--MORE--(41%)>>tools. I'd like him to enter my world with a 30 year old Rockwell

>>table saw, a taiwan drill press, and an AMT jointer. Here, 90

>>degree cuts are a major achievement. I saw him once putting up a

>>casing around a window with a pneumatic nailer. The guy

>>watching said "I guess you don't use hammers anymore." Norm replied

>>"Naaah, nobody uses a hammer anymore." I don;t know who he is supposed

>>to appeal to but it isn't me.

>I thought woodworking/carpentry was Norm's job. So its understandable

>why he has all those tools and 3 walls of pipe clamps. I wouldn't

>doubt it if he writes all those tools off as a business expense.

It's a real eye-opener when you look at his shop in the 1st season of NYW

and in the last. In the 1st, he has lots of space and just a few basic

stationary tools (table saw, radial-arm, jointer, shopsmith and maybe a few

more) but he ALWAYS has had gobs of portable tools (esp. drills and routers).

Of course, companies give Norm more tools which he then uses on his show

(as well as on TOH) and the exposure boosts sales...

He still uses the same table saw and jointer though... these must have special

meaning to him...

--

==============================================================================

--MORE--(88%)Article 4582 (119 more) in rec.woodworking:

From: stanj@hpnmdla.sr.hp.com (Stan Jaffe)

Subject: Re: Need advice: looking for a slow grinder.

Message-ID: <4890172@hpnmdla.sr.hp.com>

Date: 23 Sep 91 21:14:31 GMT

References: <1991Sep22.034249.3950@agate.berkeley.edu>

Organization: HP Network Measurements Div, Santa Rosa, CA

Lines: 23

I'm in a similar situation: looking for an inexpensive solution to the

grinding problem.  Here's what I've found:

The big advantage of using a wheel grinder is that the blade surface is now

hollow-ground.  This means that when you progress to your flat stones after

grinding, the surface area in contact with the stone is minimal.  Now you have

only a small amount of metal to sharpen/hone.

I have not decided yet, but am considering running a belt and two pulleys

from a motor (my present high speed grinder, perhaps) to a mandrel.  The

mandrel will have a 6" "friable" (spelling?) wheel on it.  I'm looking at

gearing down to a few hundred RPM or less.  The idea is to avoid overheating

the tool, so quenching it in water while grinding is another approach.

--MORE--(79%)I have a professional woodworker friend (a Krenov student, BTW) who has used

the above approach with much success.

Stan Jaffe    A.K.A.   stanj@sr.hp.com

HP Santa Rosa, Calif.

"Not responsible for any opinions but my own"

------------------------------------------------------------------

End of article 4582 (of 4701)--what next? [npq] Article 4583 (118 more) in rec.woodworking:

From: jps@uh.msc.edu (Jeff P. Sorvik)

Subject: BETMAR tools : INFO NEEDED FAST

Keywords: bandsaw betmar tools

Message-ID: <4904@uc.msc.umn.edu>

Date: 24 Sep 91 13:32:37 GMT

Sender: news@uc.msc.umn.edu

Reply-To: jps@msc.umn.edu

Organization: Minnesota Supercomputer Center

Lines: 16

I have an opportunity to buy an older saw.  It is a Betmar 14" bandsaw.  Does

anyone have any information on this brand?  Is the quality good?  What about

spare parts?  I need to act fast, so quick responses would be appreciated.

Thanks,

_                  _

( )                ( )

/\__/~\-/______________/~\-/_/\

|   ----/-----sawyer-------/-   |

_______________________________\_____/__________________/____/_____________

|Jeff Sorvik (jps@msc.edu)      | "He is no fool who gives up that which    |

--MORE--(76%)Article 4584 (117 more) in rec.woodworking:

From: ISW@cup.portal.com (Isaac S Wingfield)

Subject: What kind of wood is this?

Message-ID: <47632@cup.portal.com>

Date: 24 Sep 91 00:16:32 GMT

Organization: The Portal System (TM)

Lines: 17

We recently got, at a garage sale, something described as "an old sea chest";

it was covered with Naugahyde (glued on),but the underlying wood is nice,

and the chest has hand-cut dovetails and handmade brass bindings on the

corners.

Peeling off the Naugahyde left a coating of adhesive which needs to

be removed, and the wood generally needs to be sanded and cleaned up. When

I hit it with sandpaper, the scent from the wood was very strong - I'd have

to guess camphor. I know what other aromatic woods smell like; it's not

eucalyptus or cedar or any of the pine family that I'm familiar with.

Could it be camphor? What does that look like? How do I refinish it? Is

camphor toxic to work with? How careful do I need to be when sanding it?

Any help will be appreciated.

--MORE--(97%)Article 4585 (116 more) in rec.woodworking:

From: bds@flounder.linus.mitre.org (Barry D. Smith)

Subject: Re: circ -> pseudo table saw : good or bad idea?

Message-ID: <BDS.91Sep24085216@flounder.linus.mitre.org>

Date: 24 Sep 91 13:52:16 GMT

References: <14845.28da1296@ecs.umass.edu> <4646.28dda7f9@dcatla.uucp>

Sender: news@linus.mitre.org (News Service)

Organization: The MITRE Corporation

Lines: 31

In-Reply-To: isjrc@dcatla.uucp's message of 23 Sep 91 08:38:17 GMT

Nntp-Posting-Host: flounder.mitre.org

In article <14845.28da1296@ecs.umass.edu>, marsden@ecs.umass.edu writes:

>> I'm a weekend woodworker on a tight budget, so gaze longingly at the

>> It's frustrating watching Norm Abrams 'cause I know I'm a long way from

>> having the variety of equipment he has (call it envy, I guess  *sigh*).

>Gee, I thought I was the only one! I'll NEVER be able to aford Nom's

>tools. I'd like him to enter my world with a 30 year old Rockwell

>table saw, a taiwan drill press, and an AMT jointer. Here, 90

>degree cuts are a major achievement.

--MORE--(48%)Article 4586 (115 more) in rec.woodworking:

From: brian@athena.cs.uga.edu (Brian Boe)

Subject: Re: Plans for Shaker furniture !

Keywords: Shaker, furniture plans

Message-ID: <1991Sep24.023029.21068@athena.cs.uga.edu>

Date: 24 Sep 91 02:30:29 GMT

References: <1991Sep23.153414.380@wg.estec.nl>

Organization: University of Georgia, Athens

Lines: 35

Per Hemsoe (psh@wg.estec.nl) writes:

>

>

>My wife and I have, through reading of various American magazines on housing

>and especially furniture, got very interested in SHAKER style furniture, and

>have decided to add building of this type of furniture to the my favourite

>hobby (you guessed it right, woodworking, mainly building eskimo kayaks from

>wood and canvas). We have although not been sucessful in finding any plans

>and/or litterature of/on this type of furniture here in Holland, so if anybody

>could help me with some titles of books with plans and/or descriptions of

>Shaker furniture I will be very grateful. I could then go to the local

>bookstore and ask them to order it for me.

>

--MORE--(60%)>

>Thanks, Per.

Here are some I own or have seen.  I haven't tried any of the plans in

them yet, though.

Measured Shop Drawings for American Furniture, Thomas Moser, Sterling

Publishing Co. Inc., New York, 1985 (ISBN 0-8069-6792-7 paper;

0-8069-5712-3 cloth)  U.S. $14.95 (paper), 320 pp. of drawings and

photographs.

How to Build Shaker Furniture, same author; has some plans if I recall

correctly.

Shop Drawings of Shaker Furniture and Woodenware, vols. I, II, III,

Ejner Handberg, Berkshire Traveller Press, Stockbridge, MA 01262,

1973, 1975, 1977 (0-912944-09-9, -29-3, -45-5), $7.95 each (paper),

each volume approx. 85 pages of drawings plus a few photos.

I hope this helps.

--Brian (brian@joe.math.uga.edu, brian@athena.cs.uga.edu)

End of article 4586 (of 4701)--what next? [npq] Article 4587 (114 more) in rec.woodworking:

From: bbenson@hpdmd48.boi.hp.com (Brian Benson)

Subject: Re: Installing a Hardwood Floor

Message-ID: <3050039@hpdmd48.boi.hp.com>

Date: 23 Sep 91 20:58:23 GMT

References: <91252.151337DM3WFINR@MIAMIU.BITNET>

Organization: Hewlett Packard - Boise, ID

Lines: 35

I'm in the process of installing 3/4 x 2 1/4" red oak floor in

an upstairs bedroom (about 160 sq. ft.).  There are some pubs

available from the National Oak Flooring Manufacturers Association

(NOFMA).  The titles are "Hardwood Flooring Installation Manual",

Hardwood Flooring Finishing/Refinishing Manual", and "Wood Floor

Care Guide".  They are available free from NOFMA, (901) 526-5016.

Also of interest, the power nailer described in the NOFMA literature

is hammer driven (no air compressor required).  My distributor has

one of this style which I will be renting.

Number 1 red oak is avaiable in my area for about $1.80 ft.

For finishing, my distributor swears by tung oil (a special formula

--MORE--(52%)for floors).  Nice satin finish and easy to redo.  The floor is

darkened (something like a light walnut color) on the floor in his

house.  He says it will darken further with age.  I'm not sure if

I will go with the Tung oil.  I am also considering a new product

from Varathane called "Diamond".  It is supposed to be twice as tough

as polyurathane and is much easier to work with.  Dries fast and very

low odor.  Available in satin, semi-gloss, and high gloss.  This is

a clear finish that doesn't color the wood.  Can be used over stains

that do not contain waxes or stearates.  I've experimented with

this stuff and it appears to be pretty good, but I haven't used it on

a floor yet.

Also, I've heard that a commonly used finish "Glitza" tends to dry

out the flooring (over time) causing gaps and cracks.  This stuff

is also reported to be very nasty in the fume department.

That's my $.02.

No opinions other than my own.

Brian Benson

End of article 4587 (of 4701)--what next? [npq] Article 4588 (113 more) in rec.woodworking:

From: XID@psuvm.psu.edu

Subject: Re: Making frame&panel doors by hand

Message-ID: <91267.103658XID@psuvm.psu.edu>

Date: 24 Sep 91 14:36:58 GMT

References: <9935@cognos.UUCP>

Organization: Penn State University

Lines: 66

In article <9935@cognos.UUCP>, richardb@cognos.UUCP (Richard Brosseau) says:

>

>I'm thinking of starting more advanced woodworking and seriously lack

>power tools. All the methods I've read about for making frame&panel

>cabinate doors involve a router or tablesaw. I sort of know how to

>make raised panels by hand (with a molding plane maybe?) but making the

>groove in the stiles and rails (I think thats what they're called)

>completly escapes me. Is there some hand tool I'm not aware of? What

>did they do in the 'old' days of frame&panel construction?

>--

They used planes made for the purpose.  There are several types of

planes for making grooves and dadoes.  The oldest and simplest are wood-

bodied with an iron of fixed width.  For a groove of a different width

the woodworker used a plane of a different width.  These look like

--MORE--(27%)molding planes but since they don't make molding they aren't molding

planes in the strict sense.  There are also wood-bodied plow planes with

an adjustable fence to set the distance of the groove from the edge or face

of the board.  Another type was matched pairs of tongue and groove planes.

Then there are the metal analogs of these types made by Stanley and others.

There are sets of fixed-width dado planes (dado planes have a cutter on the

side ahead of the blade that cuts the wood fibers, preventing tear-out).

There are metal tongue-and-groove pairs, or a single plane with two cutters

for doing both tonguing and grooving.  There are metal plow planes with

adjustable fences which take a variety of widths of blades.  Very similar

are beading planes for doing decorative work; these could take straight

blades for grooving and rabbetting as well.  These sorts of planes range

in complexity up to the combination planes which combined these functions

with a fence or fences and many cutter profiles.  The Stanley 45 came with

18 to 23 cutters through its history, with others available optionally.

The Stanley 55 (truly incredible) came with 52 to 55 cutters with 41 others

available from stock and special orders possible, and adjusted every way

you would think of and some you wouldn't.

The info on the 45 and 55 above comes from Alvin Sellens, The Stanley Plane:

A history and Descriptive Inventory, 1975, The Early American Indistries

Association, 216 pp.

--MORE--(62%)If you're interested in the general question of what old tools did what job,

and how to use them now, you might enjoy Michael Dunbar's Restoring, Tuning

and Using Classic Woodworking Tools, 1989, Sterling Publishing Co., New York,

256 pp.  There's also Salaman's book (Dictionary or Encyclopedia of

Woodworking Tools or Tools used in Woodworking or something like that)

now available from Taunton Press.  Older introductory books on

woodworking in your library may also mention these tools.

Other responses to the original post suggested sawing the sides of the

groove or using a router plane.  Sawing the sides of the groove still leaves

the wood in the middle to be removed; the proper plane removes the wood

without the need for sawing first.  A router plane, especially if used

with a fence, could be made to work, although the types with no sole in

front of the blade would need some auxiliary sole.  The router plane

is likely to pivot some in use and leave the edges of the groove

ragged.

Another source of info would be The Woodwright's Shop on PBS - in the

series shown here about 8 months ago (?) Roy did a few programs on

frame and panel construction (always with hand tools).  He (Roy Underhill)

has also written The Woodwright's Shop (I think that's correct),

The Woodwright's Workbook, and

The Woodright's Companion on hand tool woodworking and related stuff, and

--MORE--(95%)I would think frame and panel construction must be in one of them, but I

don't know which one.

Best wishes for your hand woodworking!

Dave Glick   Penn State Energy & Fuels Research Center   XID@psuvm.psu.edu

End of article 4588 (of 4701)--what next? [npq] Article 4589 (112 more) in rec.woodworking:

From: jmr@mebsuta.rtp.semi.harris.com (Mike Reid design eng x636)

Subject: Threading pipe for pipe clamps

Message-ID: <1991Sep24.160813.6604@mlb.semi.harris.com>

Date: 24 Sep 91 16:08:13 GMT

References: <14845.28da1296@ecs.umass.edu> <4646.28dda7f9@dcatla.uucp> <BDS.91Se p24085216@flounder.linus.mitre.org>

Sender: news@mlb.semi.harris.com

Organization: Harris Semiconductor, Research Triangle Park, NC

Lines: 5

How hard is it to thread 3/4" black pipe for pipe clamps?

--

Mike Reid (jmr@rtp.semi.harris.com)

Harris Corporation, Research Triangle Park, NC

(919) 361-1636

End of article 4589 (of 4701)--what next? [npq] Article 4590 (111 more) in rec.woodworking:

From: jeff@jazz.b11.ingr.com (J. Jeff Roberts)

Subject: Re: What to do, how to spend it.

Message-ID: <1991Sep24.164427.22796@b11.ingr.com>

Date: 24 Sep 91 16:44:27 GMT

References: <MILLERMA.91Sep17230315@dragon.cpe.ulowell.edu> <1991Sep19.223041.28 233@uicc.com> <2373@camex.COM>

Sender: usenet@b11.ingr.com (Usenet Network)

Reply-To: jeff@jazz.b11.ingr.com

Distribution: rec.woodworking

Organization: Intergraph Corporation

Lines: 22

In article <2373@camex.COM>, chrisi@lloyd.Camex.COM (Chris Ischay) writes:

|> I'm SO glad to hear someone say this.  I have never been a fan of machine

|> sanding of any kind, and I thought I was just being a crank.  I find every

|> excuse possible to use a scraper...

|> ...

|> Try it.  You'll like it!

OK, you've piqued my curiosity.  I always wondered what those darned things were

when I went through the catalogues!  Do they work well on end grain?

--MORE--(59%)Article 4591 (110 more) in rec.woodworking:

From: tds@hopaw6.ATT.COM (Antonio DeSimone (908-949-4521))

Subject: jigsaws: a summary

Message-ID: <TDS.91Sep24133525@hopaw6.ATT.COM>

Date: 24 Sep 91 18:35:25 GMT

Sender: tds@cbnewsh.cb.att.com (antonio.desimone)

Reply-To: tds@hocus.att.com

Distribution: rec

Organization: AT&T Bell Laboratories

Lines: 100

Last week I posted a query about jigsaws.  As promised, here's a

summary of the responses I've gotten.

>>>>> "John" == John Ahearn <cds2050.ahearn@uunet.uu.net> writes:

John>   I have a Bosch 1581 VS jigsaw.  This is a variable speed,

John> variable orbit D-handle model.  The newer model has a quick change blade

John> mechanism.

> 1) What's the advantage to an orbital jigsaw versus the regular

> old jigsaw?  Is it worth the extra $$?

--MORE--(17%)

John>   I find the variable orbital very useful.  Mine has 4 settings.

John> No orbit for metals, slight orbit for detailed work or hardwood,

John> more orbit for regular work, full blown orbit for hack away work.

> 2) Is variable speed useful, and again, worth the extra $$?

John>   Again, I find it useful.  VS allows me to control the rate of

John> cut.

> 3) Is a cordless a good idea for my only jigsaw?  I think Bosch

> sells one that's not much more expensive than the conventional

> variety, and I think Makita has a couple of models.  I only

> occasionally used the old one, since most things I can do with

> either the circular saw or a handsaw.

John>   Now this I cannot see being worth it.  Cordless models are

John> typically underpowered.  I rarely need to use the jigsaw in a manner

John> in which the cord interferes with the work.

John>   Overall I think you need to temper my comments with how much you

John> use your saw.  It doesn't sound like you use it very often.  I like my

John> model because it is very accurate.  It has a guide for the blade and

--MORE--(40%)John> when mounted in a table is as good as a small band saw.

John>   You also didn't mention the D-handle vs the barrel type.  I

John> prefer the  handle because it gives me better control.

John>   I hope this helps.

John>                                                   :John Ahearn

>>>>> "Robert" == Robert S Parnass <parnass@ihlpy.att.com> writes:

Robert> News-Path: cbnewsd!cbfsb!att!cbnewsh!hopaw16!tds

Robert> I owned a Sears Craftsman variable speed jig saw and was satisfied

Robert> with it.  It was an older model with a cast metal front.

Robert> Then, I replaced the Sears after getting a good price on

Robert> a Milwaukee counterpart.

Robert> I've used both jig saws more than I ever used a circular saw and

Robert> cut all sorts of material with them.  I wouldn't recommend a

Robert> single speed model.  The variable speed is a must

Robert> for cutting non-wood materials, especially steel and plexiglass

Robert> (which can melt from blade friction).

Robert> Look for a jig saw with a small wheel in back of the blade

--MORE--(61%)Robert> to help suport it.  I wouldn't trust battery operated jigsaw

Robert> to have the sustained power I demand from a jig saw.

Robert> Good luck.

Robert> ---

Robert> ======================================================================== ====

Robert> Bob Parnass, AJ9S - AT&T Bell Laboratories - att!ihlpy!parnass (708)979- 5414

>>>>> "Howard" == Howard Cohen <sybase!hsc@Sun.COM> writes:

Howard> Variable speed is worthwhile.  If you ever cut plastic with

Howard> the saw, you will appeciate its low speed capability.  Also,

Howard> when doing fine work, slower speed helps keep mistakes from

Howard> getting away from you.

Howard> Cordless ... thats up to you.  If you really use it seldom,

Howard> then you can probably get away with cordless, but I see little

Howard> advantage.  Cordless drills are a big hit because one need's

Howard> to drill/drive screws just about anywhere.  But cutting wood

Howard> with a jigsaw is done fewer places.  The cord doesn't really

Howard> get in the way, and you never run out of juice in the middle

--MORE--(84%)Howard> of a cut.  If I were buying a new jigsaw, I wouldn't buy a

Howard> cordless.

Howard>         Howard

Howard> ------------------------------------------------------------------------ -------

Howard>   Howard "I wood if I could" Cohen

Howard>   hsc@sybase.com  {pacbell,pyramid,sun,{uunet,ucbvax}!mtxinu}!sybase!hsc

Howard>   Sybase, Inc.    6475 Christie Avenue, Emeryville, CA 94608, 510-596-34 06

So what am I going to do?  Haven't decided yet.  This has been put on

hold by the small matter of having a baby (tomorrow, at 5:45 AM).

I have decided, based on John and Bob's comments, that I probably

didn't use my old jigsaw very much because it was a pretty crummy

model.

--

Tony DeSimone

AT&T Bell Laboratories

--MORE--(99%)Article 4592 (109 more) in rec.woodworking:

From: solomon@math.indiana.edu (bruce solomon)

Subject: Re: What to do, how to spend it.

Message-ID: <1991Sep24.174645.21436@cica.indiana.edu>

Date: 24 Sep 91 17:46:45 GMT

References: <1991Sep24.164427.22796@b11.ingr.com>

Sender: news@cica.indiana.edu (News System)

Reply-To: solomon@ucs.indiana.edu

Distribution: rec.woodworking

Organization: Center for Innovative Computing Applications (CICA), Indiana Unive rsity

Lines: 23

In article <1991Sep24.164427.22796@b11.ingr.com> jeff@jazz.b11.ingr.com (J.

Jeff Roberts) writes:

> ... I thought I was just being a crank.  I find every

> |> excuse possible to use a scraper...

> |> ...

> |> Try it.  You'll like it!

>

> OK, you've piqued my curiosity.  I always wondered what those darned things

> were when I went through the catalogues!  Do they work well on end grain?

> ...

--MORE--(76%)Article 4593 (108 more) in rec.woodworking:

From: phil@brahms.amd.com (Phil Ngai)

Subject: Re: circ -> pseudo table saw : good or bad idea?

Message-ID: <1991Sep24.161542.24170@amd.com>

Date: 24 Sep 91 16:15:42 GMT

References: <14845.28da1296@ecs.umass.edu> <4646.28dda7f9@dcatla.uucp>

Sender: usenet@amd.com (NNTP Posting)

Organization: Advanced Micro Devices, Inc.

Lines: 11

isjrc@dcatla.uucp (John R. Caruso) writes:

>I saw him once putting up a

>casing around a window with a pneumatic nailer. The guy

>watching said "I guess you don't use hammers anymore." Norm replied

>"Naaah, nobody uses a hammer anymore." I don;t know who he is supposed

>to appeal to but it isn't me.

But it's true. I have yet to see a contractor without a p.n.

--

This posting is purely a personal opinion.

End of article 4593 (of 4701)--what next? [npq] Article 4594 (107 more) in rec.woodworking:

From: phil@brahms.amd.com (Phil Ngai)

Subject: Re: Motor power

Message-ID: <1991Sep24.161355.23782@amd.com>

Date: 24 Sep 91 16:13:55 GMT

References: <1991Sep20.160046.274@caliban.uucp>

Sender: usenet@amd.com (NNTP Posting)

Organization: Advanced Micro Devices, Inc.

Lines: 20

ig@caliban.uucp (Iain Bason) writes:

>The recent discussion about the power limits of saws that run off

>110 volts was very interesting.  But it just occurred to me that the

>same limits ought to apply to routers, yet reputable companies

>claim to sell 3 h.p. routers.  Does anyone know how much power they

>actually produce?

Recently, there was a flame about willful numerical illiteracy.

I feel tempted to flame scientific illiteracy.

The laws of physics are fixed. Voltage times amperage = power.  If a so

called reputable company makes a claim that defies the laws of physics,

then the so called reputable companies are liars, plain and simple.

--MORE--(83%)

If you want to know how much power a (single phase) tool produces, just

take the rated amperage and multiply by the operating voltage. That's

all there is to it. Period.

--

This posting is purely a personal opinion.

End of article 4594 (of 4701)--what next? [npq] Article 4595 (106 more) in rec.woodworking:

From: danmc@hpspdla.spd.HP.COM (Dan McGee)

Subject: Norm and the NYW needs some new spark ...

Message-ID: <15230015@hpspdla.spd.HP.COM>

Date: 24 Sep 91 16:08:14 GMT

Organization: HP Stanford Park - Palo Alto, CA

Lines: 16

I really like watching Norm and the NYW although I'm getting kinda tired

of the projects he is choosing.  I, for one, have absolutly no interest

in putting any Shaker furniture in my house and I'm beginning to not

watch the show anymore.

Does anybody else think it might be a nice idea for Norm to try some

different furniture projects from other era's?  What about taking on a

more challenging project that lasts a couple of shows rather than those

that only take one show?

Because I'm no longer interested in the "projects" he is taking on, I

find that all I focus on is his technique ... and believe me, watching

him explain his Biscuit Joiner for the six billionth time is really

beginning to bore me.

--MORE--(95%)NYW needs a kick in the rear ... anybody else agree?

End of article 4595 (of 4701)--what next? [npq] Article 4596 (105 more) in rec.woodworking:

From: dal@boombox.micro.umn.edu (Dave Larsen)

Subject: Re: What to do, how to spend it.

Message-ID: <1991Sep24.191255.2836@paperboy.micro.umn.edu>

Date: 24 Sep 91 19:12:55 GMT

Sender: news@paperboy.micro.umn.edu (Usenet News)

Distribution: rec.woodworking

Organization: University of Minnesota CIS

Lines: 45

Nntp-Posting-Host: dal.micro.umn.edu

>> OK, you've piqued my curiosity.  I always wondered what those darned

things

>> were when I went through the catalogues!  Do they work well on end grain?

>> ...

>Alright. Now *_I'll_* admit it.  My curiosity is highly piqued as well, and I

>haven't looked at enough catalogues to even know what a `scaper' looks like!

>Could someone post a description, price range, etc.?

I'll take a stab at this one. A scraper is nothing more than a piece of

--MORE--(37%)Article 4597 (104 more) in rec.woodworking:

From: mdhotchi@descartes.waterloo.edu (Mike Hotchin)

Subject: Re-wiring motor?

Keywords: Second hand, no info

Message-ID: <1991Sep24.180530.3067@descartes.waterloo.edu>

Date: 24 Sep 91 18:05:30 GMT

Distribution: na

Organization: University of Waterloo

Lines: 23

Hi there, I just bought a second hand jointer/planer, and ran into a problem.

I only have 110 available to me, so I need to re-wire the motor from 220.

Unfortunately, the wiring diagram has completely faded away.  So, is there

anybody out there who might be able to either tell me the right way of wiring

it, or pointers to people/companies who might know?  This is what I know-

Planer - General, mnodel 1180

Motor - "Prestolite-Leland", of Sarnia, Ont (but not any more - I called)

model - 911813-01

type  - CSBKJI

frame E56

--MORE--(80%)230 wiring - Yellow and black to one line

Blue to the other

orange and red connected to each other.

Please mail any responses...

Mike Hotchin

mdhotchi@descartes.waterloo.edu  OR

nsweeny@descartes.waterloo.edu

End of article 4597 (of 4701)--what next? [npq] Article 4598 (103 more) in rec.woodworking:

From: johnb@uwovax.uwo.ca

Subject: <None>

Message-ID: <1991Sep24.155635.9907@uwovax.uwo.ca>

Date: 24 Sep 91 19:56:35 GMT

Lines: 6

Could anyone tell me what the proper dimensions

are for a Checker/Chess board?

EMAIL TO JOHNB@UWOVAX.UWO.CA

End of article 4598 (of 4701)--what next? [npq] Article 4599 (102 more) in rec.woodworking:

From: Nick_Janow@mindlink.bc.ca (Nick Janow)

Subject: Cabinet Scrapers

Message-ID: <7603@mindlink.bc.ca>

Date: 25 Sep 91 07:23:36 GMT

Organization: MIND LINK! - British Columbia, Canada

Lines: 66

solomon@math.indiana.edu (bruce solomon) writes:

> Alright. Now *_I'll_* admit it.  My curiosity is highly piqued as well, and I

> haven't looked at enough catalogues to even know what a `scaper' looks like!

> Could someone post a description, price range, etc.?

Description: a piece of hard, flexible steel.  Mine is 2 1/2" x 6 " x 1/32".

Longer ones are good for wide surfaces; thicker ones for heavy scraping (paint

removal, fast stock removal); thinner ones for fine finishing.  You can get

tough ones (easy to sharpen) and hard ones (longer between sharpenings).

Price range: free (scrap metal) to a few dollars (I think mine was $1.75).  My

first scraper--and one a lot of people use--was a wide hacksaw blade.  It

works, but the commercial ones are easier to sharpen and flex (steel selected

for its properties).  Other tools are a flat mill file and something to burnish

--MORE--(31%)the edge with (hard screwdriver shank, chisel, etc).  I use a piece of smooth

file stock (very hard steel).

Sharpening is fairly easy.  Clamp the blade in a vise and drawfile a flat edge

on it.  For fine work, use a fine sharpening stone after the filing (you want

smooth, sharp 90 degree edges).

To burnish the edge, lay the blade flat on a table edge, then stroke the edge

with your burnishing tool almost parallel to the flat side.  The intent is to

smooth microridges and work-harden the edge.  Next, clamp the blade in the vise

again and stroke the edge with the tool 90 degrees to the edge (a few strokes

is enough).  The intent  is as the first one, and also to start curling the

edge.  At this point, the scraper is suitable for really fine finishing.  For

faster scraping (more agressive cut), burnish the edge with the tool slightly

off 90 degrees (5-10 degrees).  The best way to learn how much pressure to

apply, or how many strokes to apply is to experiment.  Apply too much pressure

and you'll see the result (very agressive cut).

You can often save the filing/stoning step when the scraper dulls.  Just lay

the scraper on the table edge again and burnish it flat again.  Then curl a new

edge on it.  This can be done a few times, though it usually requires more curl

to get the same scraping action.

--MORE--(64%)Once you get the hang of it, you'll learn what shortcuts you can take.  I often

sharpen (for rough work) by holding the file on the table and drawing the

scraper edge along it.  Then I hold the scraper upright to burnish the edge.

It saves time rather than going to the vise.  I scraped an entire living room

oak floor with just hand scrapers (after the removal of old finish with a drum

sander).  It was faster than a belt sander and left a much better finish.

Important note*  One of the most important things to learn about using a

scraper is how to tell when it is dull.  A scraper is dull when it produces

powder instead of shavings.  A sharp scraper produces clean, fine curls of

wood.  A dull scraper will leave you disappointed; don't give up on scraping

until you've tried a properly sharp scraper on a piece of hard wood (oak,

maple).

Above all, experiment.  :)

ps.  Yes, scrapers do work on endgrain.  As with regular grain, it works best

with a sharp scraper on hard wood (soft wood mushes).

Oh, you may find yourself trying handled scrapers (for heavy-duty scraping) and

scraping planes (for fine work).  I made a heavy-duty handled scraper from a

circular-saw blade and some scrap wood.  It's usually termed a floor scraper,

--MORE--(95%)since it was used to finish hardwood floors in pre-power sander days.  Mine

works great on floors (for rough work) an for removing old paint from doors.

--

Nick_Janow@mindlink.bc.ca

End of article 4599 (of 4701)--what next? [npq] Article 4600 (101 more) in rec.woodworking:

From: Nick_Janow@mindlink.bc.ca (Nick Janow)

Subject: Re: What to do, how to spend it.

Message-ID: <7610@mindlink.bc.ca>

Date: 25 Sep 91 07:58:43 GMT

Organization: MIND LINK! - British Columbia, Canada

Lines: 20

dal@boombox.micro.umn.edu (Dave Larsen) writes (edited slightly):

> There is probably going to be a lot of discussion on this, but here is how I

> sharpen mine. I put the [scraper blade] in the vise with the flat edge

> sticking up and use a fine flat file to make the flat side FLAT (flat in the

> long axis and 90 degrees to the blade). Push the file along the long axis of

> the blade at a slight angle to the long axis. This will create a burred edge

> on one side of the scraper. This is your cutting edge.

>

> There are special tools, called burnishers, that will horn the edge to a

> better degree. I never found the need to use one.

Your method is okay for rough work, though you'd get a facter-cutting edge with

a properly curled edge.  For fine finishing (ie. final preparation of hard

maple before oiling), your method would leave noticeable scratches.  For that

--MORE--(90%)work, I file, then stone (1200 grit), then burnish the edge.  I find that it

makes a definite difference.

--

Nick_Janow@mindlink.bc.ca

End of article 4600 (of 4701)--what next? [npq] Article 4601 (100 more) in rec.woodworking:

From: dickey@oasys.dt.navy.mil (Joseph Dickey)

Subject: Woodturning Conference

Message-ID: <11233@oasys.dt.navy.mil>

Date: 24 Sep 91 19:18:13 GMT

Reply-To: dickey@oasys.dt.navy.mil (Joseph Dickey)

Organization: David Taylor Research Center, Bethesda, MD

Lines: 4

The"Chesapeak Woodturners Conference" will be helo in Annapolis, Maryland,

25 - 27 October, 1991. Lots of lectures and demonstrations by nationally

know woodturners. For more information contact: Frank Amigo at 301-923-2513

or me at the above email.

End of article 4601 (of 4701)--what next? [npq] Article 4602 (99 more) in rec.woodworking:

From: bds@maui.linus.mitre.org (Barry D. Smith)

Subject: Re: Motor power

Message-ID: <BDS.91Sep24161939@maui.linus.mitre.org>

Date: 24 Sep 91 21:19:39 GMT

References: <1991Sep20.160046.274@caliban.uucp> <1991Sep24.161355.23782@amd.com>

Sender: news@linus.mitre.org (News Service)

Organization: The MITRE Corporation

Lines: 23

In-Reply-To: phil@brahms.amd.com's message of 24 Sep 91 16:13:55 GMT

Nntp-Posting-Host: maui.mitre.org

>The laws of physics are fixed. Voltage times amperage = power.  If a so

>called reputable company makes a claim that defies the laws of physics,

>then the so called reputable companies are liars, plain and simple.

>

>If you want to know how much power a (single phase) tool produces, just

>take the rated amperage and multiply by the operating voltage. That's

>all there is to it. Period.

Unfortunately that's not all there is to it.  If that were so, then

while many motors are overrated (12A rated at 3HP for an Hitachi

--MORE--(56%)router, etc.), according to this many motors are grossly underrated,

which makes little or no sense at all from a sales standpoint.  For

example, I have a 10.4A/220V Dayton motor rated at 2HP that drives a

planer and I have a 8.6A/110V Gould motor rated at 0.5HP that drives a

well pump.  When I told this story to a physicist friend of mine, he

said it made perfect sense.  No matter how well made a motor is, the

continuous horsepower rating will be significantly LESS than 746

watts/HP due to friction and any other ways of losing power.  He also

said the overrated motors were also correct in that they used a

standard test for rating a motor that gives the peak horsepower a

motor generates before it will break under load.  Working with 746

watts/HP is like doing freshman physics problems with the coefficient

of friction=0.

End of article 4602 (of 4701)--what next? [npq] Article 4603 (98 more) in rec.woodworking:

From: clewis@ferret.ocunix.on.ca (Chris Lewis)

Subject: Re: Plans for Shaker furniture !

Keywords: Shaker, furniture plans

Message-ID: <2609@ecicrl.ocunix.on.ca>

Date: 24 Sep 91 14:40:09 GMT

References: <1991Sep23.153414.380@wg.estec.nl>

Organization: Elegant Communications Inc., Ottawa, Canada

Lines: 48

In article <1991Sep23.153414.380@wg.estec.nl> psh@wg.estec.nl writes:

>My wife and I have, through reading of various American magazines on housing

>and especially furniture, got very interested in SHAKER style furniture, and

>have decided to add building of this type of furniture to the my favourite

>hobby (you guessed it right, woodworking, mainly building eskimo kayaks from

>wood and canvas). We have although not been sucessful in finding any plans

>and/or litterature of/on this type of furniture here in Holland, so if anybody

>could help me with some titles of books with plans and/or descriptions of

>Shaker furniture I will be very grateful. I could then go to the local

>bookstore and ask them to order it for me.

"How to Build Shaker Furniture", Thomas Moser, 1980, Sterling

Publishing Co., Inc.  New York City.  ISBN 0-8069-8392-2.

--MORE--(40%)

"Measured Shop Drawings for American Furniture", Thomas Moser, 1985.  Sterling

Publishing Co., Inc. ISBN 0-8069-5712-3 or 0-8069-6792-7 (paperback)

Distributed in Europe by Cassell PLC, Artillery House, Artillery Row,

London SW1P 1RT, England.

Thomas Moser runs a cabinet making shop in Maine which has been in operation

about 20 years.  Most of the furniture he builds is shaker, tho some of the

designs are modified a bit to take into account changes in lifestyle and

the fact that people have gotten a little taller since then.  The emphasis

is on very simple lines but high levels of craftsmanship.  The second book

branches out a bit from shaker, but the majority of the designs are at least

shaker inspired.  You have to realize that because of the shaker philosophy,

they didn't build certain types of furniture (ie: display cabinets) and others

didn't exist (coffee tables).  Moser has designed shaker style versions.

These aren't step by step construction articles.  HtBSF does have sections

on technique.   MSDfAF does not.  In the designs section, each piece gets at

most a paragraph or two and up to several pages of detailed drawings.

Both books have quite a few designs.  Especially MSDfAF.  Covers every

type of furniture I can think of.

--MORE--(80%)I haven't yet built anything from either because my skills (and time) aren't

quite up to fine furniture yet.  But I have certainly used the ideas and

inspiration in most of the stuff I've built so far.

Both books are recommended.  HtBSF particularly so for the aspiring woodworker

that wants to build real furniture with real wood and real techniques, but

isn't ready for fancy stuff.

--

Chris Lewis; UUCP: ...!{cunews,uunet,latour}!ecicrl!clewis;

DOMAIN: clewis@ferret.ocunix.on.ca; Phone: Canada 613 832-0541

Psroff info: psroff-request@ferret.ocunix.on.ca

Ferret mailing list: ferret-request@ferret.ocunix.on.ca

End of article 4603 (of 4701)--what next? [npq] Article 4604 (97 more) in rec.woodworking:

From: kkuehl@cbnewsf.cb.att.com (kevin.r.kuehl)

Subject: Re: Norm and the NYW needs some new spark ...

Message-ID: <1991Sep25.122008.11193@cbfsb.att.com>

Date: 25 Sep 91 12:20:08 GMT

References: <15230015@hpspdla.spd.HP.COM> <CPETTERB.91Sep24162102@mickey.glacier .sim.es.com>

Sender: news@cbfsb.att.com

Organization: AT&T Bell Laboratories

Lines: 30

In article <CPETTERB.91Sep24162102@mickey.glacier.sim.es.com> cpetterb@glacier.s im.es.com (Cary Petterborg) writes:

>Roy Underhill many times had to complete a project on the next or even the

>next show after the next show.  I would tune in every week to get the

>"rest of the story."  I think that Roy did a much better job of explaining

>technique and the reasons behind it than Norm EVER has.  Roy may show

>techniques that people don't have the hand tools to duplicate, but they were

>good explanations, and could be transferred many times to power tools.

And often he showed how to make the tool he used.  Often they were little

more than sharpened screws in wood blocks.  And with this he would turn

out items (I feel) of much higher quality and warmth than Norm.

--MORE--(70%)Article 4605 (96 more) in rec.woodworking:

From: dsr@mitre.org (Douglas S. Rand)

Subject: Re: norm..blah blah blah

Message-ID: <DSR.91Sep24165511@chickpea.mitre.org>

Date: 24 Sep 91 21:55:11 GMT

References: <14845.28da1296@ecs.umass.edu> <4646.28dda7f9@dcatla.uucp>

<1991Sep23.175405.8834@genie.slhs.udel.edu>

<1991Sep23.131054.1596@arizona.edu>

Sender: news@linus.mitre.org (News Service)

Distribution: world,local

Organization: The MITRE Corporation, Bedford, MA

Lines: 32

In-Reply-To: dolata@lead.uazaic.arizona.edu's message of 23 Sep 91 20:10:53 GMT

Nntp-Posting-Host: chickpea.mitre.org

In article <1991Sep23.131054.1596@arizona.edu> dolata@lead.uazaic.arizona.edu (D olata) writes:

This Saturday our local PBS showed Norm building a Shaker inspired drop

leaf table.

Any other turners out there?   What did you think of Norm's lathe

technique?

--MORE--(49%)Article 4606 (95 more) in rec.woodworking:

From: cpetterb@glacier.sim.es.com (Cary Petterborg)

Subject: Re: Norm and the NYW needs some new spark ...

Message-ID: <CPETTERB.91Sep24162102@mickey.glacier.sim.es.com>

Date: 24 Sep 91 20:21:02 GMT

References: <15230015@hpspdla.spd.HP.COM>

Sender: news@javelin.sim.es.com

Organization: Evans & Sutherland Computer Corp.

Lines: 66

In-Reply-To: danmc@hpspdla.spd.HP.COM's message of 24 Sep 91 16:08:14 GMT

Nntp-Posting-Host: mickey

In article <15230015@hpspdla.spd.HP.COM> danmc@hpspdla.spd.HP.COM (Dan McGee) wr ites:

>   I really like watching Norm and the NYW although I'm getting kinda tired

>   of the projects he is choosing.  I, for one, have absolutly no interest

>   in putting any Shaker furniture in my house and I'm beginning to not

>   watch the show anymore.

I started getting tired through the first season.  He seemed to be doing

projects that would allow him to make money on a book and make each

project last only one show.  There were times when I found he said too

--MORE--(27%)Article 4607 (94 more) in rec.woodworking:

From: steveo@think.com (stephen anthony)

Subject: Norm and the NYW needs some new spark ...

Message-ID: <1991Sep25.134553.10156@Think.COM>

Date: 25 Sep 91 13:45:53 GMT

Sender: news@Think.COM

Distribution: usa

Organization: Thinking Machines Corporation, Cambridge MA, USA

Lines: 50

Originator: steveo@olen.think.com

From: danmc@hpspdla.spd.HP.COM (Dan McGee)

NYW needs a kick in the rear ... anybody else agree?

I've had somewhat the same reaction. The first season was great, the second

ok and the third ho-hum (all IMHO, of course).

I wrote a letter to Morash, etal, with some complaints and ideas. What

really got me was when Norm made the 6 drawer chest (real big one; 6' high

or so). His (Norm's) quote was something like "well after sanding this with

every tool in the shop..."; that was somewhat frustrating. I mean what

--MORE--(33%)Article 4608 (93 more) in rec.woodworking:

From: bryan@intvax2.cs.sandia.gov (Jon R Bryan)

Subject: Re: Threading pipe for pipe clamps

Message-ID: <1991Sep24.212653.16596@intvax2.cs.sandia.gov>

Date: 24 Sep 91 21:26:53 GMT

References: <1991Sep24.160813.6604@mlb.semi.harris.com>

Organization: Sandia National Labs, Org. 1411, Albq., NM

Lines: 9

From article <1991Sep24.160813.6604@mlb.semi.harris.com>, by jmr@mebsuta.rtp.sem i.harris.com (Mike Reid design eng x636):

> How hard is it to thread 3/4" black pipe for pipe clamps?

I buy it from a hardware store and have them thread it for me.  Costs

very little, and I don't have to own a pipe threader.

--

Jon R. Bryan    (505) 844-2015  <=>     bryan@istg.cs.sandia.gov

Sandia National Laboratories, Intelligent Machine Principles Division

End of article 4608 (of 4701)--what next? [npq] Article 4609 (92 more) in rec.woodworking:

From: trocklin@umaxc.weeg.uiowa.edu (Tom Rocklin)

Subject: Re: norm..blah blah blah

Message-ID: <8305@ns-mx.uiowa.edu>

Date: 25 Sep 91 14:22:46 GMT

References: <1991Sep23.224905.20775@news.nd.edu>

Sender: news@ns-mx.uiowa.edu

Lines: 25

From article <1991Sep23.224905.20775@news.nd.edu>, by jstiver@seurat.helios.nd.e du (James Stiver):

[stuff about Norm's tools and their tax deductibility deleted]

>

> Wouldn't it be possible for anyone to take a tax deduction for the

> cost of their tools if they declared their shop a business?  I was

> considering doing this some day.  Just do what's necessary to be

> officially a small business.  If someone comes by to order something,

> show them some appallingly bad piece and they'd leave.

>

> -Jim

The IRS is infinitely clever when it comes to taking your money.

If you don't make make a profit in X out of Y years (where I've forgotten

--MORE--(69%)Article 4610 (91 more) in rec.woodworking:

From: clewis@ferret.ocunix.on.ca (Chris Lewis)

Subject: Re: Need advice: looking for a slow grinder.

Message-ID: <2610@ecicrl.ocunix.on.ca>

Date: 25 Sep 91 12:26:40 GMT

References: <1991Sep22.034249.3950@agate.berkeley.edu> <4890172@hpnmdla.sr.hp.co m>

Organization: Elegant Communications Inc., Ottawa, Canada

Lines: 56

In article <4890172@hpnmdla.sr.hp.com> stanj@hpnmdla.sr.hp.com (Stan Jaffe) writ es:

>I'm in a similar situation: looking for an inexpensive solution to the

>grinding problem.  Here's what I've found:

>The big advantage of using a wheel grinder is that the blade surface is now

>hollow-ground.  This means that when you progress to your flat stones after

>grinding, the surface area in contact with the stone is minimal.  Now you have

>only a small amount of metal to sharpen/hone.

>I have not decided yet, but am considering running a belt and two pulleys

>from a motor (my present high speed grinder, perhaps) to a mandrel.  The

>mandrel will have a 6" "friable" (spelling?) wheel on it.  I'm looking at

--MORE--(33%)>gearing down to a few hundred RPM or less.  The idea is to avoid overheating

>the tool, so quenching it in water while grinding is another approach.

I was talking to the owner of a woodturning shop (that does spindle turning,

table legs, etc. for the furniture industry) about this very topic.

[I had him custom-turn some legs for a child bed I'm building - I have a

lathe, but not the skills yet...].

He said that the store-bought grinders are much too fast, and the stones that

you get with them much too rough for lathe tools, chisels, knives or other fine

woodworking tools.  It's very hard to avoid burning, and the edges aren't very

good.  He suggested buying a double ended arbor and mount it on blocks on a tabl e,

with a 1/4 HP motor mounted below the table, with a slot cut for the belt.

1750 RPM (nominal shaft speed of the most common AC motors) was a good speed,

but 1200 was better.  A forced-air furnace blower motor is ideal.

A double ended arbor is like this (schematically):

=||==XX===HH===XX==||=

XX        XX

|| is the mounting collar and nut for a wheel.  XX is a pillow block (bearings)

--MORE--(65%)plus mount that you bolt the thing to stand-off blocks with, and HH is the

drive pulley.  These are available from most places that have pulleys, belts and

motors.  Lee Valley (and Garret Wade presumably), Busy Bee and Canadian Tire

certainly carry them.  You'd then calculate the motor pulley you'd need to

produce a 1200 RPM speed.  If you had a 1800 RPM motor, you'd want a 1.5:1 ratio

to the arbor pulley.  Or 3:1 with a 3600 RPM motor.

Then, as many others have suggested, use one of the white (friable) wheels.

Lee Valley carries them too.

He also said that until I had one of these built, I could use the 6" delta

grinder I have, just quench the tools in water *often*.

He went on to complain that it isn't possible to buy suitable grinders for

lathe tools, so the professionals have to make their own.  That is, unless

they want to buy one of those water grinder units.  Which Lee Valley also

carries (at a very high price I might add).

--

Chris Lewis; UUCP: ...!{cunews,uunet,latour}!ecicrl!clewis;

DOMAIN: clewis@ferret.ocunix.on.ca; Phone: Canada 613 832-0541

Psroff info: psroff-request@ferret.ocunix.on.ca

--MORE--(98%)Article 4611 (90 more) in rec.woodworking:

From: wentzel@hplabs.hpl.hp.com (Kevin Wentzel)

Subject: Re: Norm and the NYW needs some new spark ...

Message-ID: <7350@hplabsz.hplabs.hpl.hp.com>

Date: 25 Sep 91 14:28:50 GMT

References: <15230015@hpspdla.spd.HP.COM>

Organization: HPLabs, Palo Alto CA

Lines: 11

AGREE!  I'd like to see Norm take on a few simpler, more basic projects that

could serve to teach basic techniques.  (How about doweling instead of the

bisquet joiner.)  I lead 4H (yes, it still exists) woodworking projects and

I'd like to be able to recommend a few NYW shows to the kids.  When I build

stuff for myself it's normally more modern (danish) style furnature.  Norm,

please do a few shows for those of us who don't have unlimited tool budgets!

Anyone know of a way to get this string to Norm?  I assume that his "computer

controlled dovetail carver" doesn't hook up to internet.

Kevin

End of article 4611 (of 4701)--what next? [npq] Skipping unavailable articleArticle 4613 (88 more) in rec.woodworking:

From: jmr@mebsuta.rtp.semi.harris.com (Mike Reid design eng x636)

Subject: Re: Norm and the NYW needs some new spark ...

Message-ID: <1991Sep25.162748.28970@mlb.semi.harris.com>

Date: 25 Sep 91 16:27:48 GMT

References: <15230015@hpspdla.spd.HP.COM> <CPETTERB.91Sep24162102@mickey.glacier .sim.es.com> <1991Sep25.122008.11193@cbfsb.att.com>

Sender: news@mlb.semi.harris.com

Organization: Harris Semiconductor, Research Triangle Park, NC

Lines: 26

Nntp-Posting-Host: mebsuta.rtp.semi.harris.com

In article <1991Sep25.122008.11193@cbfsb.att.com> kkuehl@cbnewsf.cb.att.com (kev in.r.kuehl) writes:

>>>   NYW needs a kick in the rear ... anybody else agree?

>

>No it needs Roy. :-)

Norm Abrams and Roy Underhill have very different approachs to woodworking.

Roy really seems to be more of a historian than a woodworker sometimes.

While Norm often approaches woodworking from the point of view of a man who

has have to make his living from wood (He had a building business in Mass.

before Russ Morash came across him).

--MORE--(60%)Article 4614 (87 more) in rec.woodworking:

From: news@walter.bellcore.com (All the News - Period)

Subject: Re: Scrapers

Message-ID: <1991Sep25.174723.26249@walter.bellcore.com>

Date: 25 Sep 91 17:47:23 GMT

References: <7610@mindlink.bc.ca>

Reply-To: marc@bellcore.com (Marc F Pucci)

Organization: Bellcore, Morristown, NJ

Lines: 9

Nntp-Posting-Host: breeze

I have used scrapers in the past; when properly sharpened and burnished

they do a great job of removing material.  I've never really tried it

on end grain.

Do you really need one of those $15 burnishers for a $3 scraper or

are there other ways to roll the edge?

Marc Pucci

marc@bellcore.com

End of article 4614 (of 4701)--what next? [npq] Article 4615 (86 more) in rec.woodworking:

From: phil@brahms.amd.com (Phil Ngai)

Subject: Re: Motor power

Message-ID: <1991Sep25.163743.21315@amd.com>

Date: 25 Sep 91 16:37:43 GMT

References: <1991Sep20.160046.274@caliban.uucp> <1991Sep24.161355.23782@amd.com>  <BDS.91Sep24161939@maui.linus.mitre.org>

Sender: usenet@amd.com (NNTP Posting)

Organization: Advanced Micro Devices, Inc.

Lines: 37

bds@maui.linus.mitre.org (Barry D. Smith) writes:

>Unfortunately that's not all there is to it.  If that were so, then

>while many motors are overrated (12A rated at 3HP for an Hitachi

>router, etc.), according to this many motors are grossly underrated,

>which makes little or no sense at all from a sales standpoint.  For

I haven't run across this, I admit I don't understand it.  Could it be

a cooling issue? That is, the "under-rated" 2 HP motor can actually

accept more than 2 HP of electricity, but you can't run it at that

continuously without overheating?

I should change my statement to:

--MORE--(48%)

The horsepower produced by a motor is no more than the

product of the rated amperage multiplied by the operating

voltage divided by the watts to horsepower conversion

factor. The continous horsepower may be less if adequate

cooling is not available.

>well pump.  When I told this story to a physicist friend of mine, he

>said it made perfect sense.  No matter how well made a motor is, the

>continuous horsepower rating will be significantly LESS than 746

>watts/HP due to friction and any other ways of losing power.  He also

Yes, but the amount of friction in a motor is probably less than 5%.

Those bearings are pretty good, you know. Look how long it takes

a router or table saw to spin down.

>said the overrated motors were also correct in that they used a

>standard test for rating a motor that gives the peak horsepower a

>motor generates before it will break under load.  Working with 746

That stuff is b.s. It depends on spinning the motor up and then using

rotational inertia to "develop" horsepower. A.K.A. the Sears method.

--MORE--(97%)--

This posting is purely a personal opinion.

End of article 4615 (of 4701)--what next? [npq] Article 4616 (85 more) in rec.woodworking:

From: phil@brahms.amd.com (Phil Ngai)

Subject: Re: Motor power

Message-ID: <1991Sep25.164032.22517@amd.com>

Date: 25 Sep 91 16:40:32 GMT

References: <1991Sep20.160046.274@caliban.uucp> <1991Sep24.161355.23782@amd.com>  <1991Sep24.235756.25827@amc.com>

Sender: usenet@amd.com (NNTP Posting)

Organization: Advanced Micro Devices, Inc.

Lines: 22

stuart@amc.com (Stuart Poulin) writes:

>In article <1991Sep24.161355.23782@amd.com> phil@brahms.amd.com (Phil Ngai) wri tes:

>>

>>If you want to know how much power a (single phase) tool produces, just

>>take the rated amperage and multiply by the operating voltage. That's

>>all there is to it. Period.

>>

>Shouldn't "produces" be changed to "consumes"?

See my amended statement in the previous posting.

--MORE--(66%)

>i.e. A 100 watt light bulb consumes 100 watts, it doesn't produce 100

>     watts of visible light.  Some of the watts come off at heat, some

>     of it gets absorbed by the frosting, ... whatever.

I feel that motors are efficient enough (95%? just a guess) that

it's not worth worrying about. (this also assumes a good power factor,

that perhaps may be an important effect)

--

This posting is purely a personal opinion.

End of article 4616 (of 4701)--what next? [npq] Article 4617 (84 more) in rec.woodworking:

From: krw@cbnewsk.att.com (keith.r.smith)

Subject: Re: Which table saw. Again! (sorry)

Message-ID: <1991Sep25.190830.15949@cbnewsk.att.com>

Date: 25 Sep 91 19:08:30 GMT

References: <4617.28d8a0ca@dcatla.uucp> <1991Sep21.043827.25979@amd.com>

Distribution: usa

Organization: AT&T Bell Laboratories

Lines: 69

In article <1991Sep21.043827.25979@amd.com> phil@brahms.amd.com (Phil Ngai) writ es:

>(warning: Sears flame follows, skip if you don't like heat)

>

>isjrc@dcatla.uucp (John R. Caruso) writes:

>>      Everyone seems to turn up their

>>      nose at Sears nowdays. Would I get the same network of parts

>>      availability in purchasing a Delta?

>

>Recently I tried to buy a single (12 mm) socket from Sears, since most

>of my auto mechanic tools had come from Sears and I had found them to

>be satisfactory. What a useless place Sears has become.  I don't think

>it was like this before, but I must admit I bought most of my tools

--MORE--(27%)Article 4618 (83 more) in rec.woodworking:

From: rhaar@albert.cs.gmr.com (Bob Haar)

Subject: Re: Scrapers

Message-ID: <64837@rphroy.ph.gmr.com>

Date: 25 Sep 91 19:08:07 GMT

Sender: news@rphroy.ph.gmr.com

Reply-To: rhaar@albert.cs.gmr.com

Organization: G.M. Research Labs

Lines: 16

Nntp-Posting-Host: albert.cs.gmr.com

In article 26249@walter.bellcore.com, news@walter.bellcore.com (All the News - P eriod writes:

|> I have used scrapers in the past; when properly sharpened and burnished

|> they do a great job of removing material.  I've never really tried it

|> on end grain.

|>

|> Do you really need one of those $15 burnishers for a $3 scraper or

|> are there other ways to roll the edge?

Try using the shaft of a screw driver (or any other tool with a

round metal shaft) instead of the burnisher.

--MORE--(78%)

Robert Haar, Ph.D.,   InterNet : rhaar@gmr.com

Computer Science Dept., G.M. Research Laboratories

DISCLAIMER: Unless indicated otherwise, everything in this note is

personal opinion, not an official statement of General Motors Corp.

End of article 4618 (of 4701)--what next? [npq] Article 4619 (82 more) in rec.woodworking:

From: seg@cbnews.cb.att.com (scott.e.gordon)

Subject: Please send letters

Keywords: New Yankee Workshop

Message-ID: <1991Sep23.173152.10660@cbnews.cb.att.com>

Date: 23 Sep 91 17:31:52 GMT

Distribution: oh

Organization: AT&T Bell Laboratories

Lines: 30

Believe it or not, WOSU TV has dropped the New Yankee Workshop, after airing

it for the summer only.  In addition, there is a new program called

"Woodworking for All Ages" which they are not going to carry either.

I don't know about you, but I am tired of seeing the same reruns of VIctory

Garden and the Frugal Gourmet 3 or four times a day and when we are lucky,

the NYW 1 time a week.

I spoke with the manager of the Columbus Woodworker's Store who indicated that

the only way to convince WOSU of the errors of their way is to write to the

station.  (A phone call falls on deaf ears.)

If you, too, would like to see either or both programs aired, send your

--MORE--(76%)letters to:

WOSU - TV

Programming Director

1201 Olentangy River Rd

Columbus, OH   43212

Hopefully, us woodworkers can get as much time as the lady with her

sewing machine.

Scott Gordon @ cblpn!seg

-----------------

Disclaimer: I didn't tell my company about my opinion and I didn't listen

to theirs.  I doubt they are the same.

End of article 4619 (of 4701)--what next? [npq] Article 4620 (81 more) in rec.woodworking:

From: mglacy@lamar.ColoState.EDU

Subject: Re: Motor power

Message-ID: <17922@ccncsu.ColoState.EDU>

Date: 25 Sep 91 15:30:29 GMT

References: <1991Sep20.160046.274@caliban.uucp> <1991Sep24.161355.23782@amd.com>  <BDS.91Sep24161939@maui.linus.mitre.org>

Sender: news@ccncsu.ColoState.EDU

Reply-To: mglacy@lamar.ColoState.EDU ()

Organization: Colorado State University, Fort Collins, CO  80523

Lines: 31

In article <BDS.91Sep24161939@maui.linus.mitre.org> bds@maui.linus.mitre.org (Ba rry D. Smith) writes:

>

>Unfortunately that's not all there is to it.  If that were so, then

>while many motors are overrated (12A rated at 3HP for an Hitachi

>router, etc.), according to this many motors are grossly underrated,

>which makes little or no sense at all from a sales standpoint.  For

>example,

..

>well pump.  When I told this story to a physicist friend of mine, he

>said it made perfect sense.  No matter how well made a motor is, the

--MORE--(49%)>continuous horsepower rating will be significantly LESS than 746

>watts/HP due to friction and any other ways of losing power.

If we restrict our focus to relatively similar tools, of relatively similar

quality (say Bosch, Hitachi, Ryobi... routers), I wouldn't think there

would be substantial differences in frictional losses.  I would be

surprised if differences > 10% existed. Yet, even within this restricted

class, I find quite substantial differences when I compare

the wattage to HP ratio based on published figures.

On that rationale, something clearly funny must be going on in HP

rating, as we all know.  Wouldn't the most accurate basis of comparison

be the wattage, so that any comparisons would be accurate within the

limits of what I presume (correctly?) to be minimal differences in

frictional losses?

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Mike Lacy  Dept. of Sociology  Colo. State Univ.   Fort Collins CO 80523

Opinions and truth-claims offered above are mine, not my employer's.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

End of article 4620 (of 4701)--what next? [npq] Article 4621 (80 more) in rec.woodworking:

From: dgbedros@vlsi.waterloo.edu (David Bedrosian)

Subject: Re: Motor power

Message-ID: <1991Sep25.200650.553@vlsi.waterloo.edu>

Date: 25 Sep 91 20:06:50 GMT

References: <1991Sep20.160046.274@caliban.uucp> <1991Sep24.161355.23782@amd.com>

Organization: University of Waterloo

Lines: 44

In article <1991Sep24.161355.23782@amd.com> phil@brahms.amd.com (Phil Ngai) writ es:

>ig@caliban.uucp (Iain Bason) writes:

>>The recent discussion about the power limits of saws that run off

>>110 volts was very interesting.  But it just occurred to me that the

>>same limits ought to apply to routers, yet reputable companies

>>claim to sell 3 h.p. routers.  Does anyone know how much power they

>>actually produce?

>

>Recently, there was a flame about willful numerical illiteracy.

>I feel tempted to flame scientific illiteracy.

>

>The laws of physics are fixed. Voltage times amperage = power.  If a so

--MORE--(46%)>called reputable company makes a claim that defies the laws of physics,

>then the so called reputable companies are liars, plain and simple.

>

>If you want to know how much power a (single phase) tool produces, just

>take the rated amperage and multiply by the operating voltage. That's

>all there is to it. Period.

>

>--

>This posting is purely a personal opinion.

Phil,

You are incorrect about your calculations of power.  In actual fact,

power is the integral of the product of the voltage and current over the

period of the signal (assuming a periodic signal).  Simply multipling

the voltage and the current will give the correct results **provided** the

voltage and current are in phase.  This is seldomly true with motors

running off AC.  For sinusoidal signals, the power can be calculated as

P=V*I*cos(theta)

where V and I are rms quantities and theta is the phase angle between

--MORE--(88%)the voltage and the current.

Next time you should check your "physics" before you make a statement as

you have above that implies you are an authority on this subject!!

David Bedrosian

Dept. of Electrical and Computer Engineering

University of Waterloo.

End of article 4621 (of 4701)--what next? [npq] Article 4622 (79 more) in rec.woodworking:

From: as@cbnewsl.cb.att.com (andrew.schwartz)

Subject: Re: Which table saw. Again! (sorry)

Summary: Sales help at Sears

Message-ID: <1991Sep25.204122.28189@cbnewsl.cb.att.com>

Date: 25 Sep 91 20:41:22 GMT

References: <4617.28d8a0ca@dcatla.uucp> <1991Sep21.043827.25979@amd.com> <1991Se p25.190830.15949@cbnewsk.att.com>

Distribution: usa

Organization: AT&T Bell Laboratories

Lines: 32

In article <1991Sep25.190830.15949@cbnewsk.att.com>, krw@cbnewsk.att.com (keith. r.smith) writes:

>

> First off, I hope that this thread does not turn into a Sears bash-a-thon.

> On the other hand, I have had the same complaints of late with the hardware

> clerks in some of the Sears' in the Philadelphia area.  Maybe I am expecting

> too much when I feel that a clerk in a tool/hardware shop/department should

> actually be able to *help* you.  One gets the impression that many of these

> folx:

>

>       a) don't know diddly about hardware/tools

--MORE--(48%)Article 4623 (78 more) in rec.woodworking:

From: jat@hpsemc.cup.hp.com (Joe Talmadge)

Subject: Taking care of wood

Message-ID: <2370001@hpsemc.cup.hp.com>

Date: 24 Sep 91 17:52:04 GMT

Organization: the Airborne Toxic Event

Lines: 23

Hi, this is a really basic (and embarassing) question.

I train in a stickfighting martial art.  I've recently obtained two

sticks that I'd like to take good care of.  One is made of bahi (I

think it's called "tigerwood" in English), the other of kamagong (I

think it's called "ebony" in English).

Can anyone tell me how I can improve the life of my sticks?  The

dealer told me that I should use Johnson's no-slip floor wax on them,

the theory being that I need to use a wax, and I definitely want it to

be no-slip.  Another person who I trust more told me to use linseed

oil, but wasn't sure if I should use raw or regular oil.

My objective here is to improve the life of the sticks, and not

necessarily to improve the color.  The kamagong (ebony) stick is the

--MORE--(91%)one I'm more worried about.

Thanks,

Joe Talmadge

jat@hpsemc.cup.hp.com

hplabs!hpda!hpsemc!jat

End of article 4623 (of 4701)--what next? [npq] Article 4624 (77 more) in rec.woodworking:

From: hsc@stroid.sybase.com (Howard Cohen)

Subject: Re: Pipe clamp fixtures

Message-ID: <15276@sybase.sybase.com>

Date: 23 Sep 91 17:27:54 GMT

References: <1991Sep16.124119.8402@mlb.semi.harris.com> <10070005@hpfcso.FC.HP.C OM>

Sender: news@Sybase.COM

Organization: Sybase, Inc.

Lines: 22

In article <10070005@hpfcso.FC.HP.COM> shaw@hpfcso.FC.HP.COM (Shaw Moldauer) wri tes:

>What am I missing by not using pipe clamps for everything?

I don't think you're missing anything.  I see no reason to use pipe-clamps

for everything, only those cases where you have a long distance to span for

the glue-up.  For example, for glue-up table tops, furniture carcases, etc.

Clearly, if you can get the job done with a clamp thats easier to use, then

thats the way to go.  I don't think anyone was suggesting using them for

everything, only that given you need one, "what size is appropriate?"

You don't need to over-tighten a pipe clamp to observe the sag.  Gravity will

--MORE--(62%)cause it any time you support the clamp from its ends with no support in the

middle.  The danger is that the clamp pads won't be parallel to each other,

causing your glue-up not to be square with respect to itself.  Sure, you could

try to compensate by adding thin wedges, etc.  But why bother ?  Its better to

just get a clamp that doesn't have that problem (as much).

Howard

-------------------------------------------------------------------------------

Howard "I wood if I could" Cohen

hsc@sybase.com  {pacbell,pyramid,sun,{uunet,ucbvax}!mtxinu}!sybase!hsc

Sybase, Inc.    6475 Christie Avenue, Emeryville, CA 94608, 510-596-3406

End of article 4624 (of 4701)--what next? [npq] Article 4625 (76 more) in rec.woodworking:

From: hsc@stroid.sybase.com (Howard Cohen)

Subject: Re: circ -> pseudo table saw : good or bad idea?

Message-ID: <15284@sybase.sybase.com>

Date: 23 Sep 91 21:26:48 GMT

References: <14845.28da1296@ecs.umass.edu>

Sender: news@Sybase.COM

Organization: Sybase, Inc.

Lines: 117

In article <14845.28da1296@ecs.umass.edu> marsden@ecs.umass.edu writes:

>

>My question:  I've heard of "workstation" tables that will allow you

>to attach hand held tools for added versatility, eg circular saw becomes

>a poor man's table saw, drill -> drill press, and such.  I'm sorry this

>sounds like such a hack, but I'd be interested in hearing if anyone

>has opinions/knowledge of such things, and where to get them.  I'm

>concerned about safety, and cost/quality trade-offs.  I plan on getting

>a router table, but thought I'd find out if a more versatile router

>plus circular saw table might be a reasonable option.

You can make your own tables for such tools, and if you do a good job, you'll

have quite a servicable unit, that can be safe to use when you follow a

--MORE--(17%)Article 4626 (75 more) in rec.woodworking:

From: hsc@stroid.sybase.com (Howard Cohen)

Subject: Re: Query about teeth on Japanese Saws [was: Making frame&panel doors by hand]

Message-ID: <15290@sybase.sybase.com>

Date: 24 Sep 91 01:03:02 GMT

References: <9935@cognos.UUCP> <15244@sybase.sybase.com> <66446@bbn.BBN.COM>

Sender: news@Sybase.COM

Organization: Sybase, Inc.

Lines: 38

In article <66446@bbn.BBN.COM> ncramer@labs-n.bbn.com (Nichael Cramer) writes:

>So my question is: Is there anything inherent in the nature of how the saw

>works that required the teeth to be this way?  I guess what I'm imagining

>is a hybrid of a thin japanese-like blade filed (re-filed?) with sturdier,

>less-complicated western style teeth.  Do such beasts exist, or am I

>missing something simple here?

You raise good points (ha, ha ... sorry).  Just looking at one of these saws

gives you the feeling that resharpening them is not cost effective.  Thats

why they come with replacable blades, which cost about half of what the saw

originally cost ($15 for a blade).  I think you could sharpen a traditional

saw to cut on the pull if you wanted, and I think I've even seen a classical

--MORE--(44%)Article 4627 (74 more) in rec.woodworking:

From: stuart@amc.com (Stuart Poulin)

Subject: Re: Motor power

Message-ID: <1991Sep24.235756.25827@amc.com>

Date: 24 Sep 91 23:57:56 GMT

References: <1991Sep20.160046.274@caliban.uucp> <1991Sep24.161355.23782@amd.com>

Reply-To: stuart@tfatf.amc.com (Stuart Poulin)

Organization: Applied Microsystems, Redmond, WA

Lines: 21

In article <1991Sep24.161355.23782@amd.com> phil@brahms.amd.com (Phil Ngai) writ es:

>

>If you want to know how much power a (single phase) tool produces, just

>take the rated amperage and multiply by the operating voltage. That's

>all there is to it. Period.

>

Shouldn't "produces" be changed to "consumes"?

i.e. A 100 watt light bulb consumes 100 watts, it doesn't produce 100

watts of visible light.  Some of the watts come off at heat, some

--MORE--(75%)     of it gets absorbed by the frosting, ... whatever.

I think the only way to actually find produced horse power is to have

some standard of torque measurement.

>--

>This posting is purely a personal opinion.

And an important opinion it is.  Ban the SHP (Sears Horse Power)

ratings!

End of article 4627 (of 4701)--what next? [npq] Article 4628 (73 more) in rec.woodworking:

From: jstiver@seurat.helios.nd.edu (James Stiver)

Subject: Re: norm..blah blah blah

Message-ID: <1991Sep23.224905.20775@news.nd.edu>

Date: 23 Sep 91 22:49:05 GMT

References: <14845.28da1296@ecs.umass.edu> <4646.28dda7f9@dcatla.uucp> <1991Sep2 3.175405.8834@genie.slhs.udel.edu>

Sender: news@news.nd.edu (USENET News System)

Reply-To: jstiver@seurat.helios.nd.edu (James Stiver)

Organization: Univ. of Notre Dame

Lines: 17

In article <1991Sep23.175405.8834@genie.slhs.udel.edu>, evh@genie.slhs.udel.edu  (Troy Saville) writes:

|>

|> I thought woodworking/carpentry was Norm's job. So its understandable

|> why he has all those tools and 3 walls of pipe clamps. I wouldn't

|> doubt it if he writes all those tools off as a business expense.

|>

|> --

|> - Maryland State Frisbee Championships,Seneca Creek State Park, Sept. 21/22,  91'

|>    evh@genie.slhs.udel.edu    - 92' PDGA points tournament, AWESOME course

--MORE--(75%)Article 4629 (72 more) in rec.woodworking:

From: chrisi@lloyd.Camex.COM (Chris Ischay)

Subject: Re: Cabinet Scrapers

Message-ID: <2392@camex.COM>

Date: 25 Sep 91 21:58:02 GMT

References: <7603@mindlink.bc.ca>

Sender: news@Camex.COM

Organization: Camex Inc., Boston MA

Lines: 32

In article <7603@mindlink.bc.ca> Nick_Janow@mindlink.bc.ca (Nick Janow) writes:

>  The best way to learn how much pressure to

>apply, or how many strokes to apply is to experiment.  Apply too much pressure

>and you'll see the result (very agressive cut).

Thanks to Nick for his very good article on preparing a scraper for use.  I

would add a couple of minor items from my own experience.  First, I use the

back of a chisel as a burnisher, and it is very easy to overdo the

pressure.  I would say you want to apply the pressure you would leverage if

you held the chisel with only thumb and forefinger -- not very hard.

Next, you will learn to feel for the edge, which you can't see without

--MORE--(55%)magnification.  Before you burnish, feel the edge and it's smooth.  After a

couple passes you'll feel the hook, but it's very minimal.  I would say as

soon as you feel anything at all, try it out.

And remember if you're making dust, it's not right.  Try springing the

blade with your thumbs a little (if you're pushing).  Or try a different

angle of contact with the work.  I find if I burnish with a 5 degree angle

I get the best hook.

Somehow I got lucky and I found it easy.  I've heard other people say they

think scrapers are for the birds.  But I guarantee once you get it going

you'll find every possible excuse to use a scraper.

And Tage Frid's articles in his first book and/or in FWW on Hand Tools are

excellent.  But they don't add much to Nick's excellent article, except for

the pictures.

Chris Ischay

chrisi@bos.camex.com

End of article 4629 (of 4701)--what next? [npq] Article 4630 (71 more) in rec.woodworking:

From: clewis@ferret.ocunix.on.ca (Chris Lewis)

Subject: Re: Motor power

Message-ID: <2611@ecicrl.ocunix.on.ca>

Date: 26 Sep 91 01:21:47 GMT

References: <1991Sep20.160046.274@caliban.uucp>

Organization: Elegant Communications Inc., Ottawa, Canada

Lines: 26

In article <1991Sep20.160046.274@caliban.uucp> ig@caliban.uucp (Iain Bason) writ es:

>The recent discussion about the power limits of saws that run off

>110 volts was very interesting.  But it just occurred to me that the

>same limits ought to apply to routers, yet reputable companies

>claim to sell 3 h.p. routers.  Does anyone know how much power they

>actually produce?

1 HP theoretically ~= 750 watts.  15A @ 120V = 1800 watts.  Theoretically,

that's 2.4HP.  However, motors tend to be 60-80% efficient, so the most

powerful motor you could hang off a normal circuit is about 1.5 - 1.9 HP.

No device attached to a 15A 120V circuit can produce more than 1.5 or

so HP.  So that 3HP router is probably really 1.5HP.  The 3HP is usually

--MORE--(64%)explained as "peak" (could actually be calculated from stall current

which may be considerably higher than the breaker trip point).

On the other hand, some 3HP routers have 220V cords on them, so they

could really develop that much.  You'd have to find out the amperage.

A 1HP motor typically draws about 10-12A at 120V.  1.5HP would be 13-15A.

Cut in half for 220V.

--

Chris Lewis; UUCP: ...!{cunews,uunet,latour}!ecicrl!clewis;

DOMAIN: clewis@ferret.ocunix.on.ca; Phone: Canada 613 832-0541

Psroff info: psroff-request@ferret.ocunix.on.ca

Ferret mailing list: ferret-request@ferret.ocunix.on.ca

End of article 4630 (of 4701)--what next? [npq] Article 4631 (70 more) in rec.woodworking:

From: armstrng@cs.dal.ca (Stan Armstrong)

Subject: Re: Motor power

Message-ID: <1991Sep25.213113.4899@cs.dal.ca>

Date: 25 Sep 91 21:31:13 GMT

References: <1991Sep24.161355.23782@amd.com> <BDS.91Sep24161939@maui.linus.mitre .org> <17922@ccncsu.ColoState.EDU>

Organization: Math, Stats & CS, Dalhousie University, Halifax, NS, Canada

Lines: 20

I can't resist any longer. I am really curious. Surely somebody knows.

I followed a similar string to this several months ago without finding

an answer. My question is this. What is "developed horsepower"?

Surely it has some real meaning. It seems to have something to do with

inertia. If I have a massive router rotor and a small router roter both

accepting the same power input and I suddenly run into denser wood or

if I suddenly push harder into the cut, it seems that the router with

the more massive rotor might respond better. It would slow down less

while giving up the same amount of rotational energy. That might be more

than advertising puffery. It could make a real difference. The question

is, does it? Does anyone know? Even better, can anyone do the math that

will illustrate the difference?

--MORE--(82%)

Advertisers are noted for half truths. Outright lies are easier to control.

What is the half of the claim to make a 3 hp router that is true?

--

Stan Armstrong. Religious Studies Dept, Saint Mary's University, Halifax, N.S.

att!clyde!watmath!water!dalcs!armstrng | armstrong@husky1.stmarys.ca

End of article 4631 (of 4701)--what next? [npq] Article 4632 (69 more) in rec.woodworking:

From: brucem@mentorg.com (Bruce McAlary)

Subject: Sears Bashing (Was Re: Which table saw. Again! (sorry))

Message-ID: <1991Sep25.205401.5658@APD.MENTORG.COM>

Date: 25 Sep 91 20:54:01 GMT

References: <4617.28d8a0ca@dcatla.uucp> <1991Sep21.043827.25979@amd.com> <1991Se p25.190830.15949@cbnewsk.att.com>

Sender: news@APD.MENTORG.COM (uucp news)

Distribution: usa

Organization: engr

Lines: 46

In article <1991Sep25.190830.15949@cbnewsk.att.com> krw@cbnewsk.att.com (keith.r .smith) writes:

>In article <1991Sep21.043827.25979@amd.com> phil@brahms.amd.com (Phil Ngai) wri tes:

>>(warning: Sears flame follows, skip if you don't like heat)

>>

>>isjrc@dcatla.uucp (John R. Caruso) writes:

>>>     Everyone seems to turn up their

>>>     nose at Sears nowdays. Would I get the same network of parts

>>>     availability in purchasing a Delta?

>>

--MORE--(37%)Article 4633 (68 more) in rec.woodworking:

From: Nick_Janow@mindlink.bc.ca (Nick Janow)

Subject: Re: Query about teeth on Japanese Saws

Message-ID: <7633@mindlink.bc.ca>

Date: 26 Sep 91 07:20:57 GMT

Organization: MIND LINK! - British Columbia, Canada

Lines: 25

hsc@stroid.sybase.com (Howard Cohen) writes:

> I think you'll be amazed at how efficient and accurate they [Japanese saws]

> are.

Efficient, yes.  Smooth cut, also yes.  Accurate, not that I've found.  Maybe I

just got a bad producer, but I tried seven different blades [fine Dozuki] at

one store and couldn't get a straight cut.  Any cut deeper than a cm or so

would bow one way or the other.  I tried different holding techniques (even

clamping the blade in a vise and moving the wood over it), different pressures

and rates of cutting, different woods--nothing worked.  Each blade would

produce a bowed cut, each blade different, but consistent for each blade

(meaning the blade was the factor, not me).  I've heard other people say

they've had similar problems.

--MORE--(71%)The Dozuki saw is supposed to be for cutting fine tenons.  I couldn't use it

for that, since the tenons shoulders would not be straight.

Those blades were from Lee Valley Tools.  I was really glad they have such a

great return policy (3 months, no arguments).  I liked the other qualities of

the saw (smooth, fast cut), but I'll have to find somewhere that supplies

blades that can produce an accurate cut.

--

Nick_Janow@mindlink.bc.ca

End of article 4633 (of 4701)--what next? [npq] Article 4634 (67 more) in rec.woodworking:

From: hamlow@sptekwv4.tek.com (Paul Hamlow)

Subject: Re: norm..blah blah blah

Message-ID: <11688@orca.wv.tek.com>

Date: 26 Sep 91 00:33:58 GMT

References: <14845.28da1296@ecs.umass.edu> <4646.28dda7f9@dcatla.uucp> <1991Sep2 3.175405.8834@genie.slhs.udel.edu> <1991Sep23.224905.20775@news.nd.edu>

Sender: nobody@orca.wv.tek.com

Reply-To: hamlow@orca.tek.com (Paul Hamlow)

Organization: /usr/ens/etc/organization

Lines: 24

In article <1991Sep23.224905.20775@news.nd.edu> jstiver@seurat.helios.nd.edu

(James Stiver) writes:

>

>Wouldn't it be possible for anyone to take a tax deduction for the

>cost of their tools if they declared their shop a business?  I was

>considering doing this some day.  Just do what's necessary to be

>officially a small business.  If someone comes by to order something,

>show them some appallingly bad piece and they'd leave.

>

>-Jim

--MORE--(59%)Article 4635 (66 more) in rec.woodworking:

From: smb@ulysses.att.com (Steven Bellovin)

Subject: Re: Norm and the NYW needs some new spark ...

Message-ID: <15539@ulysses.att.com>

Date: 25 Sep 91 22:31:08 GMT

References: <15230015@hpspdla.spd.HP.COM> <1991Sep25.122008.11193@cbfsb.att.com>

Sender: netnews@ulysses.att.com

Lines: 15

In article <1991Sep25.122008.11193@cbfsb.att.com>, kkuehl@cbnewsf.cb.att.com (ke vin.r.kuehl) writes:

> In article <CPETTERB.91Sep24162102@mickey.glacier.sim.es.com> cpetterb@glacier .sim.es.com (Cary Petterborg) writes:

> >Norm seems to be "power tool crazy", probably in part because he is given

> >so many by the manufacturers that he has to feature at least 3 new tools

> >each episode.

>

> Norm seems to be a bit more commercial than Roy.  Good or bad?  Norm is

> on every week, yet I can't remember the last time I saw Roy. :-(

You can't criticize Norm for being ``power tool crazy'' when that's the

whole point of the show, in more or less so many words.  That is, he

--MORE--(82%)Article 4636 (65 more) in rec.woodworking:

From: david@talgras.UUCP (David Hoopes)

Newsgroups: rec.woodworking,trial.rec.metalworking

Subject: Re: Wood Carving Tools

Message-ID: <135@talgras.UUCP>

Date: 26 Sep 91 14:19:22 GMT

References: <1590001@hpfcdq.fc.hp.com> <1991Sep23.132839.1597@arizona.edu>

Reply-To: david@talgras.UUCP (David Hoopes)

Followup-To: trial.rec.metalworking

Distribution: usa

Organization: Tallgrass Technologies Inc.

Lines: 82

In article <1991Sep23.132839.1597@arizona.edu> you write:

>

>Carving tools are soooo expensive I didn't want to take up the craft, for

>fear it would be a never ending sink for money.   Then I met a woodworker

>who showed me how easy and cheap it is to make a lot of your own tools.

>

>I had a forge made out of a 50 gallon drum bottom and some tubing welded

>on.  I lined it with clay,  and put it near my shop.  I use the outlet

>from my shop vac for the air source.   I got a 600 pound

>block of scrap iron for an anvil, and bought a little anvil for fine shaping.

--MORE--(24%)>A few hammers, pliers and tongs from the local swap meet, and for less than

>$75, viola,  I'm set!

>

I had to laugh when I read this.  My hobby is blacksmithing and carriage

restoration with the main interest being the blacksmithing.  I got started

in the smithing as a way to recreate missing hardware for the wagons and

carriages.  Now I prefer the smithing to the woodworking.  I really enjoy

making tools.  Right now I am working on making an old fashion molding

plane.

>I use old files.  I anneal them,  grind or hammer out the teeth,  and then

>shape them with a combination of forging and grinding techniques.  The forging

>defines the sweep,  the grinding refines it and is how I make fishtail

>gouges, etc...    I can make all sorts of gouges, chisels, bent gouges and

>larger vieners with no trouble.

>

Other swources of good metal are railroad spikes, car springs and axles.  I

have been working with railroad spikes lately and have had good luck.  What

do you use for grinding?  I use a small table top belt sander made by drimel.

It works ok but there are some larger (and more expensive) belt sanders that

do a better job.

--MORE--(49%)

>I can't make small vieners, V tools, and macaroni tools. My spoon gouges

>aren't very good, yet.   I need a swage block for that.

>

You might try open end of differant size sections of pipe for a swage block.

If you file the sharp edge off of the inside they make fair swages.  I have

also used the end grain of a heavey piece of wood.  As you work the hot

metal it will burn a depression into the wood.  I have had good sucess

with that making spoons.

For light work a piece of angle iron with a base on it to hold the open

end up might work for v shaped tools.

>There are several books out there that actually tell you how to make your

>own carving tools!   I am at work,  so I don't know the names or authors.

>But if you are dextrous enough to learn carving,  you can easily learn

>how to make simple tools.

>

There are some smithing discussions in trial.rec.metalworking that might

be of interest to you.

--MORE--(70%)

>This is the only way I can afford a large enough set of tools.   And, you

>can make tools for the specific job your are doing.  Just

>a little while ago I decided I needed a 1/8" wide, spear pointed bent chisel

>for finishing some details in the folds of a leek I was carving...   It

>took me about 3 hours,  but I now have a set of three.  Total cost,  about

>$2 - 3 for the coal and about $1 for the dead files,  and three small

>bits of wood for the turned handles.

--

---------------------------------------------------------------------

David Hoopes                              Tallgrass Technologies Inc.

uunet!talgras!david                       11100 W 82nd St.

Voice: (913) 492-6002 x323                Lenexa, Ks  66214

---

---------------------------------------------------------------------

David Hoopes                              Tallgrass Technologies Inc.

uunet!talgras!david                       11100 W 82nd St.

Voice: (913) 492-6002 x323                Lenexa, Ks  66214

--

---------------------------------------------------------------------

--MORE--(95%)David Hoopes                              Tallgrass Technologies Inc.

uunet!talgras!david                       11100 W 82nd St.

Voice: (913) 492-6002 x323                Lenexa, Ks  66214

End of article 4636 (of 4701)--what next? [npq] Article 4637 (64 more) in rec.woodworking:

From: brankin@cbnewsk.att.com (william.t.rankin)

Subject: Re: Using nails and screws

Keywords: screws, nails

Message-ID: <1991Sep26.204650.24565@cbnewsk.att.com>

Date: 26 Sep 91 20:46:50 GMT

References: <1991Sep26.175904.6018@cbnewsh.cb.att.com> <1991Sep26.183942.9056@sc haefer.math.wisc.edu>

Distribution: usa

Organization: AT&T Bell Laboratories

Lines: 40

In article <1991Sep26.183942.9056@schaefer.math.wisc.edu> wilson@schaefer.UUCP ( Bob Wilson) writes:

>I too cringe at all the nails and to a lesser amount at the screws. I

>think many of the nails, though, are gluing clamps. Note how often he

>uses a nail either instead of clamps or else to let him take the

>clamps off right away.

Yup, in fact I believe that Norm has mentioned his use of nails

and screws as such a number of times during his shows.

>I am sure the glued joint is MUCH stonger than

--MORE--(42%)>the nails, so the nails become irrelvant as the glue dries. I might

>not do it this way, but (a) it is in keeping with a background of

>doing it commercially, where you have to get more done per hour, and

>(b) for that matter it is potentially the way to go for all the people

>who have been complaining that they can't have so many clamps!

When I took a cabinet making class at the local community collage, our

instructor made cabinets professionally, at a company of about thirty

or so cabinet makers.

We were putting together a good number of flat-panel doors, with

floating panels.  The process went like: (1) insert panel, glue rails

to stiles; (2) clamp the joints with a couple pipe clamps; (3) square

the door up; (4) shoot 4-5 1/4" brads into each joint; and (5) unclamp,

set the door aside to dry, and set up for the next one.

The point being: even in a commercial house, nobody has enough pipe

clamps to spare two for every door you're going to make that afternoon.

I was impressed by her (our instructor was also the only female

cabinetmaker at her company) ability to throw together doors so

quickly.  When we asked her about it, she said that when she first

started working at the company, all she did for the longest time

--MORE--(95%)was make doors (low on the totem pole and such.)

>Bob W.

>wilson@math.wisc.edu

bill rankin

wtr@bromo.att.com

End of article 4637 (of 4701)--what next? [npq] Article 4638 (63 more) in rec.woodworking:

From: gringort@wsl.dec.com (Joel Gringorten)

Subject: Re: Query about teeth on Japanese Saws

Message-ID: <1991Sep26.133347@wsl.dec.com>

Date: 26 Sep 91 20:33:47 GMT

References: <7633@mindlink.bc.ca> <1991Sep26.114138.19364@cbfsb.att.com>

Sender: news@PA.dec.com (News)

Reply-To: gringort@wsl.dec.com (Joel Gringorten)

Organization: DEC Western Software Lab

Lines: 24

In article <1991Sep26.114138.19364@cbfsb.att.com>, kkuehl@cbnewsf.cb.att.com (Ke vin Kuehl) writes:

|> In article <7633@mindlink.bc.ca> Nick_Janow@mindlink.bc.ca (Nick Janow) write s:

|> >Maybe I

|> >just got a bad producer, but I tried seven different blades [fine Dozuki] at

|> >one store and couldn't get a straight cut.

[...]

|> I might be the woodworker.  I once read that a skilled ``Western woodworker''

|> will have great difficulty using Japanese-style woodworking tools because

--MORE--(55%)|> one has developed certain muscles and techniques that are completely wrong

|> for their style of tools.

I'm pretty sure there was a discussion here about this many months

ago.  Then again, maybe it was on an internal corporate conference.

Anyway, the bottom line with Dozukis is that you have to set them up before

they'll be of much use.  The teeth come set totally wrong and have to be

completely reset for use.  Kinda like chisels and planes, neither of which can

be used out of the box.  Infact it's detrimental to them;  I know this from

experience!  I remember seeing a pretty extensive procedure for tuning a

dozuki somewhere...  Supposedly after this, they cut straight as an arrow.

Dunno, because I've stayed away from them because they seemed like too much

trouble to me.

-joel

End of article 4638 (of 4701)--what next? [npq] Article 4639 (62 more) in rec.woodworking:

From: danmc@hpspdla.spd.HP.COM (Dan McGee)

Subject: Re: Norm and the NYW needs some new spark ...

Message-ID: <15230016@hpspdla.spd.HP.COM>

Date: 26 Sep 91 16:10:01 GMT

References: <1991Sep25.134553.10156@Think.COM>

Organization: HP Stanford Park - Palo Alto, CA

Lines: 15

As the original poster of the string I was really trying to focus on the

types of projects that Norm does ... NOT on the tools he uses (that's

been discussed to death and I was hoping to avoid the same old stuff).

All I'm suggesting is that they try some projects from other era's other

than Shaker.  I don't care if they're more complex or easier I just

think the show could do with some variety.

I don't consider what I'm doing is "Norm Bashing."  I like Norm and

think he's pretty down-to-earth on his show.  I just thing some

constructive critisism/discussion about the content of the show (other

than on the tools he uses) would be interesting.

Dan

--MORE--(97%)Article 4640 (61 more) in rec.woodworking:

From: dsr@mitre.org (Douglas S. Rand)

Subject: Re: Scrapers

Message-ID: <DSR.91Sep25180252@chickpea.mitre.org>

Date: 25 Sep 91 23:02:52 GMT

References: <7610@mindlink.bc.ca> <1991Sep25.174723.26249@walter.bellcore.com>

Sender: news@linus.mitre.org (News Service)

Organization: The MITRE Corporation, Bedford, MA

Lines: 20

In-Reply-To: news@walter.bellcore.com's message of 25 Sep 91 17:47:23 GMT

Nntp-Posting-Host: chickpea.mitre.org

In article <1991Sep25.174723.26249@walter.bellcore.com> news@walter.bellcore.com  (All the News - Period) writes:

I have used scrapers in the past; when properly sharpened and burnished

they do a great job of removing material.  I've never really tried it

on end grain.

Do you really need one of those $15 burnishers for a $3 scraper or

are there other ways to roll the edge?

--MORE--(69%)You can make your own burnisher out of an old ratty file.  Just smooth

and polish an edge of the tang.  You could also harden a smoothed piece

of high carbon steel.  The important thing is for the burnisher to

be very hard (a high Rockwell hardness).

--

Douglas S. Rand

Internet:   <dsrand@mitre.org>

Snail:      MITRE, Burlington Road, Bedford, MA

Disclaimer: MITRE might agree with me - then again...

Amateur Radio: KC1KJ

End of article 4640 (of 4701)--what next? [npq] Article 4641 (60 more) in rec.woodworking:

From: dsr@mitre.org (Douglas S. Rand)

Subject: Re: Taking care of wood

Message-ID: <DSR.91Sep25181645@chickpea.mitre.org>

Date: 25 Sep 91 23:16:45 GMT

References: <2370001@hpsemc.cup.hp.com>

Sender: news@linus.mitre.org (News Service)

Organization: The MITRE Corporation, Bedford, MA

Lines: 44

In-Reply-To: jat@hpsemc.cup.hp.com's message of 24 Sep 91 17:52:04 GMT

Nntp-Posting-Host: chickpea.mitre.org

In article <2370001@hpsemc.cup.hp.com> jat@hpsemc.cup.hp.com (Joe Talmadge) writ es:

Hi, this is a really basic (and embarassing) question.

I train in a stickfighting martial art.  I've recently obtained two

sticks that I'd like to take good care of.  One is made of bahi (I

think it's called "tigerwood" in English), the other of kamagong (I

think it's called "ebony" in English).

Can anyone tell me how I can improve the life of my sticks?  The

--MORE--(38%)   dealer told me that I should use Johnson's no-slip floor wax on them,

the theory being that I need to use a wax, and I definitely want it to

be no-slip.  Another person who I trust more told me to use linseed

oil, but wasn't sure if I should use raw or regular oil.

My objective here is to improve the life of the sticks, and not

necessarily to improve the color.  The kamagong (ebony) stick is the

one I'm more worried about.

Ebony is a very hard wood and will pick up oils from your hands over

time.  It should last pretty well with no treatment.  If you must,

then I would use butchers wax (which you say don't want to use) or

processed linseed oil.  Raw linseed oil has no dryers and will just

make the stick sticky.  Other oil products of the same nature and

better film resistance are Watco danish oil and tung oil.  I'd stay away

from most floor waxes,  many of them are polymere coatings and not

waxes these days.

All that said,  I've no idea how healthy it is to constantly handle an

object coated with any of the above.  Did the dealer say that the sticks

would deteriorate if you don't coat them?  Another more impervious coating

would be polyurethane varnish,  but that would be fairly slippery.

--MORE--(84%)I hope that your style doesn't use these sticks in contact sparring.  Ebony

is a brittle wood,  fine for showing off forms with but I think it

would probably crack if it contacted another stick, wall, or other hard

surface.

--

Douglas S. Rand

Internet:   <dsrand@mitre.org>

Snail:      MITRE, Burlington Road, Bedford, MA

Disclaimer: MITRE might agree with me - then again...

Amateur Radio: KC1KJ

End of article 4641 (of 4701)--what next? [npq] Article 4642 (59 more) in rec.woodworking:

From: steve@panews (Steve DeJarnett)

Subject: Re: Norm and sparks

Message-ID: <1991Sep26.205412.19573@ibmpa.awdpa.ibm.com>

Date: 26 Sep 91 20:54:12 GMT

References: <1991Sep26.163055.13529@Think.COM>

Sender: news@ibmpa.awdpa.ibm.com (News Master)

Distribution: usa

Organization: IBM PSP Development, Palo Alto

Lines: 26

In article <1991Sep26.163055.13529@Think.COM> steveo@think.com (stephen anthony)  writes:

>BTW: I'm convinced that the prototype is the actual piece that gets

>finished at the end of the show. I couldn't see making a prototype out of

>cherry! Any corroboration on this?

I saw Norm at a DIY show last year, and he commented on several aspects

of NYW.  First, he said he'll definitely do one more season (this one), and if

interest holds up, he might do one more after that.  That final season would

be dedicated to going to France, finding some furniture there that would be

somewhat challenging to build, and then coming back to the US and building them.

--MORE--(56%)Article 4643 (58 more) in rec.woodworking:

From: erickson@atlas.cs.nps.navy.mil (David Erickson)

Subject: Re: norm..blah blah blah

Message-ID: <2917@atlas.cs.nps.navy.mil>

Date: 26 Sep 91 21:33:42 GMT

References: <14845.28da1296@ecs.umass.edu> <4646.28dda7f9@dcatla.uucp> <1991Sep2 3.175405.8834@genie.slhs.udel.edu> <1991Sep23.224905.20775@news.nd.edu> <11688@o rca.wv.tek.com>

Reply-To: erickson@taurus.cs.nps.navy.mil (David Erickson)

Organization: Naval Postgraduate School, Monterey CA

Lines: 15

In article <1991Sep23.224905.20775@news.nd.edu> jstiver@seurat.helios.nd.edu

(James Stiver) writes:

>Wouldn't it be possible for anyone to take a tax deduction for the

>cost of their tools if they declared their shop a business?

>-Jim

Another point that is worth mentioning: you may be able to add all or

part of the cost of your tools to the basis of your house when you sell

it, thereby reducing your capital gains tax.  The percentage depends on

--MORE--(91%)Article 4644 (57 more) in rec.woodworking:

From: dave@hpdmd48.boi.hp.com (Dave Fujii)

Subject: Re: Which table saw. Again! (sorry)

Message-ID: <3050040@hpdmd48.boi.hp.com>

Date: 25 Sep 91 16:28:15 GMT

References: <4617.28d8a0ca@dcatla.uucp>

Organization: Hewlett Packard - Boise, ID

Lines: 29

I purchased a Sears 10" 3HP peak (marketing fluff 1.5HP) table saw

#29876 -- cast iron extensions, cheap fence, plastic arbor adjustment

wheels.  I paid in the mid $300 range on sale.

The saw went together very nicely, with the only difficulty being

coplanarity between the extensions and the table top; the extension

bowed about 1/8".  Pulling the extension even with the table top

with a C-clamp before tightening the attachment bolts solved that

problem.

I spent the extra bucks and replaced the el cheapo blade that came

with the saw with a nice carbide tipped combination blade, and

love the saw -- with a little extra set-up time for each different

cut, the performance of the saw is smooth and precise.

--MORE--(61%)Article 4645 (56 more) in rec.woodworking:

From: brucem@mentorg.com (Bruce McAlary)

Subject: Re: Motor power

Message-ID: <1991Sep26.160446.13508@APD.MENTORG.COM>

Date: 26 Sep 91 16:04:46 GMT

References: <BDS.91Sep24161939@maui.linus.mitre.org> <17922@ccncsu.ColoState.EDU > <1991Sep25.213113.4899@cs.dal.ca>

Sender: news@APD.MENTORG.COM (uucp news)

Organization: engr

Lines: 60

In article <1991Sep25.213113.4899@cs.dal.ca> armstrng@cs.dal.ca (Stan Armstrong)  writes:

>I can't resist any longer. I am really curious. Surely somebody knows.

>

>I followed a similar string to this several months ago without finding

>an answer. My question is this. What is "developed horsepower"?

>Surely it has some real meaning. It seems to have something to do with

>inertia. If I have a massive router rotor and a small router roter both

>accepting the same power input and I suddenly run into denser wood or

>if I suddenly push harder into the cut, it seems that the router with

>the more massive rotor might respond better. It would slow down less

>while giving up the same amount of rotational energy. That might be more

--MORE--(32%)>than advertising puffery. It could make a real difference. The question

>is, does it? Does anyone know? Even better, can anyone do the math that

>will illustrate the difference?

>

>Advertisers are noted for half truths. Outright lies are easier to control.

>What is the half of the claim to make a 3 hp router that is true?

>

>--

>Stan Armstrong. Religious Studies Dept, Saint Mary's University, Halifax, N.S.

>att!clyde!watmath!water!dalcs!armstrng | armstrong@husky1.stmarys.ca

Router motors are brush type motors and have a different setup than the

induction motors found on table saws and other stationary tools. One difference

is the rpm of these motors, my tablesaw is 16 amp, 1.5 hp @ 3450, and my router  is

8.0 amp, 1.5 hp at 22000 rpm. The other router is 5.0 amps, 1 hp at 23000 rpm.

The 1.5 hp router is a sears but seems to be in line for the advertised hp for

the rpm and amps it draws, at least compared to my 12 year old B&D 1 hp router

(Back about the time they quit making real power tools).

I'm sure the rpm has something to do with the hp rating, but I believe the

construction of the motor has something to do with it as well.

--MORE--(62%)

If you get a chance to look at the motors on the new Japanese equipment,

Hitatchi for example has a 3" bandsaw with a 3 hp motor that looks similar

in size to a router motor. I don't know the spec's, but suppose it is a

110vac, 10 amp or so motor running at very high rpms, and geared down, thus

producing adequate torque to run a load of that size.

Sears ads touting "Develops 3 hp" is bogus, I know it, you know it, but there

are lots of people out there that are fooled by such marketing. When you

get down to the rpm/amps/volts and motor type, sears 3hp is really just

a 1.5 hp.

Price/Performance is what you look for, and if you choose to spend $59 on a

sears 1.5 hp router that has cheap plastic adjustment rings and a real

flimsey base, if it fits your percieved needs, it is fine. If you need a

precision depth adjustment, a solid base and motor holding mechanism, and

a 3 or 4 finger collet rather than a split collet, you will spend $180-$300

on a good Porter-Cable or Makita or Milwalkie.

I would like to hear for a professional who designs and/or rebuilds electrical

motors who can impart some real information on this subject. Maybe a trip

to the library is in order...

--MORE--(94%)Happy woodworking! Bruce (Keeping all ten digits) McAlary...

--

brucem@pdx.MENTOR.COM, Mentor Graphics Corporation!Wilsonville!Oregon!USA!Earth!

Solar System!Milky Way!Known Universe!Unknown Universe...

End of article 4645 (of 4701)--what next? [npq] Article 4646 (55 more) in rec.woodworking:

From: kkuehl@cbnewsf.cb.att.com (Kevin Kuehl)

Subject: Re: Query about teeth on Japanese Saws

Message-ID: <1991Sep26.114138.19364@cbfsb.att.com>

Date: 26 Sep 91 11:41:38 GMT

References: <7633@mindlink.bc.ca>

Sender: news@cbfsb.att.com

Organization: AT&T Bell Laboratories

Lines: 22

In article <7633@mindlink.bc.ca> Nick_Janow@mindlink.bc.ca (Nick Janow) writes:

>Efficient, yes.  Smooth cut, also yes.  Accurate, not that I've found.  Maybe I

>just got a bad producer, but I tried seven different blades [fine Dozuki] at

>one store and couldn't get a straight cut.  Any cut deeper than a cm or so

>would bow one way or the other.  I tried different holding techniques (even

>clamping the blade in a vise and moving the wood over it), different pressures

>and rates of cutting, different woods--nothing worked.  Each blade would

>produce a bowed cut, each blade different, but consistent for each blade

>(meaning the blade was the factor, not me).  I've heard other people say

>they've had similar problems.

I might be the woodworker.  I once read that a skilled ``Western woodworker''

--MORE--(78%)will have great difficulty using Japanese-style woodworking tools because

one has developed certain muscles and techniques that are completely wrong

for their style of tools.  But I can't say for sure since I have never used

Japanese-style saws.

--

Kevin Kuehl

ihlpm!kkuehl@att.att.com

cbnewsf!kkuehl@att.att.com

IH 4E-313 (708)979-9996

End of article 4646 (of 4701)--what next? [npq] Article 4647 (54 more) in rec.woodworking:

From: cola@hplvec.LVLD.HP.COM (Ken Colasuonno)

Subject: Re: Threading pipe for pipe clamps

Message-ID: <3560008@hplvec.LVLD.HP.COM>

Date: 25 Sep 91 14:40:44 GMT

References: <1991Sep24.160813.6604@mlb.semi.harris.com>

Organization: Hewlett-Packard Co., Loveland, CO

Lines: 8

>    How hard is it to thread 3/4" black pipe for pipe clamps?

Easy!  I just go to my favorite plumbing supply store here in town

(R-N-R) and say: "4 feet of black 3/4 inch pipe threaded on the ends

please"!

:-)

Ken

End of article 4647 (of 4701)--what next? [npq] Article 4648 (53 more) in rec.woodworking:

From: phil@brahms.amd.com (Phil Ngai)

Subject: Re: Motor power

Message-ID: <1991Sep26.164829.29646@amd.com>

Date: 26 Sep 91 16:48:29 GMT

References: <1991Sep20.160046.274@caliban.uucp> <1991Sep24.161355.23782@amd.com>  <1991Sep25.200650.553@vlsi.waterloo.edu>

Sender: usenet@amd.com (NNTP Posting)

Organization: Advanced Micro Devices, Inc.

Lines: 11

dgbedros@vlsi.waterloo.edu (David Bedrosian) writes:

>You are incorrect about your calculations of power.  In actual fact,

>power is the integral of the product of the voltage and current over the

>period of the signal (assuming a periodic signal).  Simply multipling

Yes, I know that. I meant that the horsepower produced by a motor can

be NO MORE than the product of the voltage and current. Remember we

were talking about claims of 3 HP on a 115 volt device.

--

This posting is purely a personal opinion.

End of article 4648 (of 4701)--what next? [npq] Article 4649 (52 more) in rec.woodworking:

From: wilson@schaefer.math.wisc.edu (Bob Wilson)

Subject: Re: Using nails and screws

Keywords: screws, nails

Message-ID: <1991Sep26.183942.9056@schaefer.math.wisc.edu>

Date: 26 Sep 91 18:39:42 GMT

References: <1991Sep26.175904.6018@cbnewsh.cb.att.com>

Reply-To: wilson@schaefer.UUCP (Bob Wilson)

Organization: Univ. of Wisconsin Dept. of Mathematics

Lines: 11

I too cringe at all the nails and to a lesser amount at the screws. I

think many of the nails, though, are gluing clamps. Note how often he

uses a nail either instead of clamps or else to let him take the

clamps off right away. I am sure the glued joint is MUCH stonger than

the nails, so the nails become irrelvant as the glue dries. I might

not do it this way, but (a) it is in keeping with a background of

doing it commercially, where you have to get more done per hour, and

(b) for that matter it is potentially the way to go for all the people

who have been complaining that they can't have so many clamps!

Bob W.

wilson@math.wisc.edu

End of article 4649 (of 4701)--what next? [npq] Article 4650 (51 more) in rec.woodworking:

From: xnf@teal.csn.org (Xcaret Research)

Subject: Mounting a router

Message-ID: <1991Sep26.221304.22366@csn.org>

Date: 26 Sep 91 22:13:04 GMT

References: <1991Sep25.134553.10156@Think.COM> <15230016@hpspdla.spd.HP.COM>

Sender: xnf@teal.csn.org

Organization: Colorado SuperNet, Inc.

Lines: 6

Nntp-Posting-Host: teal.csn.org

I have a 3 hp Makita router that is very heavy, especially for small

pieces.  I have been told to cut a hole in by work bench (4x8 on horses)

and mount the router underneath.  My main concern with this setup is safety.

Any thoughts?

-JSKG

End of article 4650 (of 4701)--what next? [npq] Article 4651 (50 more) in rec.woodworking:

From: donb@crash.cts.com (Donald Bowen)

Subject: Unknowledgble sales clerks

Message-ID: <1991Sep26.195009.9542@crash.cts.com>

Date: 26 Sep 91 19:50:09 GMT

References: <4617.28d8a0ca@dcatla.uucp> <1991Sep21.043827.25979@amd.com> <1991Se p25.190830.15949@cbnewsk.att.com>

Distribution: usa

Organization: Crash TimeSharing, El Cajon, CA

Lines: 28

There has been a recent thread about service at sears and other

stores.  The trend has been solid for about 15 years now.  I buy a lot

of Sears tools and a lot of car parts.  About 15 years ago the good parts

houses and clerks started to dissapear.  They were replaced by places that

just want to sell chrome junk and kids that knew little more than the buzz

words on product sales lit.  The same thing occured in Sears and other

stores, large or small.  At the same time the computer retail store came

into being and these were staffed with people that could just have easily

sold shoes accross the street.  It is so bad in recent years that I almost

never ask anything of the floor people.  I just find it myself, pay for it,

then leave.  Mail order has also become a primary source.

--MORE--(62%)

One parts house I delt with in Spokane WA had been in the same

location since 1946.  The people behind the counter were family and had

grown up with the place.  When I needed a part, often they did not even

look in the book but went to the shelves.  The owner retired and sold

the place.  The people who took over gave over half the building to

chrome junk and worthless speed equipment.  When I went looking for parts

for my old cars they did not have them and were not interested in ordering.

Another parts house went the same direction when the owner died.

The kids running the place wanted to sell only speed equipment and would

only consent to sell other parts if no one else was around.

Both of these stores are no longer around.

DonB

End of article 4651 (of 4701)--what next? [npq] Article 4652 (49 more) in rec.woodworking:

From: Nick_Janow@mindlink.bc.ca (Nick Janow)

Subject: Re: Query about teeth on Japanese Saws

Message-ID: <7647@mindlink.bc.ca>

Date: 27 Sep 91 07:30:58 GMT

Organization: MIND LINK! - British Columbia, Canada

Lines: 17

gringort@wsl.dec.com (Joel Gringorten) writes:

> Anyway, the bottom line with Dozukis is that you have to set them up before

> they'll be of much use.  The teeth come set totally wrong and have to be

> completely reset for use. ... I remember seeing a pretty extensive procedure

> for tuning a dozuki somewhere...  Supposedly after this, they cut straight as

> an arrow.

I have yet to come across any mention of this procedure.  The books that do

mention the "disposable" dozukis don't mention any need to prepare them.  Those

same books do discuss the need to tune up planes, chisels, etc, before the

first use.

Some books do mention that saw sharpeners in Japan are quite respected.  :)

--

--MORE--(97%)

Nick_Janow@mindlink.bc.ca

End of article 4652 (of 4701)--what next? [npq] Article 4653 (48 more) in rec.woodworking:

From: smb@ulysses.att.com (Steven Bellovin)

Subject: Re: Norm and the NYW needs some new spark ...

Message-ID: <15544@ulysses.att.com>

Date: 26 Sep 91 23:20:31 GMT

References: <1991Sep25.134553.10156@Think.COM> <1991Sep25.213516.15498@amd.com>  <CPETTERB.91Sep26104450@mickey.glacier.sim.es.com>

Sender: netnews@ulysses.att.com

Distribution: usa

Lines: 9

In article <CPETTERB.91Sep26104450@mickey.glacier.sim.es.com>, cpetterb@glacier. sim.es.com (Cary Petterborg) writes:

> Actually, I'd like to see him hit a nail with a hammer and have the project

> that he has been carefully holding together fall apart.  ;-)

That's actually not quite a joke.  The times I've been a bit annoyed

by Norm's pneumatic nailer have been when he uses it to fasten something

that wouldn't stay together under the impacts of a regular hammer.

And I suspect that of all the tools he regularly uses, the nailer

is the one least likely to be owned by amateurs.

End of article 4653 (of 4701)--what next? [npq] Article 4654 (47 more) in rec.woodworking:

From: whit@milton.u.washington.edu (John Whitmore)

Subject: Hand planes versus power joiner

Message-ID: <1991Sep27.005936.5481@milton.u.washington.edu>

Date: 27 Sep 91 00:59:36 GMT

References: <1991Sep25.134553.10156@Think.COM> <1991Sep25.213516.15498@amd.com>  <DSR.91Sep26111022@chickpea.mitre.org>

Distribution: usa

Organization: University of Washington, Seattle

Lines: 26

In article <DSR.91Sep26111022@chickpea.mitre.org> dsr@mitre.org (Douglas S. Rand ) writes:

>Phil writes:

>>  I think using handplanes and such require time to develop the

>>  technique.

>It's more then that.  I find hand jointing a board edge positively

>tedious.  I've done it many times and I'm even reasonably good at it,

>but slight variance in pressure will yield a bad edge.  I really

>intend to buy a small jointer soon.

Visiting the Rotunda at U.Va., I noticed the paneled doors

--MORE--(58%)on several of the rooms (elliptical rooms, by the way... Mr.

Jefferson, the architect, had a certain flair).  The doors followed

the curvature of the walls, so the doors and frames were curved.

I think this would be very difficult to do with power tools,

but simply curving the sole of a (wooden) plane would make it

a simple task for hand tools.  There are a LOT of designs that

can be done freehand, and are both easier and more esthetic

than the crisp right-angled planes of 'perfect boxes'.

I like to think that anything I can design, I should

be able to build (somehow).  Hand tools give me more flexibility

than most of the professional power equivalents, and I ENJOY

that flexibility.  That's what makes woodworking a good hobby.

John Whitmore

End of article 4654 (of 4701)--what next? [npq] Article 4655 (46 more) in rec.woodworking:

From: crumley@doghouse.rtp.dg.com (Steve Crumley)

Subject: Re: Norm and the NYW needs some new spark ...

Message-ID: <1991Sep27.143338.8092@dg-rtp.dg.com>

Date: 27 Sep 91 14:33:38 GMT

References: <1991Sep25.134553.10156@Think.COM> <1991Sep25.213516.15498@amd.com>  <BDS.91Sep26083310@maui.linus.mitre.org>

Sender: usenet@dg-rtp.dg.com (Usenet Administration)

Reply-To: crumley@dg-rtp.dg.com

Distribution: usa

Organization: Data General Corporation, Research Triangle Park, NC

Lines: 22

In article <BDS.91Sep26083310@maui.linus.mitre.org>, bds@maui.linus.mitre.org (B arry D. Smith) writes:

|> >>Another comment I had was that there's a heavy reliance on power tools. Not

|>

|> >That's why it's the NEW YW. He specifically says he's going to use

|> >power tools.

|>

|> Actually, the Shakers made extensive use of power tools, even 150

|> years ago.  A couple of years ago I asked this newsgroup how that was

--MORE--(59%)Article 4656 (45 more) in rec.woodworking:

From: phil@brahms.amd.com (Phil Ngai)

Subject: Re: Phil Ngai's information was Re: Motor power

Message-ID: <1991Sep27.004038.18298@amd.com>

Date: 27 Sep 91 00:40:38 GMT

References: <1991Sep24.161355.23782@amd.com> <1991Sep25.200650.553@vlsi.waterloo .edu> <2394@camex.COM>

Sender: usenet@amd.com (NNTP Posting)

Organization: Advanced Micro Devices, Inc.

Lines: 30

chrisi@lloyd.Camex.COM (Chris Ischay) writes:

>Normally, I would not respond to a followup like David's, since it's on a

>topic in which I am not at all expert.  But this is the second example in

>recent memory of Phil Ngai giving technical information that has resulted

>in multiple followups containing corrections.  I'm also thinking of the

>table saw guide.

The table saw guide wasn't intended to be "technical information", just

a description of my biases in table saws and what I know about them.

Bennett Leeds was the primary objector (as I recall) and that was

because his biases were almost the opposite of mine (expensive and

lightweight vs inexpensive and heavy).

--MORE--(61%)

>So I'd like to ask publicly, and as politely as possible: Phil, what is

>your response?  Your information is likely to be taken as gospel truth by

I've already posted responses, I hope they make it to your site.

>You state in your .sig that the contents of your postings are purely

>personal opinion, but a statement like "voltage x amperage = power" isn't

>really an opinion.  In fact, you say it's a law of physics.

It is a law of physics, just depends on your terms and how picky you

are. Voltage and current in AC systems are vectors, power is usually

considered over a time period instead of instantaneous, etc, but all

this nit-picking is aside from the question of "what is this 3 HP on

115V stuff?"

--

This posting is purely a personal opinion.

End of article 4656 (of 4701)--what next? [npq] Article 4657 (44 more) in rec.woodworking:

From: dolata@lead.uazaic.arizona.edu (Dolata)

Subject: Re: Using nails and screws

Summary: Hidden nails -  okay.  Nails in the face - YUCK!

Keywords: screws, nails

Message-ID: <1991Sep26.191626.1644@arizona.edu>

Date: 27 Sep 91 02:16:25 GMT

References: <1991Sep26.175904.6018@cbnewsh.cb.att.com>

Sender: dolata@lead.aichem.arizona.edu

Distribution: world,local

Organization: University of Arizona AI Chemistry Lab, Tucson, AZ

Lines: 11

Nntp-Posting-Host: lead.aichem.arizona.edu

I watch NYW for grins and chuckles.   I like Norm... think I'd like to have

him as a neighbor.   (maybe I could borrow his tools then ;->  )

And I agree that his style is directed towards a different style of

woodworker than me,  so it doesn't bother me if he does things differently

than I do...   with one major exception -

I want to hit him when I see him using that damned pneumatic nailer to

put nails in the front of a carcase!   I'm sure he sets them and fills with

--MORE--(91%)wood putty,  which hardly shows after you stain it,  but...    how about

using hidden nails or screws???

End of article 4657 (of 4701)--what next? [npq] Article 4658 (43 more) in rec.woodworking:

From: bds@maui.linus.mitre.org (Barry D. Smith)

Subject: Re: Norm and the NYW needs some new spark ...

Message-ID: <BDS.91Sep26083310@maui.linus.mitre.org>

Date: 26 Sep 91 14:33:10 GMT

References: <1991Sep25.134553.10156@Think.COM> <1991Sep25.213516.15498@amd.com>

Sender: news@linus.mitre.org (News Service)

Distribution: usa

Organization: The MITRE Corporation

Lines: 12

In-Reply-To: phil@brahms.amd.com's message of 25 Sep 91 21:35:16 GMT

Nntp-Posting-Host: maui.mitre.org

>>Another comment I had was that there's a heavy reliance on power tools. Not

>That's why it's the NEW YW. He specifically says he's going to use

>power tools.

Actually, the Shakers made extensive use of power tools, even 150

years ago.  A couple of years ago I asked this newsgroup how that was

possible and a few people gave indepth discussions of shops with

shafts and pulleys across the ceiling continuously driven by water or

horses.  Belts would hang from the pulleys to table saws, lathes, etc.

--MORE--(88%)Article 4659 (42 more) in rec.woodworking:

From: clewis@ferret.ocunix.on.ca (Chris Lewis)

Subject: Re: Motor power

Message-ID: <2624@ecicrl.ocunix.on.ca>

Date: 27 Sep 91 04:48:50 GMT

References: <1991Sep20.160046.274@caliban.uucp> <1991Sep24.161355.23782@amd.com>  <BDS.91Sep24161939@maui.linus.mitre.org> <1991Sep25.163743.21315@amd.com>

Organization: Elegant Communications Inc., Ottawa, Canada

Lines: 104

In article <1991Sep25.163743.21315@amd.com> phil@brahms.amd.com (Phil Ngai) writ es:

>bds@maui.linus.mitre.org (Barry D. Smith) writes:

>>Unfortunately that's not all there is to it.  If that were so, then

>>while many motors are overrated (12A rated at 3HP for an Hitachi

>>router, etc.), according to this many motors are grossly underrated,

>>which makes little or no sense at all from a sales standpoint.  For

>I haven't run across this, I admit I don't understand it.  Could it be

>a cooling issue? That is, the "under-rated" 2 HP motor can actually

>accept more than 2 HP of electricity, but you can't run it at that

>continuously without overheating?

--MORE--(19%)Electric motors come in three flavours: continuous duty, medium

duty and light duty.  These distinctions are very important in

larger (>3HP) motors, but with the motors we typically use in a

shop (drills, saws, sanders etc.) they get a bit blurred.

Most medium sized universal motors (eg: 1/4-2HP) are continuous duty,

because they tend to be used in things that may be run for long periods

of time.  Like pumps, furnace blower fans and so on.

Normal HP ratings are made when taking the duty cycle into account.

For example, a continuous rated 2HP motor is designed to deliver

2HP continously.  This means that a 2HP continuous rated motor

can often deliver *more* than 2HP for short periods of time, but

if you were to try to run the motor continuously above that

rating, you'd reduce the lifetime of the motor (ie: running too

hot and the windings deteriorate).

Big motors often have separate HP ratings for continuous or

momentary use.  Where momentary is usually meant to be in minutes.

As one example, I remember reading of a gokart that was powered

by four automotive starter motors.  A starter motor is probably rated

around 1-2HP momentary.  Continous rating would be far lower.

But this gokart was capable of hitting >100 MPH - The battery pack

--MORE--(41%)was made up of 5 sets of 12V batteries in series, so they were

feeding 70 volts to the poor starter motors.  Which, given the top

speed this thing could hit, suggests to me that they were getting

somewhere around 10-15HP *each*.  The lifetime at that power normally

figured in seconds, but they packed the motors with dry ice before

each run.

"a 2 HP can actually accept more than 2 HP of electricity" is

essentially true - when you start loading the motor beyond 2HP,

it will increase the current drain substantially trying to keep

up.  There is two limits to this: one that beyond a certain point

the motor will stall when it runs out of flux - and stall current

is usually WAY above the motor's rated current (and above the

breaker).  The other factor is heating.

>I should change my statement to:

>       The horsepower produced by a motor is no more than the

>       product of the rated amperage multiplied by the operating

>       voltage divided by the watts to horsepower conversion

>       factor. The continous horsepower may be less if adequate

>       cooling is not available.

--MORE--(59%)Then derate by 25-40%.

>>well pump.  When I told this story to a physicist friend of mine, he

>>said it made perfect sense.  No matter how well made a motor is, the

>>continuous horsepower rating will be significantly LESS than 746

>>watts/HP due to friction and any other ways of losing power.  He also

>Yes, but the amount of friction in a motor is probably less than 5%.

>Those bearings are pretty good, you know. Look how long it takes

>a router or table saw to spin down.

The one factor you're missing is that of coil heating.  A modern

1 HP motor is probably producing 200-300 watts in heat - that is

power lost.  That's what the "F" in TEFC is for.  As a general rule

of thumb, an "honest" 1HP motor will consume somewhere around 10-12A on

a normal 110V circuit.  Which represents 1100 to 1320 watts.  Ie: 55-65%

efficiency.  But I've seen a 1/2HP motor rated at 12A (A table saw made

in the 40's - the motor weighed a LOT more than a comparable modern

motor.  It took two people to comfortably lift the motor.)

In industry (my father is a civil engineer and designs with electric

motors up to 1000HP and larger), they use a 1KW/HP rule-of-thumb.

(bigger motors are more efficient.)

--MORE--(80%)

>>said the overrated motors were also correct in that they used a

>>standard test for rating a motor that gives the peak horsepower a

>>motor generates before it will break under load.  Working with 746

>That stuff is b.s. It depends on spinning the motor up and then using

>rotational inertia to "develop" horsepower. A.K.A. the Sears method.

Well, there's a certain amount of justification to it.  In a table saw

a heavier armature is better than a lighter one - the motor speed

will remain more consistent, and be less likely to bog down.  BUT,

it is BS when one manufacturer compares his developed HP (or stall

HP) to another's continuous shaft HP.

Stall HP is another measure that some companies use.  This is the

maximum power that the motor can deliver before stalling out.  But

running the motor in that condition will destroy it quickly, and

your circuit breakers are likely to trip.

--

Chris Lewis; UUCP: ...!{cunews,uunet,latour}!ecicrl!clewis;

DOMAIN: clewis@ferret.ocunix.on.ca; Phone: Canada 613 832-0541

Psroff info: psroff-request@ferret.ocunix.on.ca

Ferret mailing list: ferret-request@ferret.ocunix.on.ca

End of article 4659 (of 4701)--what next? [npq] Article 4660 (41 more) in rec.woodworking:

From: tw@Atherton.COM (Tw Cook)

Subject: Re: Norm and the NYW needs some new spark ...

Message-ID: <38484@athertn.Atherton.COM>

Date: 26 Sep 91 17:28:56 GMT

References: <15230015@hpspdla.spd.HP.COM> <7350@hplabsz.hplabs.hpl.hp.com>

Sender: news@athertn.Atherton.COM

Organization: Atherton Technology - Sunnyvale, CA

Lines: 9

One of our local PBS stations mentioned something to the effect that the upcomin g new season would have more basic projects that are doable in a home shop with  basic tools. I hope they know what they're talking about.

I particularly agree with the comment about spending more time on design. Having  just gone through the process of designing and building a crib-mattress sized b ed for my 2.5 year old (first semi-big project I've done without Norm or Woodsmi th or the like

to help out), I realize that I could use some help in this area (although it did n't turn out bad-I stole design portions from various similar projects I found h ere and there). I also would enjoy more depth on finishing, although I'm beginni ng to figu

t out now.

--MORE--(73%)Article 4661 (40 more) in rec.woodworking:

From: wojcik@crl.dec.com (Ted Wojcik)

Subject: Re: Norm and sparks

Message-ID: <1991Sep27.004928.22364@crl.dec.com>

Date: 27 Sep 91 00:49:28 GMT

References: <1991Sep26.163055.13529@Think.COM> <1991Sep26.205412.19573@ibmpa.awd pa.ibm.com>

Sender: news@crl.dec.com (USENET News System)

Reply-To: wojcik@crl.dec.com

Distribution: usa

Organization: DEC Cambridge Research Lab

Lines: 32

In article <1991Sep26.205412.19573@ibmpa.awdpa.ibm.com>, steve@panews (Steve DeJ arnett) writes:

>

>       But then again, who knows.  Also, the real New Yankee Workshop (the

> building) is not at Norm's house.  It's on someone else's property (my guess

> would be Morash's, but who knows).

>

> Steve DeJarnett                       Internet: steve@ibmpa.awdpa.ibm.com

> IBM PSP Palo Alto             UUCP:     uunet!ibmpa.awdpa.ibm.com!steve

> (415) 855-3510                        IBM VNET: dejarnet at ausvmq

--MORE--(50%)Article 4662 (39 more) in rec.woodworking:

From: leavey@tis.llnl.gov (Jeff Leavey)

Subject: Jet 14 in and Delta 16 inch comments PLEASE!

Message-ID: <107822@lll-winken.LLNL.GOV>

Date: 27 Sep 91 12:14:05 GMT

Sender: usenet@lll-winken.LLNL.GOV

Distribution: na

Organization: Lawrence Livermore National Labs, Livermore CA

Lines: 13

Nntp-Posting-Host: ncis.tis.llnl.gov

Hi All:

I hope this won't be duplication but the local tool store is having

a sale:

Any one have comments on either the JET 14 inch bandsaw or the

Delta 16 inch bandsaw??  Xmas is coming soon and guess what my

present is going to be!! :-)   Please send e-mail direct or post if

of general interest. Looking for comments on tool quality, good / bad

feelings, adjustability, etc...

--MORE--(95%)Article 4663 (38 more) in rec.woodworking:

From: ahearn@cadence.com (John Ahearn)

Subject: Re: Using nails and screws

Keywords: screws, nails

Message-ID: <1991Sep27.145352.3140@cadence.com>

Date: 27 Sep 91 14:53:52 GMT

References: <1991Sep26.175904.6018@cbnewsh.cb.att.com>

Sender: usenet@cadence.com (USENET News)

Organization: Harris Semiconductor

Lines: 26

I like to build furniture and while I don't use nails, I

certainly use screws.  Why? because I feel they lend strength to a joint.

I use screws in joints that are hidden.  Take pinning a tenon.  If

I don't intend the pin to show (i.e. maple pin into mahogany) I will recess

a screw and put in a plug.  Certainly the screw will hold as well as a pin

in this example.  Aside from being easier I don't have to worry about the

pin splitting the wood (more of a humidity problem than initial fit).

Screws are easier for pieces that you just don't want to take the

time to make a joint for, such as arm rests on chairs.  Drop a couple of

--MORE--(63%)screws through the top, plug it, and your done.

Another nice feature is you can screw something together, without

glue, and be able to use it.  Sure there are plenty joints that do the same

but I wouldn't want to rely on the joint not opening.  Some of the projects

I have built need to be taken apart from time to time, such as my table

clock.  If I had glued it together then I would have a difficult time

cleaning the clock mechanism.  Screws hold the project together while

remaining hidden.

This does not mean that I don't use or like joints, just MHO.

:John Ahearn

ahearn@cadence.com

End of article 4663 (of 4701)--what next? [npq] Article 4664 (37 more) in rec.woodworking:

From: cooper@sauron.Columbia.NCR.COM (bob.cooper)

Subject: Re: norm..blah blah blah

Message-ID: <1991Sep27.083122.5324@nncrcae.Columbia.NCR.COM>

Date: 27 Sep 91 12:31:21 GMT

References: <4646.28dda7f9@dcatla.uucp> <1991Sep23.175405.8834@genie.slhs.udel.e du> <1991Sep23.224905.20775@news.nd.edu>

Organization: NCR Corp, E&M-Columbia, Columbia, SC

Lines: 30

Nntp-Posting-Host: sauron.columbiasc.ncr.com

In article <1991Sep23.224905.20775@news.nd.edu> jstiver@seurat.helios.nd.edu (Ja mes Stiver) writes:

>In article <1991Sep23.175405.8834@genie.slhs.udel.edu>, evh@genie.slhs.udel.edu  (Troy Saville) writes:

>|>

>|> I thought woodworking/carpentry was Norm's job. So its understandable

>|> why he has all those tools and 3 walls of pipe clamps. I wouldn't

>|> doubt it if he writes all those tools off as a business expense.

>|>

>|> --

>|> - Maryland State Frisbee Championships,Seneca Creek State Park, Sept. 21/22,  91'

--MORE--(65%)Article 4665 (36 more) in rec.woodworking:

From: dsr@mitre.org (Douglas S. Rand)

Subject: Re: Hand planes versus power joiner

Message-ID: <DSR.91Sep27105958@chickpea.mitre.org>

Date: 27 Sep 91 15:59:58 GMT

References: <1991Sep25.134553.10156@Think.COM> <1991Sep25.213516.15498@amd.com>

<DSR.91Sep26111022@chickpea.mitre.org>

<1991Sep27.005936.5481@milton.u.washington.edu>

Sender: news@linus.mitre.org (News Service)

Distribution: usa

Organization: The MITRE Corporation, Bedford, MA

Lines: 52

In-Reply-To: whit@milton.u.washington.edu's message of Fri, 27 Sep 1991 00:59:36  GMT

Nntp-Posting-Host: chickpea.mitre.org

In article <1991Sep27.005936.5481@milton.u.washington.edu> whit@milton.u.washing ton.edu (John Whitmore) writes:

In article <DSR.91Sep26111022@chickpea.mitre.org> dsr@mitre.org (Douglas S. R and) writes:

>Phil writes:

>>  I think using handplanes and such require time to develop the

--MORE--(31%)   >>  technique.

>It's more then that.  I find hand jointing a board edge positively

>tedious.  I've done it many times and I'm even reasonably good at it,

>but slight variance in pressure will yield a bad edge.  I really

>intend to buy a small jointer soon.

Visiting the Rotunda at U.Va., I noticed the paneled doors

on several of the rooms (elliptical rooms, by the way... Mr.

Jefferson, the architect, had a certain flair).  The doors followed

the curvature of the walls, so the doors and frames were curved.

I think this would be very difficult to do with power tools,

but simply curving the sole of a (wooden) plane would make it

a simple task for hand tools.  There are a LOT of designs that

This was more likely cut with a saw and then smoothed with a plane.  You

couldn't afford Mr. Jefferson's carpenters either.

can be done freehand, and are both easier and more esthetic

than the crisp right-angled planes of 'perfect boxes'.

I agree.  But most of those designs still have more flat straight joints

between pieces then curved.  Joinery is not (at least normally) curved.

--MORE--(66%)

I like to think that anything I can design, I should

be able to build (somehow).  Hand tools give me more flexibility

than most of the professional power equivalents, and I ENJOY

that flexibility.  That's what makes woodworking a good hobby.

I was certainly not arguing that a shop should have a power jointer and

no hand planes.  I love hand planes.  They leave a very nice surface,  and,

as you've pointed out,  can do things that would be impracticle or unsafe

with a power tool.  But hand planes *are* very tedious when you are

squaring up lumber for glue up or flattening a board.

As an amateur, I have limited time to split between woodworking and

other responsibilities and hobbies (like taking care of the house,

raising our daughter,  reading, working, etc.).  Spending time

being frustrated over hand jointing isn't a turnon for me.  I don't

speak for the world,  just myself.

--

Douglas S. Rand

Internet:   <dsrand@mitre.org>

Snail:      MITRE, Burlington Road, Bedford, MA

Disclaimer: MITRE might agree with me - then again...

--MORE--(99%)Article 4666 (35 more) in rec.woodworking:

From: gibbonsj@iccgcc.decnet.ab.com

Subject: Motor Basics - was Re: Motor Power

Message-ID: <1991Sep27.080343.5853@iccgcc.decnet.ab.com>

Date: 27 Sep 91 13:03:43 GMT

Lines: 113

In article <1991Sep25.200650.553@vlsi.waterloo.edu>, dgbedros@vlsi.waterloo.edu  (David Bedrosian) writes:

> In article <1991Sep24.161355.23782@amd.com> phil@brahms.amd.com (Phil Ngai) wr ites:

- Previous 'discussions' deleted -

> Phil,

>

> You are incorrect about your calculations of power.  In actual fact,

> power is the integral of the product of the voltage and current over the

> period of the signal (assuming a periodic signal).  Simply multipling

> the voltage and the current will give the correct results **provided** the

> voltage and current are in phase.  This is seldomly true with motors

We're finally getting closer to the truth!!

--MORE--(17%)

> running off AC.  For sinusoidal signals, the power can be calculated as

>

> P=V*I*cos(theta)

This IS a basic law of physics. If the load is PURELY RESISTIVE, then

P = V * I ( COS(0)=1 ) <-- Don't need a calculator for that one!

Also, power (P) consumed and Horsepower Produced are not equal.

>

> where V and I are rms quantities and theta is the phase angle between

> the voltage and the current.

>

> Next time you should check your "physics" before you make a statement as

> you have above that implies you are an authority on this subject!!

>

>

> David Bedrosian

> Dept. of Electrical and Computer Engineering

> University of Waterloo.

Dave,

Thanks for stating that so kindly; I am always wary of self-proclaimed

--MORE--(30%)     'experts'...

(all flames from this will be automatically directed to /dev/null) 8-}

It seems that this thread has gotten so tangled up that apples and oranges

and now being interchanged (and then thrown! 8-}  )

Please forgive the fact that it has been 11 years since I took my E + M

Fields class in college and may not remember all of the terms correctly

(I'm sure someone will flame away...).

First, there are 2 distinctly different classes of motors used in power

tools:

1.) The induction motor.  It gets its name from the fact that its

magneto-motive force is generated thru 'induced currents' in the

rotating armature by the field generated in the windings surrounding it.

There is a phase lag between the exciting field (created in the windings

surrounding the armature) and the opposing induced field (generated in

the armature) which attract each other and create torque.  The 2 interacting

--MORE--(47%)    fields do not track each other exactly (if they did, there would be no

torque; one field has to lead (and pull) the other one around - basic

operation of a motor). This 'S' (interacting magnetic field wave) rotates

circularly around the gap between the armature and field pole faces and

pulls the armateur around.  The 'SLIP' associated with this type of

excitation is why you will never see induction motors that spin at multiples

of 60HZ (such as 1800 RPM (you usually see 1750 RPM) or 3600 RPM (you

usually see 3450 RPM)).  Induction motors are limited (via the number of

pole faces) to multiples of the excitation frequency for their speed.

They will not run on DC and their speed can only be varied by changing

the frequency of excitation.

2.) The other variation uses brushes on a commutator to generate the opposing

field (generated in the armature) that is interacting with the fixed

field coils.  Since the particular armature coil that is being excited

is a function of armature position (or more correctly commutator blocks

coming in contact with the correct brushes) the speed is not dependant

on the excitation frequency, just the excitation voltage (hence magnetic

field strength).  Brush motors come in several flavors (shunt wound, etc.)

but I will not go into that right now.  The disadvantage of brush motors

is that they are more complicated (brush assemblies, commutators, etc),

but have the advantage that their speed can be varied (via a voltage change) --MORE--(74%)

and they can run on DC.

Horsepower, work performed, etc is an often misleading spec.  Torque would

probably be a more straightforward specification.  There are 2 components

that generate torque in a motor:

1 - The torque generated by the interacting fields in the motor itself and

2 - The rotational Inertia of the motor (or whole rotating system) as it is

spinning (energy of rotation).

A classic trick (most notably played by Sears) is to include the above two

together (includes rotational inertia of motor, belt, pulleys, blade) and

uses the instantaneous torque generated as a 'peak' torque, not a sustainable

torque of the system.  So when they say 'Generates 3 HP', this is when the

system is up to speed, no load, and you shove a board into the blade.  For an

instant it looks (to the board entering the blade) like the the torque generated

is 3HP, but as the blade slows down, the effective torque being passed to the

blade drops.  Neat marketing misconception - but it sells table saws...

I am not an expert on motors, nor do I claim to be an expert on physics...

--MORE--(94%)Just a EE who remembers some of his physics and EE classes info..

Hope this helps clear up this thread.....

---

John Gibbons                 N8OBJ                 Macedonia, Ohio

Internet Address:      gibbonsj%iccgcc.decnet@consrt.rok.com

"Welcome My Son, Welcome To The Machine" - Pink Floyd

End of article 4666 (of 4701)--what next? [npq] Article 4667 (34 more) in rec.woodworking:

From: gibbonsj@iccgcc.decnet.ab.com

Subject: Motor Basics - again

Message-ID: <1991Sep27.094308.5856@iccgcc.decnet.ab.com>

Date: 27 Sep 91 14:43:08 GMT

Lines: 92

I am reposting this due to confusion on rotational inertia vs momentum.

(Sorry 'bout the BRAIN CRAMP...)   8-]

Please forgive the fact that it has been 11 years since I took my E + M

Fields class in college and may not remember all of the terms correctly

(I'm sure someone will flame away...).

First, there are 2 distinctly different classes of motors used in power

tools:

1.) The induction motor.  It gets its name from the fact that its

magneto-motive force is generated thru 'induced currents' in the

rotating armature by the field generated in the windings surrounding it.

There is a phase lag between the exciting field (created in the windings

--MORE--(19%)    surrounding the armature) and the opposing induced field (generated in

the armature) which attract each other and create torque.  The 2 interacting

fields do not track each other exactly (if they did, there would be no

torque; one field has to lead (and pull) the other one around - basic

operation of a motor). This 'S' (interacting magnetic field wave) rotates

circularly around the gap between the armature and field pole faces and

pulls the armature around.  The 'SLIP' associated with this type of

excitation is why you will never see induction motors that spin at multiples

of 60HZ (such as 1800 RPM (you usually see 1750 RPM) or 3600 RPM (you

usually see 3450 RPM)).  Induction motors are limited (via the number of

pole faces) to multiples of the excitation frequency for their speed.

Induction motors usually require a seperate start winding to give

them a preferred direction on startup (usually done with another smaller

start winding with a capacitor in series to give a phase angle lead to

pull the armature in a preferred direction - usually dropped out of the

circuit by a centrifical (sp?) clutch when the motor is up to speed).

They will not run on DC and their speed can only be varied by changing

the frequency of excitation.  Startup current is also 3-5 times running

current.  Low speed startup torque is not the greatest.

2.) The other variation uses brushes on a commutator to generate the opposing

--MORE--(48%)    field (generated in the armature) that is interacting with the fixed

field coils.  Since the particular armature coil that is being excited

is a function of armature position (or more correctly commutator blocks

coming in contact with the correct brushes) the speed is not dependent

on the excitation frequency, just the excitation voltage (hence magnetic

field strength).  Brush motors come in several flavors (shunt wound, etc.)

but I will not go into that right now.  The disadvantage of brush motors

is that they are more complicated (brush assemblies, commutators, etc),

but have the advantage that their speed can be varied (via a voltage change)

and they can run on DC.  Their startup torque is very good, and the

startup current is less than that of an induction motor.

Horsepower, work performed, etc is an often misleading spec.  Torque would

probably be a more straightforward specification.  There are 2 components

that generate torque output in a motor:

1 - The torque generated by the interacting fields in the motor itself

(Generated energy of rotation) and

2 - The rotational momentum of the motor (or whole rotating system) as it is

spinning (Stored energy of rotation).

--MORE--(73%)A classic trick (most notably played by Sears) is to include the above two

together (includes rotational momentum of motor, belt, pulleys, blade) and

uses the instantaneous torque generated as a 'peak' torque, not a sustainable

torque of the system.  So when they say 'Generates 3 HP on a table saw', this

is when the system is up to speed, no load, and you shove a board into the

blade.  For an instant it looks (to the board entering the blade) like the the

torque generated is 3HP, but as the blade slows down, the effective torque

being passed to the blade drops.  Neat marketing misconception - but it

sells table saws...

I am not an expert on motors (although I did work on designing DC motor

controllers for a company whose logo looks like a dripping faucet), nor do I

claim to be an expert on physics...

Just a EE who remembers some of his physics and EE classes info..

Hope this helps clear up this thread.....(and doesn't generate too much more!)

---

John Gibbons                 N8OBJ                 Macedonia, Ohio

Internet Address:      gibbonsj%iccgcc.decnet@consrt.rok.com

--MORE--(94%)

"Welcome My Son, Welcome To The Machine" - Pink Floyd

--

John Gibbons                 N8OBJ                 Macedonia, Ohio

Internet Address:      gibbonsj%iccgcc.decnet@consrt.rok.com

"Welcome My Son, Welcome To The Machine" - Pink Floyd

End of article 4667 (of 4701)--what next? [npq] Article 4668 (33 more) in rec.woodworking:

From: bennett@adobe.com (Bennett Leeds)

Subject: Re: Phil Ngai's information was Re: Motor power (long)

Message-ID: <21087@adobe.UUCP>

Date: 27 Sep 91 23:22:47 GMT

References: <2394@camex.COM>

Sender: news@adobe.COM

Reply-To: bennett@adobe.com

Lines: 103

In article <2394@camex.COM> chrisi@lloyd.Camex.COM (Chris Ischay) writes:

> In article <1991Sep25.200650.553@vlsi.waterloo.edu>

> dgbedros@vlsi.waterloo.edu (David Bedrosian) writes:

> >In article <1991Sep24.161355.23782@amd.com> phil@brahms.amd.com (Phil

> Ngai) writes:

> <<The laws of physics are fixed. Voltage times amperage = power.  If a so

> <<called reputable company makes a claim that defies the laws of physics,

> <<then the so called reputable companies are liars, plain and simple.

>

> <<If you want to know how much power a (single phase) tool produces, just

> <<take the rated amperage and multiply by the operating voltage. That's

> <<all there is to it. Period.

> {...}

--MORE--(17%)> <You are incorrect about your calculations of power.  In actual fact ...

> <     [different way to calcuate power]

>

> <Next time you should check your "physics" before you make a statement as

> <you have above that implies you are an authority on this subject!!

>

> Normally, I would not respond to a followup like David's, since it's on a

> topic in which I am not at all expert.  But this is the second example in

> recent memory of Phil Ngai giving technical information that has resulted

> in multiple followups containing corrections.

> {...}

> I don't at all mean to publicly embarrass you or anything like that, but you

> put out these posts and then people write and yell at you about them.  What

> gives?

And to think just a few days ago posters were commenting themselves on

the lack of flame wars in rec.woodworking. IMHO, this thing has gotten way out

of hand. Not to defend Phil, as I'm sure he is more than capable of defending

himself should he choose to do so, but his information is not nearly as

"incorrect" as the followup posts imply. I think the problem the followup

posters have had is with the tone of absoluteness they sense in his posting.

--MORE--(37%)

Practically speaking, looking at the amperage rating stamped on the

housing of different motors *is* often a more reliable way to compare power

output than to look at the manufacturer's hp ratings. And if a manufacturer

rates a motor with a higher hp than you'd get by doing the appropriate math on

voltage and amperage then the manufacturer *is* misleading you. That's what

Phil was saying - he just didn't couch it in the niceties that I did, probably

because he's (rightfully) pissed at manufacturers for inflating hp claims.

In article <BDS.91Sep24161939@maui.linus.mitre.org> bds@maui.linus.mitre.org

(Barry D. Smith) writes:

> >The laws of physics are fixed. Voltage times amperage = power.  If a so

> >called reputable company makes a claim that defies the laws of physics,

> >then the so called reputable companies are liars, plain and simple.

> >

> >If you want to know how much power a (single phase) tool produces, just

> >take the rated amperage and multiply by the operating voltage. That's

> >all there is to it. Period.

>

> Unfortunately that's not all there is to it.  If that were so, then

--MORE--(57%)> while many motors are overrated (12A rated at 3HP for an Hitachi

> router, etc.), according to this many motors are grossly underrated,

> which makes little or no sense at all from a sales standpoint.

Amperage ratings are not stamped on motors for sales purposes - they are

there so you know how many amps the motor may draw under certain circumstances

and so be able to size your wiring and circuit breakers. Does anyone know what

these "certain circumstances" are (free running, running under load, stall

current, startup current, or ?), and whether there are some laws (or UL

requirements) that control what gets stamped for amperage on motors?

> a physicist friend of mine {...} also

> said the overrated motors were also correct in that they used a

> standard test for rating a motor that gives the peak horsepower a

> motor generates before it will break under load.

Notice he said "*a* standard test" implying that there is more than one

standard test for hp ratings. This is like amperage ratings in audio

amplifiers. A couple of decades ago some used RMS, some used peak to peak

--MORE--(76%)(remember "music power"?), some used peak with only one channel running and

then multiplied the result by two. The differences between rating systems

affected the numerical results much more than the true differences in amp power

output.

Usually one finds that universal motors have more inflated hp ratings

than induction motors. Comparing two motors of these different construction is

difficult - gearing friction can play a not insubstantial part in reducing

effective hp (universals rotate much faster than inductions, hence they need to

be geared down for use in table saws, etc.).

> Working with 746

> watts/HP is like doing freshman physics problems with the coefficient

> of friction=0.

Forget all that stuff. The rule of thumb I use is that somewhere

between 8 and 10 amps at 110 volts is 1 hp (220 volt motors need half as many

amps to produce the same hp). This gives results that more or less jive with

the hp ratings of the better motor manufacturers (Baldor, Leeson, Dayton,

Delta, etc.) Be careful, though. Small amperage rating differences may not be

--MORE--(94%)meaningful because amperage ratings appear to vary somewhat with the

conservativeness of the company. I wouldn't be surprised to find that a 13.5

amp motor is more powerful than another motor rated at 15 amps, for instance.

"Just my personal opinions."

- Bennett Leeds

bennett@adobe.com

End of article 4668 (of 4701)--what next? [npq] Article 4669 (32 more) in rec.woodworking:

From: wilson@schaefer.math.wisc.edu (Bob Wilson)

Subject: Re: Motor Basics - again

Message-ID: <1991Sep27.175546.26476@schaefer.math.wisc.edu>

Date: 27 Sep 91 17:55:46 GMT

References: <1991Sep27.094308.5856@iccgcc.decnet.ab.com>

Reply-To: wilson@schaefer.UUCP (Bob Wilson)

Organization: Univ. of Wisconsin Dept. of Mathematics

Lines: 55

All of the discussion of the phase angles and power factors is

interesting, and so far as my senile memory allows me an opinion seems

largely correct. It is pretty far from the thread of rec.woodworking,

though. If we see clerks who cannot even tell us what aisle the

chisels are on, how far will we get asking them for the inductive

reactance of motor?

I think the bottom line is this: (assuming really only the laws of

thermodynamics and a few definitions of units) No motor, on AC at any

frequency or on DC, can deliver MORE than the power corresponding to

the product of the amps it is drawing and the volts it sees at its

terminals, where "delivers" means what the motor itself can produce.

--MORE--(33%)A tool with a motor in it can draw on energy stored as momentum for a

short while, but I think the various moments of inertia in any home power

tool make that "short while" last only a small fraction of an inch

when cutting wood with a router, a jointer, a planer, or any kind

of circular saw. (You could try adding lead wheels to your bandsaw

to get you through knots...)

A tool and/or motor can deliver LESS than the power consumed due

to inefficiencies which may be friction in bearings, magnetic flux

leakage, etc. The power consumed may be less than volts x amps because

of the power factor, i.e. the slipped phase between voltage and

current. BUT, short of a mechanical brake for measurements, we won't

know those. Even if the motor carries a power factor rating on the

label that is for certain load conditions we probably won't know.

For the purposes of rec.woodworking I think all that matters, i.e. all

we can make use of, are: (1) If tool A draws more power than tool B

from the same line voltage, A MIGHT be able to deliver more power to

the blade. (But are you going to believe the current draw figures from

two manufacturers are comparable? Measured under the same

circumstances? Honest and not a marketing reaction to the points we

have been discussing?) (2) If tool A claims to deliver more power (in

HP) than roughly 4/3 times the volts times the amps, and claims

--MORE--(73%)to deliver that continuously (not for a few milliseconds as it

draws on momentum,) it is fundamentally lying. The lie may be in

using a new definition for horsepower, when there is a standard one,

but whatever it is makes the claimed figure useless for comparison

shopping. (An exception, perhaps: The vacuum cleaner manufacturers

have an agreed on unit of horsepower which isn't a horsepower but may

be comparable among vacuum cleaners.)

I suggest we could get better information by asking lots of people (a)

what tools do you have and (b) how are they at cutting oak crossgrain,

ripping maple, ..... Of course we would still get lots of noise: How

sharp do YOU keep your blades and router bits? How do you describe

"cuts easily" vs. "bogs down." My point is that the labels are almost

worthless and, if this issue is this important, we have to look

elsewhere.

Bob Wilson

wilson@math.wisc.edu

Any more refined m

End of article 4669 (of 4701)--what next? [npq] Article 4670 (31 more) in rec.woodworking:

From: willo@youcon.Central.Sun.COM (Doug Willoughby  Area SE  - Midwest Area )

Subject: Re: Norm and the NYW needs some new spark ...

Message-ID: <ke7579INN5k@cronkite.Central.Sun.COM>

Date: 27 Sep 91 20:30:33 GMT

References: <15230015@hpspdla.spd.HP.COM> <7350@hplabsz.hplabs.hpl.hp.com> <3848 4@athertn.Atherton.COM>

Sender: willo@youcon (Doug Willoughby  Area SE  - Midwest Area )

Organization: Sun Microsystems - Southfield, Michigan

Lines: 36

NNTP-Posting-Host: youcon.central.sun.com

Well, I've kept quite long enough...

It may see like Norm has no spark left, but I don't believe

that's the case.  What Norm lacks is soul.  All of his projects

are copies (with some small modification) of existing pieces.

No added creativity.  No design required.

Most of his woodworking techniques are production oriented.

Skill isn't much of an issue.  Can you read and follow directions?

Then you can set up a dovetail jig.  Jointing board?  Set up your

jointer correctly and clamp hard.  No problem.

--MORE--(49%)Article 4671 (30 more) in rec.woodworking:

From: donb@crash.cts.com (Donald Bowen)

Subject: motors

Message-ID: <1991Sep27.190402.2343@crash.cts.com>

Date: 27 Sep 91 19:04:02 GMT

Distribution: usa

Organization: Crash TimeSharing, El Cajon, CA

Lines: 69

I found an article on electric motors in the Fine Woodworking

book "Power Saws And Planers."  It discussed the  types of motors,

motor wiring, motor identification tags, and motor power.  All

these things are standardized by the National Electrical

Manufactures Association (NEMA).  Not every one now uses these

standards and many older motors didn't either.

As far as motor power, it said much the same thing as had been

said on the net.

Continous duty HP is the maximum power that can be developed

by the motor for 24 hours.  The limiting factor is how well the

--MORE--(28%)motor gets rid of heat.  Some times the manufacture will rate the

motor for shorter periods in order to get a higher rating, but that

should be indicated on the tag.

Developed peak HP is the maximum power developed just before

the motor stalls.  In this state the motor heats rapidly.  If there

is an internal thermo overload switch, the motor will then shut

down.  The article did not recomend the internal thermo over load

protection for motors on wood working machinery.  The reason is

that when they cool, they will automaticly restart.

The article also described the NEMA tag.  This tag shows the

voltage, current, HP, effiency, power factor, and several other

things.  Not all motors have this tag.

The wiring diagrams for single phase and three pase motors

were also given.

single phase induction motor wiring

Volt    Line wire color         Switched          NEMA wire number

115     Black (hot)             Yes               1,3,8

White (neutral)                           2,4,5

--MORE--(59%)        Green (ground)          No                motor housing

230     Black (hot)             Yes               1

Red (hot)               Yes               4,5

Green (ground)          No                motor housing

No                2,3,8

Swap 5 & 8 to reverse direction.

Three phase induction motor wiring.

Volt    Line wire color         Switched          NEMA wire number

230     Black (hot)             Yes               3,9

RED (hot)               Yes               2,8

White (hot)             Yes               1,7

Green (ground)          No                motor housing

No                4,5,6

460     Black (hot)             Yes               3

RED (hot)               Yes               2

White (hot)             Yes               1

Green (ground)          No                motor housing

No                4,7

No                5,8

--MORE--(91%)                                No                6,9

To reverse direction, interchange any two motor leads.

Wire numbers seperated by commas are tied together.  If there is

no connect to the line, these wires are insulated from line wires

and housing.

DonB

End of article 4671 (of 4701)--what next? [npq] Article 4672 (29 more) in rec.woodworking:

From: lester@naomi.b23b.ingr.com

Subject: Re: Motor Basics - was Re: Motor Power

Message-ID: <1991Sep27.213308.8317@b11.ingr.com>

Date: 27 Sep 91 21:33:08 GMT

References: <1991Sep27.080343.5853@iccgcc.decnet.ab.com>

Sender: usenet@b11.ingr.com (Usenet Network)

Reply-To: lester@naomi.b23b.ingr.com

Organization: Dazix, An Intergraph Company

Lines: 23

In article <1991Sep27.080343.5853@iccgcc.decnet.ab.com>,

gibbonsj@iccgcc.decnet.ab.com writes:

|>

|> A classic trick (most notably played by Sears) is to include the above two

|> together (includes rotational inertia of motor, belt, pulleys, blade) and

|> uses the instantaneous torque generated as a 'peak' torque, not a sustainable

|> torque of the system.  So when they say 'Generates 3 HP', this is when the

|> system is up to speed, no load, and you shove a board into the blade.  For an

|> instant it looks (to the board entering the blade) like the the torque genera ted

--MORE--(68%)|> is 3HP, but as the blade slows down, the effective torque being passed to the

|> blade drops.  Neat marketing misconception - but it sells table saws...

|>

So why did Sears stop at 3HP?  If you shove the board in faster, it looks

(to the board entering the blade) like you will have even more horsepower.

I guess they didn't want to be too unrealistic with their HP ratings.

My saw can develop almost infinite horsepower!!!! :-)

- Les

--

Les Bartel                      lester@naomi.b23b.ingr.com

End of article 4672 (of 4701)--what next? [npq] Article 4673 (28 more) in rec.woodworking:

From: sgs8r@kelvin.seas.Virginia.EDU (Steve Strickland, sgs8r@virginia.edu)

Subject: Re: Norm and the NYW needs some new spark ...

Message-ID: <1991Sep25.231418.6334@murdoch.acc.Virginia.EDU>

Date: 25 Sep 91 23:14:18 GMT

References: <CPETTERB.91Sep24162102@mickey.glacier.sim.es.com> <1991Sep25.122008 .11193@cbfsb.att.com> <1991Sep25.162748.28970@mlb.semi.harris.com>

Sender: Steve Strickland, sgs8r@virginia.edu

Organization: Dept. of Systems Eng., U.Va.

Lines: 61

Well, I may as well add my 2 cents....

I spoke to Norm a couple of years back (after the first NYW season) at a book

signing appearance in the Boston area (the 1st book). He made the following

comments:

- The book project came first. The TV show was a spur-of-the-moment thing at

the last minute. A slot opened up in the PBS schedule or something.

-The target was an "average" (whatever that is) home handyperson, who was

interested in making good quality furniture, but unlikely to feel up to carving

a ball-and-claw leg, for example. Shaker style furniture was chosen because it

--MORE--(30%)Article 4674 (27 more) in rec.woodworking:

From: howardr@hp-lsd.COL.HP.COM (Howard Rebel)

Subject: Re: Sears Bashing (Was Re: Which table saw. Again! (sorry))

Message-ID: <8940010@hp-lsd.COL.HP.COM>

Date: 27 Sep 91 16:16:10 GMT

References: <4617.28d8a0ca@dcatla.uucp>

Organization: Colorado Springs Division--PL 17

Lines: 25

My sears tablesaw had flat cast iron extensions but a

warped (front to rear) table.  It is the three horse

really 1.5 horse unit refered to in an earlier note

in this string.

I was not looking forward to setting up the saw again

so I waited about 8 months befor asking sears to do

somthing about it.

The said that the saw was not a professional tool and

that I could not expect it to be perfect.  The did

order a new top for me and gave me the choice of keeping

the one that came with the saw or having them install

the new one.

--MORE--(74%)Article 4675 (26 more) in rec.woodworking:

From: phil@brahms.amd.com (Phil Ngai)

Subject: Re: Norm and the NYW needs some new spark ...

Message-ID: <1991Sep25.213516.15498@amd.com>

Date: 25 Sep 91 21:35:16 GMT

References: <1991Sep25.134553.10156@Think.COM>

Sender: usenet@amd.com (NNTP Posting)

Distribution: usa

Organization: Advanced Micro Devices, Inc.

Lines: 56

Well, I like Norm and Roy. I learn a lot from both of them They are

different, but there is no right or wrong, one is not better than the

other, they have different goals.

steveo@think.com (stephen anthony) writes:

>I wrote a letter to Morash, etal, with some complaints and ideas. What

>really got me was when Norm made the 6 drawer chest (real big one; 6' high

>or so). His (Norm's) quote was something like "well after sanding this with

>every tool in the shop..."; that was somewhat frustrating. I mean what

Can't you tell a joke when you hear one? And can't someone who enjoys

using a handplane also enjoy scraping a big project?

--MORE--(37%)Article 4676 (25 more) in rec.woodworking:

From: mlord@bnr-rsc.UUCP (Mark Lord)

Subject: Re: circ -> pseudo table saw : good or bad idea?

Message-ID: <5480@bnr-rsc.UUCP>

Date: 25 Sep 91 19:08:13 GMT

References: <14845.28da1296@ecs.umass.edu> <15284@sybase.sybase.com>

Reply-To: mlord@bnr-rsc.UUCP (Mark Lord)

Organization: Bell-Northern Research, Ottawa, Canada

Lines: 24

<>...question:  I've heard of "workstation" tables that will allow you

<>to attach hand held tools for added versatility, eg circular saw becomes

<>a poor man's table saw, drill -> drill press, and such.  I'm sorry this

<>sounds like such a hack, but I'd be interested in hearing if anyone

<>has opinions/knowledge of such things, and where to get them.  I'm

<>concerned about safety, and cost/quality trade-offs.  I plan on getting

<>a router table, but thought I'd find out if a more versatile router

<>plus circular saw table might be a reasonable option.

Keep in mind that such a "poor man's tablesaw" is usually only a poor

partial implementation of the real thing.  It will not be as stable, vibration

free, or as easily adjusted as the real thing, and certainly not nearly as

accurate without a LOT of fiddling in use.  In addition, tablesaws have

--MORE--(69%)Article 4677 (24 more) in rec.woodworking:

From: de@moscom.UUCP (David Esan)

Subject: Re: Norm

Message-ID: <3033@moscom.UUCP>

Date: 26 Sep 91 12:07:40 GMT

References: <14845.28da1296@ecs.umass.edu> <4646.28dda7f9@dcatla.uucp>

Reply-To: de@moscom.UUCP (David Esan)

Organization: Moscom Corp., E. Rochester, NY

Lines: 26

In article <4646.28dda7f9@dcatla.uucp> isjrc@dcatla.uucp (John R. Caruso) writes :

>                                     I saw him once putting up a

>casing around a window with a pneumatic nailer. The guy

>watching said "I guess you don't use hammers anymore." Norm replied

>"Naaah, nobody uses a hammer anymore." I don;t know who he is supposed

>to appeal to but it isn't me.

You have taken his statement out of context.  The nobody that was refered to

was contractors.  Surely most of the people on the net reading this use a

hammer.  For the small projects that we do a pneumatic nailer is overkill.

But for a contractor whose income is derived from doing the most work in

--MORE--(65%)Article 4678 (23 more) in rec.woodworking:

From: peterd@hpgrla.gr.hp.com (Peter Davidoff)

Subject: Re: Sears Bashing (Was Re: Which table saw. Again! (sorry))

Message-ID: <2430009@hpgrla.gr.hp.com>

Date: 26 Sep 91 18:00:07 GMT

References: <4617.28d8a0ca@dcatla.uucp>

Organization: Hewlett-Packard, Greeley, CO

Lines: 42

John:

I bought a Sears table saw in 1986.  It was their top of the line flex

drive.  I suffered kick-back, burn marks and discouragement until 1988.

It was then that I met a real wood-worker (he did it for a living and

was good at it!).  I said, "Dwight, what can I get for this saw and what

should I buy?."  He said that the saw was probably fine, just needed a

tuning.  The next Saturday Dwight showed up and we spent two hours

tuning up the saw.  It cuts, in my humble saw-dust opinion, just fine.

The tuning required making the saw blade parallel with the mitre guage

slot, then making the rip fence parallel with the blade.  At the

beginning of every major project, I retune the rip fence, it takes about

2 minutes, if that.  I always check the blade alignment.  I now use

as an outline a tune-up guide printed by Woodsmith.  Dwight also

--MORE--(48%)Article 4679 (22 more) in rec.woodworking:

From: dolata@lead.uazaic.arizona.edu (Dolata)

Subject: Re: Taking care of wood

Summary: Ebony is brittle!!!

Message-ID: <1991Sep25.172841.1623@arizona.edu>

Date: 26 Sep 91 00:28:41 GMT

References: <2370001@hpsemc.cup.hp.com> <DSR.91Sep25181645@chickpea.mitre.org>

Sender: dolata@lead.aichem.arizona.edu

Distribution: world,local

Organization: University of Arizona AI Chemistry Lab, Tucson, AZ

Lines: 21

Nntp-Posting-Host: lead.aichem.arizona.edu

> I hope that your style doesn't use these sticks in contact sparring.  Ebony

> is a brittle wood,  fine for showing off forms with but I think it

> would probably crack if it contacted another stick, wall, or other hard

> surface.

A friend turned two walking sticks out of Ebony.   He said it was a pain

due to whipping and the fact that the tool wanted to walk over the surface

rather than bite.  I could see why they would be whippy;  they were 3' long

--MORE--(66%)and the majority of the shaft was about 5/8 or 1/2 inch dia.

Eventually he got 'em done. Lovely!!!  He gave me one in payment

for a debt, and we went out for a walk.   About 2 blocks into the walk

he commented on how he liked the heft and feel,  and thwacked his stick

with moderate (not high) force against a small ornamental tree trunk.

His stick shattered into about 5 pieces!  He stood there for at least

a minute and then just kinda sagged onto the curb.

I still have mine (that was 15 years ago) but I don't use it.

End of article 4679 (of 4701)--what next? [npq] Article 4680 (21 more) in rec.woodworking:

From: wilson@schaefer.math.wisc.edu (Bob Wilson)

Subject: Re: Norm and the NYW needs some new spark ...

Message-ID: <1991Sep26.143026.1842@schaefer.math.wisc.edu>

Date: 26 Sep 91 14:30:26 GMT

References: <15230015@hpspdla.spd.HP.COM> <CPETTERB.91Sep24162102@mickey.glacier .sim.es.com> <1991Sep25.122008.11193@cbfsb.att.com> <1991Sep25.162748.28970@mlb. semi.harris.com>

Reply-To: wilson@schaefer.UUCP (Bob Wilson)

Organization: Univ. of Wisconsin Dept. of Mathematics

Lines: 54

>>>>   NYW needs a kick in the rear ... anybody else agree?

>>No it needs Roy. :-)

>Norm Abrams and Roy Underhill have very different approachs to woodworking.

>I enjoy both NYW and The Woodwright Shop (filmed less than 15 miles from

>where I work BTW). I don't have any problem with Norm's endless procession

>of power tools. I like power tools and I enjoy getting to see some of them

>in use. Many people have commented about the lack of complexitiy or variety

>in Norms projects. You should write WGBH and tell them. The third season of

>NYW had many projects that were simpler than the second this was a result

>of requests from the viewers. It's probably too late to effect the 1992

>season. Norm has probably already started shooting those (they air in

--MORE--(36%)Article 4681 (20 more) in rec.woodworking:

From: styborsk@hpspkla.spk.hp.com (Randy K. Styborski)

Subject: Re: Threading pipe for pipe clamps

Message-ID: <4920053@hpspkla.spk.hp.com>

Date: 25 Sep 91 17:57:55 GMT

References: <1991Sep24.160813.6604@mlb.semi.harris.com>

Organization: Hewlett Packard Company, Spokane, Wa.

Lines: 8

In rec.woodworking, jmr@mebsuta.rtp.semi.harris.com (Mike Reid design eng x636)  writes:

How hard is it to thread 3/4" black pipe for pipe clamps?

Not hard at all.  You just pay the guy at the hardware store $1.50 per

clamp to do the work :-)

(Seriously, I probably wouldn't consider doing it myself.  I just couldn't

resist posting here.)

End of article 4681 (of 4701)--what next? [npq] Article 4682 (19 more) in rec.woodworking:

From: stanj@hpnmdla.sr.hp.com (Stan Jaffe)

Subject: Re: Using nails and screws

Message-ID: <4890173@hpnmdla.sr.hp.com>

Date: 27 Sep 91 18:09:28 GMT

References: <1991Sep26.175904.6018@cbnewsh.cb.att.com>

Organization: HP Network Measurements Div, Santa Rosa, CA

Lines: 28

In rec.woodworking, warren@cbnewsh.cb.att.com (warren.a.montgomery) writes:

The ongoing discussion of NYW brings to mind one of my reactions

to the show, which I find entertaining in any case.  Norm seems to

put all of his projects together with screws and nails, and I

cringe every time I see him haul out that nailer or power

screwdriver, not because it''s an exotic power tool, but because

my woodworking training and reading taught suggested that screws

>>  and nails are for rough carpentry and special situations, not for

holding together wooden furniture.

--

Warren Montgomery

att!iexist!warren

--MORE--(65%)

Warren, I think you hit the nail on the head there (ooh! sorry ;^) ).

Norm is a "master" carpenter, not a fine woodworker or designer/craftsman.

Most people who do "fine woodworking" prefer to use wood joints (dovetails,

mortise and tenon, etc) rather than metal fasteners.  I think Norm's

technique is appropriate for the pieces he makes.

Stan Jaffe    A.K.A.   stanj@sr.hp.com

HP Santa Rosa, Calif.

"Not responsible for any opinions but my own"

------------------------------------------------------------------

End of article 4682 (of 4701)--what next? [npq] Article 4683 (18 more) in rec.woodworking:

From: bach@jalapeno.cs.wisc.edu (Eric Bach)

Subject: Re: Motor power

Message-ID: <1991Sep26.150117.15067@spool.cs.wisc.edu>

Date: 26 Sep 91 15:01:17 GMT

References: <BDS.91Sep24161939@maui.linus.mitre.org> <17922@ccncsu.ColoState.EDU > <1991Sep25.213113.4899@cs.dal.ca>

Sender: news@spool.cs.wisc.edu (The News)

Organization: U of Wisconsin CS Dept

Lines: 32

In article <1991Sep25.213113.4899@cs.dal.ca> armstrng@cs.dal.ca (Stan Armstrong)  writes:

>I can't resist any longer. I am really curious. Surely somebody knows.

>

>I followed a similar string to this several months ago without finding

>an answer. My question is this. What is "developed horsepower"?

I got sufficiently curious about this to go look at a book about

electric motors.  (I'm not an engineer, in case you couldn't tell!)

Apparently they have a speed-torque curve that looks something like

this,

--MORE--(66%)        ^

|

|      y

torque  |    *    *

|   *       *

| *          x

|             *

--------------------->

RPM

and they are designed to run at the point "x".  Now, if you realize

that power = RPM * torque (up to constant factors), you can see plenty

of room for a manufacturer to fake it, say by quoting the power for

point "y", even though continuous use at that point would blow up

the motor.

I conjecture that something like this is going on.  Perhaps an expert

could comment.

--Eric Bach

bach@cs.wisc.edu

End of article 4683 (of 4701)--what next? [npq] Article 4684 (17 more) in rec.woodworking:

From: budd@elxsi.dfrf.nasa.gov (Gerry Budd)

Subject: Re: norm..blah blah blah

Message-ID: <BUDD.91Sep27103044@drynix.dfrf.nasa.gov>

Date: 27 Sep 91 17:30:44 GMT

References: <1991Sep23.224905.20775@news.nd.edu> <8305@ns-mx.uiowa.edu>

Sender: news@skipper.dfrf.nasa.gov

Organization: NASA Dryden, Edwards, Cal.

Lines: 34

In-reply-to: trocklin@umaxc.weeg.uiowa.edu's message of 25 Sep 91 14:22:46 GMT

In article <8305@ns-mx.uiowa.edu> trocklin@umaxc.weeg.uiowa.edu (Tom Rocklin) wr ites:

From article <1991Sep23.224905.20775@news.nd.edu>, by jstiver@seurat.helios.n d.edu (James Stiver):

[stuff about Norm's tools and their tax deductibility deleted]

>

> Wouldn't it be possible for anyone to take a tax deduction for the

> cost of their tools if they declared their shop a business?  I was

> considering doing this some day.  Just do what's necessary to be

> officially a small business.  If someone comes by to order something,

> show them some appallingly bad piece and they'd leave.

--MORE--(55%)Article 4685 (16 more) in rec.woodworking:

From: phil@brahms.amd.com (Phil Ngai)

Subject: developed horsepower

Message-ID: <1991Sep28.075319.17886@amd.com>

Date: 28 Sep 91 07:53:19 GMT

Sender: usenet@amd.com (NNTP Posting)

Organization: US Militia Title 10, Section 311(a)

Lines: 41

There have been some theories made about what developed horsepower is.

I won't claim to know, but want to present my theory for comment.  Some

people have suggested that developed horsepower is based on rotational

inertia. (I used to think this also.) I was reading "Power Primer", an

article on electric motors in Fine Woodworking's "Power Saws and

Planers" and it says:

'maximum developed horsepower' is advertising talk for a motor

whose claimed output is right up near its pullout point.

pullout or breakdown point ... If you don't cut the power or

reduce the load, the motor will overheat and burn up.

So that would indicate it's a heat dissipation issue.

--MORE--(46%)

One thing that didn't seem right to me is that even with all the

benefit of the doubt, you just can't get 3 horsepower, or 2238 watts of

power out of a 15 amp, 115 volt circuit.

I wonder if 3 developed horsepower is measured on a 115 volt circuit

which is NOT limited to 15 amps. If so, then that 3 HP is even more

bogus than I thought, in the case of routers with 5-15P ("normal")

plugs!

Some other interesting things from that article. They showed a id plate

from a (typical?) motor.  It was rated at 2 HP using 22 amps at 115

volts (you'd need a 5-30P plug?).  Full-load efficiency was 75%, power

factor 81%, rating was continuous at 40 degrees C.  Other common

time-duty ratings are 15 minutes, 30, and 60.

Also, "Practical Electrical Wiring" says that the wires to a motor must

have an ampacity of at least 125% of the name-plate ampere rating of

the motor. A disconnecting switch (for motors 2 HP and less) must be

rated at twice the ampere rating of the motor. I haven't found startup

current, the closest reference was "the motor will consume many more

amperes while starting, so that while starting, the drop will be much

higher than while running."

--MORE--(97%)

--

This posting is purely a personal opinion.

End of article 4685 (of 4701)--what next? [npq] Article 4686 (15 more) in rec.woodworking:

From: cervenka@oasys.dt.navy.mil (Peter Cervenka)

Subject: Re: Jet 14 in and Delta 16 inch comments PLEASE!

Message-ID: <11365@oasys.dt.navy.mil>

Date: 28 Sep 91 14:53:03 GMT

References: <107822@lll-winken.LLNL.GOV>

Reply-To: cervenka@oasys.dt.navy.mil (Peter Cervenka)

Organization: David Taylor Research Center, Bethesda, MD

Lines: 4

I would like to know where are JET tools manufactured, are they USA

made or imports? Some of their equipment and prices seemed interesting

to me, so I was wondering about the quality and if they are Taiwanese

clones. Tks.

End of article 4686 (of 4701)--what next? [npq] Article 4687 (14 more) in rec.woodworking:

From: wack@brutus.ncsl.nist.gov (John P. Wack)

Subject: Re: Norm

Message-ID: <920@dove.nist.gov>

Date: 26 Sep 91 15:12:49 GMT

References: <14845.28da1296@ecs.umass.edu> <4646.28dda7f9@dcatla.uucp> <3033@mos com.UUCP>

Sender: news@dove.nist.gov

Organization: National Institute of Standards and Technology

Lines: 4

One other question: if you had to have one plane in your shop, what type would

it be? Jack, fore, smoothing?

Thanks, John Wack - NIST

End of article 4687 (of 4701)--what next? [npq] Article 4688 (13 more) in rec.woodworking:

From: jps@uh.msc.edu (Jeff P. Sorvik)

Subject: Tuning a Bandsaw

Keywords: bandsaw tuning

Message-ID: <4914@uc.msc.umn.edu>

Date: 26 Sep 91 15:33:31 GMT

Sender: news@uc.msc.umn.edu

Reply-To: jps@msc.umn.edu

Organization: Minnesota Supercomputer Center

Lines: 24

I am looking for information regarding a book about tuning a band saw. I

know this was covered a while ago, but at the time I did not have a

decent band saw. I just purchased an older saw made by Bett-Mar

(minneapolis).  Apparently this

company was in business for a short while, but they made what appears to be

a great saw (all the goodies like cast-iron frame, good bearings, very

adjustable blade guides, etc..).  Anyway, I know I need to make some

adjustments, so I am looking for some help.  Any suggestions?

Cheers,

--MORE--(56%)

_                  _

( )                ( )

/\__/~\-/______________/~\-/_/\

|   ----/-----sawyer-------/-   |

_______________________________\_____/__________________/____/_____________

|Jeff Sorvik (jps@msc.edu)      | "He is no fool who gives up that which    |

|Minnesota Supercomputer Center | he cannot keep for that which he cannot   |

|Telephone: (612) 626-1659      | lose."                                    |

|Fax      : (612) 624-6550      |                     -Jim Elliot           |

#---------------------------------------------------------------------------

End of article 4688 (of 4701)--what next? [npq] Article 4689 (12 more) in rec.woodworking:

From: dsr@mitre.org (Douglas S. Rand)

Subject: Re: Norm and the NYW needs some new spark ...

Message-ID: <DSR.91Sep26111022@chickpea.mitre.org>

Date: 26 Sep 91 16:10:22 GMT

References: <1991Sep25.134553.10156@Think.COM> <1991Sep25.213516.15498@amd.com>

Sender: news@linus.mitre.org (News Service)

Distribution: usa

Organization: The MITRE Corporation, Bedford, MA

Lines: 23

In-Reply-To: phil@brahms.amd.com's message of 25 Sep 91 21:35:16 GMT

Nntp-Posting-Host: chickpea.mitre.org

Phil writes:

I think using handplanes and such require time to develop the

technique.  Power tools are faster and easier to use and that's the

audience Norm is addressing. Judging from the response, I'd say Norm is

doing a valuable service. Maybe you'd like to see something else.  That

doesn't mean NYW is bad, just not your preference.

It's more then that.  I find hand jointing a board edge positively

tedious.  I've done it many times and I'm even reasonably good at it,

--MORE--(70%)Article 4690 (11 more) in rec.woodworking:

From: steveo@think.com (stephen anthony)

Subject: Norm and sparks

Message-ID: <1991Sep26.163055.13529@Think.COM>

Date: 26 Sep 91 16:30:55 GMT

Sender: news@Think.COM

Distribution: usa

Organization: Thinking Machines Corporation, Cambridge MA, USA

Lines: 17

Originator: steveo@olen.think.com

Ooops... I gave the wrong impression. I certainly didn't mean to bash Norm

personnaly. I love the show and have all the shows on video; which I pull

out and watch relatively frequently.

My comments were directed more at my feeling that the show needs to move on

a bit and test some new ground.

Sorry for any undue distress I may have caused.

BTW: I'm convinced that the prototype is the actual piece that gets

finished at the end of the show. I couldn't see making a prototype out of

cherry! Any corroboration on this?

--MORE--(92%)Article 4691 (10 more) in rec.woodworking:

From: chrisi@lloyd.Camex.COM (Chris Ischay)

Subject: Phil Ngai's information was Re: Motor power

Message-ID: <2394@camex.COM>

Date: 26 Sep 91 16:55:00 GMT

References: <1991Sep24.161355.23782@amd.com> <1991Sep25.200650.553@vlsi.waterloo .edu>

Sender: news@Camex.COM

Organization: Camex Inc., Boston MA

Lines: 49

In article <1991Sep25.200650.553@vlsi.waterloo.edu>

dgbedros@vlsi.waterloo.edu (David Bedrosian) writes:

>In article <1991Sep24.161355.23782@amd.com> phil@brahms.amd.com (Phil

Ngai) writes:

<<The laws of physics are fixed. Voltage times amperage = power.  If a so

<<called reputable company makes a claim that defies the laws of physics,

<<then the so called reputable companies are liars, plain and simple.

<<If you want to know how much power a (single phase) tool produces, just

<<take the rated amperage and multiply by the operating voltage. That's

--MORE--(38%)<<all there is to it. Period.

<<This posting is purely a personal opinion.

<Phil,

<

<You are incorrect about your calculations of power.  In actual fact ...

<       [different way to calcuate power]

<Next time you should check your "physics" before you make a statement as

<you have above that implies you are an authority on this subject!!

Normally, I would not respond to a followup like David's, since it's on a

topic in which I am not at all expert.  But this is the second example in

recent memory of Phil Ngai giving technical information that has resulted

in multiple followups containing corrections.  I'm also thinking of the

table saw guide.

So I'd like to ask publicly, and as politely as possible: Phil, what is

your response?  Your information is likely to be taken as gospel truth by

some reader, whether you intend it to be taken that way or not.  And if you

are in fact giving misinformation, then you seem to doing a disservice,

which I wouldn't expect to be your intention.  I'm not saying that you

--MORE--(76%)*are* giving misinformation, but I notice that others *do* say that.

You state in your .sig that the contents of your postings are purely

personal opinion, but a statement like "voltage x amperage = power" isn't

really an opinion.  In fact, you say it's a law of physics.

If you would be willing to post a reply, it would help me and maybe some

others figure out what you have in mind in some of your pronouncements.  I

don't at all mean to publicly embarrass you or anything like that, but you

put out these posts and then people write and yell at you about them.  What

gives?

Chris Ischay

chrisi@bos.camex.com

End of article 4691 (of 4701)--what next? [npq] Article 4692 (9 more) in rec.woodworking:

From: cpetterb@glacier.sim.es.com (Cary Petterborg)

Subject: Re: Norm and the NYW needs some new spark ...

Message-ID: <CPETTERB.91Sep26104450@mickey.glacier.sim.es.com>

Date: 26 Sep 91 17:44:50 GMT

References: <1991Sep25.134553.10156@Think.COM> <1991Sep25.213516.15498@amd.com>

Sender: news@javelin.sim.es.com

Distribution: usa

Organization: Evans & Sutherland Computer Corp.

Lines: 20

In-Reply-To: phil@brahms.amd.com's message of 25 Sep 91 21:35:16 GMT

Nntp-Posting-Host: mickey

In article <1991Sep25.213516.15498@amd.com> phil@brahms.amd.com (Phil Ngai) writ es:

>   steveo@think.com (stephen anthony) writes:

>   >that I don't have my fair share, but the pnuematic nailer is pushing the

>

>   Why does this bother so many people? Did Norm come to your shop and steal

>   your hammer?

Actually, I'd like to see him hit a nail with a hammer and have the project

--MORE--(71%)Article 4693 (8 more) in rec.woodworking:

From: smb@ulysses.att.com (Steven Bellovin)

Subject: Re: Norm and the NYW needs some new spark ...

Message-ID: <15542@ulysses.att.com>

Date: 26 Sep 91 17:37:32 GMT

References: <1991Sep25.134553.10156@Think.COM> <1991Sep25.213516.15498@amd.com>  <BDS.91Sep26083310@maui.linus.mitre.org>

Sender: netnews@ulysses.att.com

Distribution: usa

Lines: 21

In article <BDS.91Sep26083310@maui.linus.mitre.org>, bds@maui.linus.mitre.org (B arry D. Smith) writes:

>

> Actually, the Shakers made extensive use of power tools, even 150

> years ago.  A couple of years ago I asked this newsgroup how that was

> possible and a few people gave indepth discussions of shops with

> shafts and pulleys across the ceiling continuously driven by water or

> horses.  Belts would hang from the pulleys to table saws, lathes, etc.

> When tension was put on a specific belt, that tool would turn.  In

> actuality, Norm is not that far off from a true Shaker furniture shop.

Very true.  In fact, according to one of the FWW books, the circular

--MORE--(69%)Article 4694 (7 more) in rec.woodworking:

From: kkuehl@cbnewsf.cb.att.com (Kevin Kuehl)

Subject: Re: Norm

Message-ID: <1991Sep26.174624.28503@cbfsb.att.com>

Date: 26 Sep 91 17:46:24 GMT

References: <4646.28dda7f9@dcatla.uucp> <3033@moscom.UUCP> <920@dove.nist.gov>

Sender: news@cbfsb.att.com

Organization: AT&T Bell Laboratories

Lines: 15

In article <920@dove.nist.gov> wack@brutus.ncsl.nist.gov (John P. Wack) writes:

>One other question: if you had to have one plane in your shop, what type would

>it be? Jack, fore, smoothing?

Block.  Anything I can't do with a block plane I can have someone at the

hardwood lumber store take care of.  A smoothing plane might be just as

valuable.

Note I am saying ONLY one plane.  I would recommend having both a block and

a jack plane.

--

Kevin Kuehl

ihlpm!kkuehl@att.att.com

--MORE--(94%)Article 4695 (6 more) in rec.woodworking:

From: warren@cbnewsh.cb.att.com (warren.a.montgomery)

Subject: Using nails and screws

Keywords: screws, nails

Message-ID: <1991Sep26.175904.6018@cbnewsh.cb.att.com>

Date: 26 Sep 91 17:59:04 GMT

Organization: AT&T Bell Laboratories

Lines: 17

The ongoing discussion of NYW brings to mind one of my reactions

to the show, which I find entertaining in any case.  Norm seems to

put all of his projects together with screws and nails, and I

cringe every time I see him haul out that nailer or power

screwdriver, not because it''s an exotic power tool, but because

my woodworking training and reading taught suggested that screws

and nails are for rough carpentry and special situations, not for

holding together wooden furniture.  I'm not a snob about using

original methods and materials versus modern improvements, but I

always assumed that wood to wood joints held together better over

the years than wood to metal, and avoid screws and nails except

for special circumstances.  Am I way out of date?

--

--MORE--(96%)

Warren Montgomery

att!iexist!warren

End of article 4695 (of 4701)--what next? [npq] Article 4696 (5 more) in rec.woodworking:

From: c2k@spassky.mentorg.com (Carl Ketcham)

Subject: Re: Tuning a Bandsaw

Message-ID: <C2K.91Sep26114444@spassky.mentorg.com>

Date: 26 Sep 91 16:44:44 GMT

References: <4914@uc.msc.umn.edu>

Sender: news@news.mentorg.com (News User)

Organization: MGC

Lines: 9

In-Reply-To: jps@uh.msc.edu's message of 26 Sep 91 15:33:31 GMT

The book you want is by Mark Duginski - I bought a copy of the book, and, a

week later, a used (25 years old?) Delta bandsaw.  Cast-iron frame, etc..

The book has been a great help in getting it properly set up, and I am now

using it almost daily.  I still can't cut 1/64" veneer off of 6" thick

stock though.. %->

Cheers,

Carl Ketcham

carl_ketcham@mentorg.com

End of article 4696 (of 4701)--what next? [npq] Article 4697 (4 more) in rec.woodworking:

From: bcbeck@netcom.COM (Brian Beck)

Subject: Jet Tablesaws

Message-ID: <1991Oct01.014844.20569bcbeck@netcom.COM>

Date: 1 Oct 91 01:48:44 GMT

Organization: Netcom - Online Communication Services  (408 241-9760 guest)

Lines: 7

Does anyone have any experience with the Jet Table saw model JCS10?  Does anyone

have any comments on the quality of merchandise produced by this company

Thanks.

Brian Beck.

brian%cemax@sj.ate.slb.com

End of article 4697 (of 4701)--what next? [npq] Article 4698 (3 more) in rec.woodworking:

From: bennett@adobe.com (Bennett Leeds)

Subject: Respirator info requested

Message-ID: <21181@adobe.UUCP>

Date: 30 Sep 91 22:08:00 GMT

Sender: news@adobe.COM

Reply-To: bennett@adobe.com

Distribution: na

Lines: 19

I already have a dust mask that works well for airborne sawdust. However, it

does nothing for vapors or mists. I'm looking for a mask to wear when applying

paint, stain, lacquer, etc., and have some questions:

1. What are some examples of "organic vapors"? What are not?

2. Some household cleaners give off very strong odors - is there a mask/filter

that is appropriate for someone doing a lot of cleaning?

3. Many mask have interchangeable filters, usually one for organic vapors and

one for pesticides. Why is there a difference between the two? Is one a subset

--MORE--(74%)of the other?

4. I've noticed the new water based finishes still give off smell. If I'm using

them a lot (like for a large floor) should I wear a mask? Which one?

5. Any specific recommendations or avoidances are, of course, welcome.

Thanks in advance.

- Bennett Leeds

bennett@adobe.com

End of article 4698 (of 4701)--what next? [npq] Article 4699 (2 more) in rec.woodworking:

From: brenner_t@wums2.wustl.edu (Tim the news reader)

Subject: Cheap/free wood sources?

Message-ID: <1991Sep30.162317.4607@wums2.wustl.edu>

Date: 30 Sep 91 16:23:17 GMT

Organization: Washington University School of Medicine

Lines: 18

Where can I find used lumber from demoltion work or thrown out furniture?  I've

looked in my yellow pages but do not find anything for this "free" wood

source under junk, scrap, or wood sections.  Where do I look to get a

listing of local junkyards (I am in the St. Louis area if anyone has

suggestons for specific places)? Or do I simply search the neighborhood

dumpsters.  Any cheap source of wood is welcome.  I am more interested in

making things out of what I have available, rather than making specific

items made out of a specific type of wood.

Thanks for your help,

--

--MORE--(93%)

Bitnet: BRENNER_T@wums

Internet: BRENNER_T@wums2.wustl.edu

End of article 4699 (of 4701)--what next? [npq] Article 4700 (1 more) in rec.woodworking:

From: weverka@spot.Colorado.EDU (Robert T. Weverka)

Subject: Re: Respirator info requested

Message-ID: <1991Oct2.113116.14022@colorado.edu>

Date: 2 Oct 91 11:31:16 GMT

References: <21181@adobe.UUCP> <2299@vtserf.cc.vt.edu>

Sender: news@colorado.edu (The Daily Planet)

Distribution: na

Organization: University of Colorado, Boulder

Lines: 17

Nntp-Posting-Host: spot.colorado.edu

In article <2299@vtserf.cc.vt.edu> cohill@vtserf.cc.vt.edu (Andrew M. Cohill) wr ites:

>   ....  However there are

>different levels of toxicity;  you ought not to breathe turpentine fumes but

>they are not going to kill you as fast as methyl Parathion.  My guess is that

>some of the filters reflect this difference.  The filters we sold, which were

>certified by NIOSH and the Bureau of Mines, had activated charcoal in them and

>were good for eight hours of continuous exposure to organic vapors.  After that

>, you had to replace them, period.  Based on what I know, I'd be suspicious of

>anything that promised much more than that.

--MORE--(72%)>

>   ...     They will

>provide some protection against airborne mists and dust for a long time,

>but again, if you are exposing yourself to vapors, eight hours is the max.

^^^^^^^^^^^^^^^^^^^^^^

I use a 3M disposable mask.  Since the solvents I use all have a strong

odor, I had thought that I would smell them when the filter was no longer

effective.  Is this not the case?

End of article 4700 (of 4701)--what next? [npq] Article 4701 in rec.woodworking:

From: cohill@vtserf.cc.vt.edu (Andrew M. Cohill)

Subject: Re: Respirator info requested

Message-ID: <2403@vtserf.cc.vt.edu>

Date: 2 Oct 91 13:48:53 GMT

References: <21181@adobe.UUCP> <2299@vtserf.cc.vt.edu> <1991Oct2.113116.14022@co lorado.edu>

Distribution: na

Organization: Fire in the Mountain

Lines: 34

In article <1991Oct2.113116.14022@colorado.edu> weverka@spot.Colorado.EDU (Rober t T. Weverka) writes:

>In article <2299@vtserf.cc.vt.edu> cohill@vtserf.cc.vt.edu (Andrew M. Cohill) w rites:

>>

>>   ...     They will

>>provide some protection against airborne mists and dust for a long time,

>>but again, if you are exposing yourself to vapors, eight hours is the max.

>                                                    ^^^^^^^^^^^^^^^^^^^^^^

>  I use a 3M disposable mask.  Since the solvents I use all have a strong

>odor, I had thought that I would smell them when the filter was no longer

>effective.  Is this not the case?

--MORE--(46%)

If you are using cartridges rated for organic vapors, they have to be

labeled for exposure time.  The ones we sold were rated for eight hours;

I don't think you will find cartridges that last much longer than that

because most of them use activated charcoal;  there is only so much

charcoal you can get in a cartridge, and once the cartridge has absorbed

all it can, everything else is going to go right through.

Having said all that, remember that the eight hour rating is performed

under lab test conditions, and is probably a "worst case" situation

where there is a heavy concentration of fumes.  If you have

well-ventilated shop with lots of moving air, it may be that you can use

the cartridges for a longer period of time, because the concentration of

fumes is much lower.

But if you are screwing around with stuff like Xylene or toulene, why

try to save $1.25 and get an extra couple of hours out of the cartridge.

--

|          ...we have to look for routes of power our teachers never

|              imagined, or were encouraged to avoid.   T. Pynchon

|

--MORE--(94%)|Andrew Cohill        cohill@vtserf.cc.vt.edu            VPI&SU

End of article 4701 (of 4701)--what next? [npq]

End of newsgroup rec.woodworking.

******** 131 unread articles in sci.energy--read now? [ynq]

********  70 unread articles in talk.environment--read now? [ynq]