Article 98 (18 more) in sci.engr.mech:

From: volf@ais1.bal.mmc.com (John Volf)

Subject: Thermoelectric Summary

Message-ID: <1992Jul7.132840.237@iplmail.orl.mmc.com>

Date: 7 Jul 92 13:28:40 GMT

Sender: news@iplmail.orl.mmc.com (NetWork News Manager)

Organization: Martin Marietta Aero & Naval Systems

Lines: 83

First, I'd like to thank everyone who responded to my request for info

concerning Thermoelectric coolers.  The info I received ranged from technical

descriptions of TEC's to "You might want to look...", but all were very

helpful and greatly appreciated.  A summary:

Background:

----------

John Nagle provides a brief history...

Thermoelectric coolers (also known as Peltier-effect devices)

are widely used in small refrigerator applications, typically for boats

and recreational vehicles.  Real Goods, the alternative-energy equipment

house, sells such units and may be a source of engineering info.

--MORE--(31%)Thermoelectric coolers are still expensive, and are seldom seen for

any cooled space bigger than a cubic foot or so.

Small Peltier-effect coolers are used in specialized

electronics applications, such as cooling infrared detectors, solid-state

lasers, and occasionally a critical IC.  Using pyramidial stacks of

devices, cyrogenic temperatures can be reached for small devices,

allowing the use of small superconducting devices without a supply of

cyrogenic liquids.

Selection Example:

-----------------

A helpful Brit submitted the following...

Peltier heat pumps

Watts    Max temp difference      size (l,w,h)mm    cost (pounds sterling)

0.32           67 C                4.2*4.2*2.7             #13

18.8           70 C                 30*30*5.6              #40

68.8           65 C                 40*40*3.3              #50

You derate poser with temp difference so if you want 35 C difference and 9

watts then the 2nd one will suit you fine.

--MORE--(60%)

Prices are for 10 off quantity, and will be higher than you would get from

looking around.

These run with a maximum voltage of 0.5, 3.75 and 15.4 volts respectively.

Because the voltage is a function primarily of temperature difference they are

ideal for self regulating coolers.

As you can see the largest is well suited for use in a car fridge, but is a

bit expensive. I have seen a fridge probably using this device for sale at

about 70 pounds.

Thermoeletric Materials:

-----------------------

NASA Tech Brief Vol 13, No. 7, Item #103 "Research in Thermoeletric Materials"

Companies That Produce TEC's:

----------------------------

TECA (Thermo Electric Cooling America Corp.).

address unknown (probably Chicago)

--MORE--(82%)(312) 342-4900.

Borg Warner Thermoelectrics

3570 North Avondale Ave

Chicago, IL 60618

(312) 588-5120

Real Goods

address unknown

Plus quite a few more are listed in the Thomas Register.

I have received a catalog from Marlow Industries (address in Thomas Register)

which provide a good deal of help in determining your heat transfer

requirments and selecting an appropriate TEC.  A price list is also included,

which range from about $15 to $150.  Installation instuctions are even

included.

********************************************************************************

Thanks again,

--MORE--(99%)John

End of article 98 (of 106)--what next? [npq]