(240)   Sat 12 Feb 94 11:15

By: Don Kulha

To: All

Re: LCBs, 1 of 3

St:

------------------------------------------------------------------------

@MSGID: 1:125/7.0 2d5d0092

Things that Work!

A WATER PUMPING SYSTEM USING AN EXPERIMENTAL LCB+

CONFIGURATION

Gary Waldsmith

Building a home is at best described as a traumatic experience.  Building

a home yourself, and trying to achieve 100% energy independence has

been called an insane undertaking by my most trusted and closest friends.

These same people cave in every month and pay the pound of flesh to

the local utility companies, so what do they know.

But now, 8 months later, I have a 90% finished home, 16 @ L-16 batteries,

an inverter, a 2500 watt Jacobs wind generator, a custom Sun Selector

NDR type charge controller, a fully autonomous water system, and a supply

of ARCO M-75 panels.

Throughout this undertaking, there have been many interesting engineering

adventures and wonderful discoveries.  These rewarding experiences should

be shared, and what better way than the "Things That Work" section in

"Home Power".

The LCB+ or Linear Current Booster, a product of West Virginia based

Sun Selector?, has been one of the more pleasant discoveries of my adventure.

This report should help explain one way I have found of utilizing the

product and the benefits thereof.

I tested the LCB+ in an unusual configuration, that at the time of the

test had not been tried in the field, although a technical note from

Sun Selector had indicated that the configuration was possible.

The test configuration involved the arrangement of the LCB's in a series

parallel combination that would allow the system as a whole to operate

at my desired 90 VDC @ 6 amps.  The normal LCB model 3-4-8 is designed

to operate at a maximum of 42 VDC and 3 amps.  With this disparity in

product design limits and system requirements, I decided to try the

series parallel configuration that Sun Selector had suggested.

I was of course reluctant to subject the LCB's to the test, being uncomfortab

le

risking my investment in the product.  A call to the Sun Selector toll

free tech. support number (1-800-222-3988) found a company eager to

extend the warranty to cover the configuration.  At this point I became

involved with Mr. Joseph Bobier.  He spent time explaining the properties

of the product and provided details and precautionary information for

a proper installation and adjustment of the LCB's.  The actual electrical

connections are depicted in chart #1

--- Squish v1.01

* Origin: Sonoma Online, HOMEPOWR Echo (707)545-0746-HST\V32b (1:125/7)

@PATH: 125/7 203/23 1 396/1 3615/50 3641/1

------------------------------------------------------------------------

(241)   Sat 12 Feb 94 11:15

By: Don Kulha

To: All

Re: LCBs, 2 of 3

St:

------------------------------------------------------------------------

@MSGID: 1:125/7.0 2d5d00ae

A very nice feature of the LCB in this configuration is that the control

system becomes extremely reliable.  This is because the power processing

is spread out over four independent devices, thereby exponentially reducing

the likelihood of a cataclysmic failure.  In this case, if the MTBF

(Mean Time Between Failure) is defined as X, the failure of the entire

system would be calculated as X4.  If one unit does fail, it will automatical

ly

bypass itself and allow the remaining units to function.  The same feature

works when part of the PV array is shaded, eliminating all stress on

the other units, and the PV array.

Pump Mechanism:

The pump is a Churchill double gear reduction walking beam type developing

a maximum of 600 ft. lbs. of torque. To develop full output of this

equipment, it was originally powered by a 4x4 matrix of 16 panels to

achieve 128 VDC @ 6 amps.  Recall from chart 2 that I now use only 12

panels to achieve optimum performance and the maximum 1750 RPM's to

drive the motor with 318.3 watts of power. NOTE: The original pump configurat

ion

required 768 watts (128 VDC X 6.0 amps) of power to achieve the same

performance.

The pump cylinder has a 2 inch bore and an 11.5 inch stroke.  Using

the formula V=+R2S we can find a volume (V) of 36.13 cubic inches when

the radius (R) equals 1 and the Stroke (S) = 11.5.  36.13 cubic inches

of water equals 0.1564 gallons.  The motor revolves 129.62 times for

every stroke, so knowing the motor RPM's, I can calculate the water

production rate from my 460 feet deep well.

Reviewing chart#2 you can see that the motor performance was decidedly

linear under ALL levels of sunlight.  This is the essence of the LCB

contribution.  Without the LCB, the motor performance would quickly

deteriorate in an exponential curve as the sunlight decreased.  The

LCB allowed FULL utilization of the available power throughout the entire

day from dawn until dusk.

INSERTCHART2&CHART3

This chart shows the ability of the LCB's to keep the system pumping

under even very low levels of light.  The bold row of data represents

an approximately 25% insolation level.  Notice that even at this low

light level you could take a bath every hour. At even a 3% power level

the pump is still running.

Of particular interest is the linearity of the power delivery curve

of chart #3 (straight line).  The LCB's are clearly doing a great job

of maintaining a high level of internal efficiency and minimal throughput

loss throughout the entire power level range.

--- Squish v1.01

* Origin: Sonoma Online, HOMEPOWR Echo (707)545-0746-HST\V32b (1:125/7)

@PATH: 125/7 203/23 1 396/1 3615/50 3641/1

------------------------------------------------------------------------

(242)   Sat 12 Feb 94 11:17

By: Don Kulha

To: All

Re: LCBs 3 of 3

St:

------------------------------------------------------------------------

@MSGID: 1:125/7.0 2d5d0130

LCB MOUNTING & ELECTRICAL WIRING

The entire electrical system is wired with #10 stranded wire utilizing

crimp terminals and 40 feet of wire.  (See chart #1) The LCB's are small

metal cans about 2" X 2" X 11/4".  The units are mounted within 2 feet

of the PV modules.

I mounted all four of the LCB's on a 12" long aluminum heat sink with

thermally conductive epoxy cement to help with heat dissipation.  This

elaborate mounting proved to be un-necessary as the LCB's never generated

a large amount of heat, even at the lower motor voltages when the conversion

ratio of current input to current output was the most demanding.

Although the LCB's are running at Sun Selectors' maximum specified limit,

there is no significant heating evident, even with last summer's 105

degree peak daytime temperatures, a good sign of high efficiency and

built in reserve power handling capability.

Cost vs. Benefit:

The LCB's are retailing in the $50.00 price range.  I've used four so

they would cost about $200.00 to the end user.  I used them to replace

four PV panels that cost about $250.00 each wholesale.  My direct savings

is about $800.00.  But we really aren't comparing apples to apples.

My system now runs better with fewer panels.  To get the same performance

without the LCB's I would need more than the extra four panels, and

since I would be dealing in groups of four, (series / parallel), I would

have to buy extra panels four at a time.  If I REALLY wanted to maximize

the system efficiency, I could; shorten wire run's, solder all of the

crimp connectors, fine tune the LCB's and remove instrumentation (meters,

etc.). I could run the pump on only 318 peak watts (see chart #3).

This would just about be achievable with only six PV modules.

Sun Selector makes LCB's in several sizes for every possible application,

and with the now proven ability to cascade units in series / parallel

to achieve any desired voltage and current level, no pumping system

should be engineered without them.

Access:  Gary R. Waldsmith, 1441 Hound Hollow Rd., Pilot Hill, CA 95664

Gary Waldsmith is both a physicist, and an electrical engineer working

for the Dept. of Defense in the Sacramento area.  He has been building

electrical / electronics projects and a ham radio operator most of his

life.  He would be glad to help anyone with similar projects or "adventures"

in the works.  An SASE is welcomed anytime.  With prior arrangements,

Mr. Waldsmith will show his equipment to interested visitors.

LCB is a trademark of, and Sun Selector is a registered trademark of

Bobier Electronics Inc.

Article courtesy of and copyrighted by Home Power Magazine

Published bi-monthly/$15 yr/POB 520, Ashland OR 97520

--- Squish v1.01

* Origin: Sonoma Online, HOMEPOWR Echo (707)545-0746-HST\V32b (1:125/7)

@PATH: 125/7 203/23 1 396/1 3615/50 3641/1