/* Written  6:33 pm  Aug  3, 1989 by tgray in cdp:en.energy */

/* ---------- "Energy Resources Report" ---------- */

Subject: Energy Resources Report

The U.S. Department of Energy has recently issued a very interesting

report which compares the size of the various domestic energy resources

available to the U.S.

The report is entitled "Characterization of U.S. Energy Resources and

Reserves," prepared by Meridian Corp., June 1989, under Contract

#DE-AC01-86CE30844 (the only identifying number).

I recommend it as a useful reference book, especially for those of you

who are interested in learning more about the potential of renewable

energy sources and technologies.

The report assesses U.S. resources of fossil, uranium and renewable

energy according to the U.S. Geological Survey (USGS) classification

system.  On p. 5, it notes that "Application of this approach to

the estimation of energy flow-based sources (such as solar and wind

energy) has not been successfully undertaken previously."

A quick overview of the report's contents are as follows:

First, it quantifies the U.S. _Total Resource Base_, defined as

"A concentration of naturally solid, liquid, or gaseous materials

in or on the Earth's crust in such form that economic extraction

of a commodity is currently or potentially feasible." (p. 6)  (It

goes on to note that this definition, developed for mineral resources,

applies equally well to other energy sources except for solar

radiation.)

The Total Resource Base is found to be 657,596 billion barrels of

oil equivalent (BBOE).  Current U.S. energy use is about 14 BBOE/year,

so the total resource base could theoretically provide 46,900 years of

consumption. (p. 19)

The Total Resource Base is distributed among energy sources as follows:

(p. 19)

Energy Source                  BBOE

Geothermal                       258,263

Photoconversion

(Solar and Biomass)            178,438

Wind                             176,910

Shale Oil                         27,518

Coal                              15,079

Petroleum                            477

Natural Gas                          294

Peat                                 244

Uranium                              203

Hydropower                           170

Total                         657,956

Next, the report quantifies the Accessible Resource Base, which is

defined as "The resource that can be located, mined or extracted

by technology currently available or that which will be available in

the very near future (3 to 5 years)."  (p. 9)

The Accessible Resource Base is found to be 115,103 BBOE, still enough

for 8,200 years of consumption at 1987 rates of usage. (p. 25).  It

is distributed among technologies as follows: (p. 25)

Energy Source                   BBOE

Photoconversion                  101,153

Coal                               6,577

Geothermal                         3,928

Shale Oil                          2,018

Wind                                 870

Petroleum                            190

Natural Gas                          153

Uranium                              126

Peat                                  61

Hydropower                            27

Total                         115,103

Next, the report quantifies the U.S. energy Reserve, which is defined

as "That resource which is identified and which can be economically

and legally extracted with existing technology and under present

economic conditions to yield useful energy or an energy commodity."

(p. 9)

The Reserve is found to be 1,096 BBOE, enough for over 78 years of

consumption at 1987 rates.  It is distributed among technologies as

follows: (p. 30)

Energy Source                BBOE

Coal                                 908

Photoconversion (biomass)             58

Geothermal                            43

Natural Gas                           40

Petroleum                             27

Hydropower                            10

Uranium                                7

Photoconversion (solar)                3

Wind                                  <1

Shale Oil                             <1

Peat                                  <1

Total                        1,096

The report notes that "Approximately 99.6% of the U.S. _reserve_ is

comprised of energy sources that are currently in use but that

constitute only 16.6% of U.S. total _accessible resources_.  This

means that there is a large amount of available energy sources

currently in use in the U.S. that cannot yet be economically tapped

as an energy supply due to current extraction or conversion efficiencies

or to technology readiness."  (p. 30)

***************************************************************

Tom Gray

American Wind Energy Association {standard disclaimer applies}

{hplabs,arisia,labrea,pyramid}!cdp!tgray

also... cdp!tgray@labrea.stanford.edu.BITNET cdp!tgray@uunet.uu.net

"Wind energy is clean energy."

####################################################################################################################