CONTENTS (Terminals Covered)

Wyse99GT

This is documentation for downloading font to the VT200 series of

terminals and their clones.  Be sure to read the README file which contains

general information and describes the input file format of characters.  The

document briefly discusses the VT200 (or VT220) character format and then

has a section for each model of terminal.  It you use a terminal not listed,

please document its features and add it to this document.

Some terminals which have a VT200+ emulation can't download softfont.

You should first make sure (by checking the manual if possible) that

your terminal can do this.

The command line options for the font generator GenFontVT200 are:

GenFontVT200  [-p 80]   -s | input_file

-p 80 => append 80 (or whatever) padding nulls to each line of output

-s => use standard input.

The generated softfont goes to std output.

The softfont is loaded sequentially into character positions of increasing

order starting at a specified character position such as 33 to start off

with a redefinition of "A" ("A" is the 33 rd printable character assuming

that <space> is the 0th printable character.)  The start location (such as

33 above) is just one of a number of items you must send as a header before

the softfont.  An example of a header is:

<ESC>P0;33;1;4{T		where:

<ESC>P is the ascii escape character followed by P.  The DCS 8-bit control

character (=90 hex) may sometimes be used instead of <ESC>P.

0 => Load "printable" character bank (above control characters).  If

it is 2 => graphic character bank (32 characters from 5F thru 7E)

33 => First character to load is # 33 (= "A") as explained above.

1 => Don't erase old characters which may exist in this bank.  0 => erase.

4 => Matrix size is 7x10.  See your manual.  May not be required.

T => Font will be named T. Be sure it doesn't conflict with an existing

name such as "B" for U.S. Ascii on a Wyse99GT terminal.

Some VT200's take two more parameters before the { symbol, but I don't

know what they mean.

*****Wyse99GT*****

See the Wyse Programmer's Guide p.8-6 to 8-10 for definition of the font.

See p. 7-4 to 7-5 for how to assign the font bank to the crt (not done in

this program).  If you don't have the Programmer's Guide, the User's Guide

has terse information in Appendix H under "Controlling Character Sets".

However I found that a "matrix size" of 7x12 is possible by neglecting to

specify the "matrix size" in the header.  This User's Guide mentions a

"middle code" (mc) but no such code can be found in the Programmer's Guide.

I never used a mc and the font worked fine. The Programmer's Guide only

shows a upper code (uc) and a lower code (lc) which corresponds to the

enumeration values upper_half and lower_half in the source code for the font

generator.  Here is a sample shell script:

# This will download Russian (Cyrillic) font (ISO-5) to a Wyse99 terminal

# and put it into 8-bit VT220 mode.  It is only for the Wyse terminals

# at UCI (University of California at Irvine) which are set up to emulate

# VT100 and are connected to the external world via their Aux Port.

# Downloading font at 9600 baud requires either Xon/Xoff flow control or

# the use of padding nulls.  This script opted to use Xon/Xoff.

# The next line defines ESC as Hex 1B, but most printers will not print it.

set ESC=#

echo "Loading 8-bit GOSTCII Cyrillic font.  Wait!  CRT will blank, flash etc."

# Sleep to have time to read echo shown above.

sleep 1

# Go to native personality since can't enable Xon/Xoff from ANSI personalities.

echo -n "${ESC}[42h"

# Set Xon/Xoff flow control for Aux port (connected to terminal server).

echo -n "${ESC}c31"

# Put message at top of CRT in status line

echo -n "${ESC}FGOSTCII Cyrillic Font"

# Change to VT220 personality and 8-bit transmission mode on.

echo -n "${ESC}~=${ESC} G"

# (Would be useful to make Funct key meta but how?)

# Send initialization sequences to the Wyse 99GT terminal per Programmers

# guide pp. 8-10 to 8-14.  Note that ! must be quoted with \ inside "".

echo -n "${ESC}P0;0;0;{\!\!C"

# Send the font named !!C (above) from the file fontv to the terminal.

cat fontv

# fontv file contains trailer for Wyse99GT: ESC \

# assign !!C font to bank  G2.

# Then assign bank G2 to GR (the rt half display bank).  See Wyse

# Programmers guide pp. 7-6 to 7-8.  Note ! quoted by \.

echo -n "${ESC}*\!\!C${ESC}}"

# Enable 8 bits chars to pass 8-bit bytes to host

stty -istrip cs8

echo "Loading of Cyrillic font (IS0 5, 8-bit) completed."