Heavy Duty Launch Pad

by Steve Lubecki

Anyone getting into high power rocketry needs a launuch pad capable of holding heavy rockets without tipping over and deflecting composite motor exhaust without burning through.  This pad will fit the bill and can be built to suit individual needs (or can be built by myself to fit your particular needs at a reasonable cost).  You will need a welder, a drill press or hand drill, a tap & die set, a cutoff saw or hacksaw, and various clamps.The first step is to drill a 5/8" hole in the blast deflector (A) and backing plate (B).  Then set the blast deflector aside.

Next, take the launch rod tube (C) and drill a 7/16" hole through both sides at one ends.  Center the jamb nut (F) over one of the holes and weld it into place.  This nut will hold the eye bolt (E) that tightens down against 5/8" or smaller launch rods.  Now weld this tube to one side of the backing plate (B), centering it over the 5/8" hole.  Take the deflector pivot (D) and drill a 7/16" hole centered in one end and weld the the other end to the backing plate (B) approximately 3/8" to 1/2" away from the the launch rod tube (C).  Set the entire assembly aside.

Here comes the fun part.  (A drill press makes this step easy, or have patience trying it without one.)  Take the main mounting block (H) and drill a 7/16" hole through it at one end centered on the 1" side of the block.  Next, drill a 3/8" hole perpendicular to the first and centered on the 1" x 1-1/2" side of the mounting block (H).  Tap this hole using a 7/16" x 14 tap.  Next, weld jamb nut (G) to one side of the 7/16" hole.

Now for the handles.  Take bolt and strap (I) and weld them together as illustrated, and do the same with nut and strap (K).

On to the legs.  Both legs (M and N) are 24" long (or longer to suit your needs).  Cut all four ends at 15 degrees.  Then take two pieces (R) 2" long and two pieces (S) 3" long and cut one end of each to 15 degrees.  Weld the pieces (S) to one 24" leg (M) and the pieces (R) to the other leg (N) with the 15 degree ends touching each other (see illustration).  Drill a 7/16" hole in the center of each leg (M and N) so

that when assembled the legs can be folded together.  Run

the pivot bolt (P) though nut (O), leg (N), and leg (M).

Make sure that the leg with the 3" ends (M) is on top.  Then

place tube (L) over the pivot bolt (P) and thread the handle

(K) over the pivot bolt (P).  Tighten handle (K) to hold the

legs rigid.

To assemble the pad, thread jamb nut (J) loosly onto the

pivot bolt (P) and then thread the mounting block (H) onto

the pivot bolt (P).  Run the jamb nut (J) up against the

mounting block (H) and tighten with a wrench as tight as

possible.  Run the handle bolt (I) through the hole in the

deflector pivot (D) and then through the mounting block (H)

and tighten it into the jamb nut (G).  Use washers between

the deflector pivot (D) and the handle (I) so that when it

is tight the handle won't interfere with the blast

deflector.  Next, center the blast deflector (A) over the

5/8" hole in the backing plate (B) and weld it into place.

Install the rubber feet and the launch rod and you are ready

to go.

This pad folds up flat 4" high and stores easily.  It can

also be scaled up or down to suit your needs.  The pivot

bolt (P) and tube (L) can be longer to make the pad as tall

as you like.  The legs can also be made longer to support

those HONKIN' HUGE rockets.  By loosening the handle (I) the

pad can be tilted to any angle or to 90 degrees to fold

flat.  By loosening handle (K) the deflector can be rotated

360 degrees to adjust for wind, etc.

µ §