The Perpetrator

Appropriate name for a model by Bob Hegwood, don'tcha think?

This design came about of necessity.  After using almost all of the LOC/Precision parts in my closet for other designs, I found myself wondering what kind of rocket could be built using the few parts that were left.  Since I just HATE to waste good LOC parts, and since I just LOVE a challenge, I started epoxying the parts into some semblance of a rocket.  During the assembly process, I took some kidding from certain unnamed MIVARS members.  It seems that the Ward Cleaver guy thought my rocket was going to look like a popsicle when it was finished.  [As I recall, it was that Eddie Haskel guy that first brought up the image of the artificially colored and flavored frozen confectionery. -ed.]  That Total Impulse guy even offered to give me some REAL high-power rocket parts so's I could build a REAL rocket.  Sheesh!  All I can say is, never underestimate the creative abilities of a die-hard rocket freak.  The rocket DOES look a little strange, but it flies like a dream, and not one of those spare LOC parts was wasted.  Now that the Perpetrator is finished, and now that those MIVARS guys have seen it fly a few times, the editor of this rag suddenly wants plans!  Not only that, but he wants the plans by tommorow!  Jeez!  Give me a break willya?  If you want to add a fairly cheap, fairly weird-looking, almost high-powered rocket to your fleet, read on.  (After all, I had to stay up all night typing this $%#&"! article didn't I?)

## The Parts: (LOC-Precision part numbers)

A)   1	PNC-3.90 nose cone

B)    1	17.5" BT-3.90 body tube

C)    1	23" MMT-1.14 motor mount tube

D)    1	LL-50 launch lug

E)    2	CR-3.90-1.14 centering rings

F)    3	1/8" plywood fins (see main drawing)

G)    1	SC-1000 15 ft 1" shock cord

H)    1	SCM-3 shock cord mount

I)     1	LHPC-35 35" parachute

## The FUN Part:

This thing is LOTS of fun to fly.  If you're into reloads, stick an H180W with a short delay into its wazoo and watch it roar.  The delay must be fairly short (6 seconds) because the model is pretty heavy with the fin design and epoxy construction.  When this thing flew at Danville (AFTER I tried to burn the hotel down), a couple of people told me that it flies one heckuva lot better than it looks.  [Kinda like a certain SNUB 487 design, 'ey Bob?  -ed.]  I have also flown this thing with an Aerotech H140-6 disposable motor.  Again, the flight profile is pleasing to the eye and somewhat surprising.  By the way, Paul Horvath of Propulsion Industries has just introduced a 29mm I motor.  I can't WAIT to try one of THOSE things in my popsicle rocket.  Take care.  -Bob

µ §

## The Effort: (You didn't think I was gonna BUILD it for ya?)

Step 1:  It's gonna look crazy, I know, but epoxy the centering rings to the motor mount tube as shown in the drawing.  When you do this, make sure that you get the centering rings on straight.  The rings must be

perpendicular to the 29mm motor tube before your epoxy cures.

Step 2:  Epoxy the assembly from one into either end of the body tube.  Leave about 1/16 of an inch between the popsicle rocket, place the nosecone in the other end of the body tube and take a look at the assembly. <<grin>>  [Although without the nosecone, it looks like a Push-up. -ed.]

Step 3:  Fill the 1/16" space between the body tube and the centering ring with epoxy.  Smooth even with the end of the body tube.  HELPFUL HINT: Use GMP's 10 minute epoxy to fill and reinforce the centering ring end.  This stuff fills easily, and it provides a good strong base for the next step.

µ §

Step 4:  Epoxy the fins onto the assembly as shown in the drawing.  Again, take the time to make sure that you get the fins on straight.  When the epoxy is cured, fillet all joints for strength.

Step 5:  Cover the motor mount tube with epoxy between all fins.  Use enough  to fill the fin fillets.  It may sound like overkill, but this step is necessary in order to keep the fins on the rocket when it lands.  It will also make for a very strong motor mount tube.  NOTE: If you're more ambitious, you may want to cut, sand, seal and epoxy some plywood over the exposed 29mm motor mount tube.  I didn't, but then again, I'm lazy.  Besides, this is just a weird-looking POPSICLE rocket!

µ §

Step 6:  Epoxy the launch lug to the body tube as shown in the drawing, and attach your parachute, nosecone and shock cord.  My version of the Perpetrator is painted dark blue and features a gloss-white nosecone.  White pinstriping, stars and lettering completes the finish.

µ §