Black Powder Express

by Steve Lubecki

[Editor's note:  This design requires special care to fly.  Experience with clustering is a must.  As the model exceeds the model rocket limits, flying it will require an FAA waiver.]

## Parts List

5	34" MMT-0.95	LOC

1	12" x 12" x 1/8" aircraft plywood 	Sig, Midwest

44" x 1/4" dia dowels	Sig, Midwest

1	34" BT-3.90	LOC

1	10" BT-3.90	LOC

1	 BA-3.90	LOC

1	 PNC-3.90	LOC

1	 SCM-2	LOC

1	 LL-50	LOC

1	 CR-3.90-1.14	LOC

1	 1.9" OD x 1.5" ID motor tube	Z-Welding

## Assembly Instructions

First take the 5 34" MMT-0.95 and cut them exactly in half.  Then, get the 1.9" OD motor tube [Editor's note:  You can also use a LOC MMT-1.52 and shim it up to 1.9" OD using packing tape.] and test fit it and nine of the MMT-0.95 pieces into one end of the 34" BT-3.90.  Make sure that all tube ends are even and that all tubes are pushed in about 5" into the BT-3.90.  Wrap rubber bands around the exposed tubes and then, using thick cyano and zip kicker, glue the outer tubes to themselves, being careful not to bond them to the BT-3.90.  When the cyano has hardened, remove the assembly and slip the center motor tube out of the other nine.  Mix up some epoxy and fillet two valleys at a time on the inside of the motor tube assembly only.  Once all the valleys are filled, epoxy the main motor tube into the center.  The voids between the MMT-0.95's and the main motor tube should be full of epoxy at this point.

Next, apply a liberal amount of epoxy into the 34" BT-3.90 about 3"-4" into one end and spread it evenly around the inside of the tube.  Slide the CR-3.90-1.14 into the tube and, using the motor tube assembly, push the centering ring in 5".  Hold the tube upright and rotate it to allow the epoxy to form a fillet around the top of the centering ring.  When the epoxy has set, turn the assembly upside down and pour epoxy into the voids between the BT-3.90 and the MMT-0.95 to a depth of about 1/4".  When the epoxy has set, pack tissue into the voids until about 1/2" is left at the top of each void.  Fill the remainder of the voids with epoxy.Cut the fins from the plywood sheet.  I used a LOC Ultimate fin pattern and made three fins.  Epoxy the fins into place 1/2" up from the bottom of the motor tubes.  When the epoxy has set, fillet each fin liberally with epoxy, one at a time.

Next, drill 4 1/4" diameter holes about 15" down from the top of the tube (2 holes opposite each other and two at 90 degrees and 1/4" up).  Install 4" x 1/4" dowels into these holes and cyano into place.  These are to keep the parachute and wadding from falling too far into the tube on launch.  (A stuffer tube will also work but I've seen too many collapse or burn through and cause ejection problems.)  Assemble the bulkhead assembly per LOC instructions and then epoxy it 3" into the 10" BT-3.90.  Install the shock cord mount per the LOC instructions.  Cut the launch lug in it on the main tube.

µ §

## Now For The FUN Part

Prepping this rocket for flight is fun.  First take 9 D12-0's and fill the ends with epoxy.  [Editor's note:  Not the nozzle end, Maurice!]  Wrap tape blocks around the nozzle ends of the D12-0's and the Rocketflite G100-7 and friction fit them into the motor tubes.  Next, cut slow thermalite into 10 equal pieces about 3" long.  Then, using nine of the pieces, fold one end of each piece over about 1/4" and take the folded end of each piece and insert it into a D12-0 nozzle.  Pack tissue into the nozzle to keep it from falling out.  With the tenth piece of thermalite, fold one end over about three times and put it into the nozzle of the G100-7 exactly the same length as the Rocketflite electric ignitor.  Tie the free ends of the nine pieces of thermalite installed in the D motors together in a bundle using another piece of thermalite.  The main motor thermalite should be resting above the wrapped bundle.To launch the rocket, use a flashbulb or a thermalite ignitor to electrically ignite the main bundle.  As it burns up it will ignite the G100-7 thermalite.  The rocket lifts off the pad on the 9 D12-0's and about 100'-200' up the G100-7 ignites.  The D's move the rocket up real slow and the G kicks it up in a hurry.  A seven second delay is perfect if everything goes right.  Use a 48" chute in this rocket.  9 D's and a G are great for average-sized flying How about 9 G42's and 1 H115 Smoky Sam?  Look out, Danville!  [But gee, Steve, then it would have to be a Composite Express!  ed.]