Warrior: A Two-Stage Cluster Design

by Thomas P. Secrist

Some of you will remember a design called the Little Beth X-2 that showed up in an issue of Model Rocket News in the early 70's.  The design consisted of a three-engine booster and single engine upper stage.  The Warrior goes one step further by adding an engine to the booster.

.µ §

## Upper Stage Assembly

Assemble the EM-2050 mount as per the instructions.  Glue the engine mount into the main tube so that the ends of the engine and main tube are even.  Cut three upper stage fins from 1/8" balsa. (Make certain that the balsa grain is parallel with the leading edge.)  Mark and draw the fin lines on the main tube and glue the fins onto the lines.  Make certain the back of the root is even with the end of the tube.  Glue the launch lug at one of the fin/tube joints.  Give all fin joints a good fillet.  Glue the screw eye into the nose block, then glue the nose block halfway into the payload tube.  Using a typical Estes type paper anchor, glue the shock cord about halfway down the main tube, then tie the free end to the screw eye.  Assemble the 12" parachute and tie it to the screw eye.  Put a finish on the upper stage as you would any model.  The upper stage can be flow single-stage with a B4-4,  B6-4 or  C6-5.

## Booster Assembly

Assemble the second EM-2050 mount but leave out the engine block. Glue the engine mount into the BT-50. The tube ends should be even.  Lay the BT-50 assembly on a flat surface.  Lay one of the BT-20

outboard tubes beside the BT-50 with two ends even, then glue the tubes together.  Allow the assembly to dry.Make a copy of the tube alignment guide and place the copy with a wax paper cover on a flat surface. Place the assembly on the guide, then glue the remaining outboard tubes to the main body.  Put heavy fillets in the tube joints, using white glue or epoxy.  Once the glue has dried, mark the fin placements on the boosters, then draw the lines as straight as possible.  Glue engine blocks into the outboard tubes so that the engines will extend about 1/4".  Cut three booster fins from 1/8" balsa (Again, make certain that the balsa grain is parallel with the leading edge.)  and glue each of them to the fin lines with the rear of the root even with the back of the tube.

## Flying

Experience with clustering is a must for the Warrior.  Any upper stage engine can be used in the second stage.  Lower stage combinations are limited as follows: the thrust duration of the central booster must be equal to or greater than the outboards.  And of course,  all the outboards must be identical.  The following booster combinations are possible:

Central	Outboard

B6-0	1/2A6-2

B6-0	A8-3

B6-0	B6-2

C6-0	A8-3

C6-0	B4-2

C6-0	B6-2

C6-0	C6-3

Prep the upper stage and outboard recovery systems as you would any model. Place the central and upper stage engines together with the booster front against the upper stage engine nozzle, and tape the engines together with only one wrap of scotch tape. (Not masking, electrical or duct tape!) Friction fit the upper stage engine into the upper stage.  Now, slide the lower stage on from the other engine end.  Some friction fit may be necessary to hold in the booster.  The outboard tubes serve as a tower for the upper stage, so no coupler is needed.  Friction fit the outboards in place.  Carefully select your igniters.  Add 3" leads of 30 ga wire to make hookup easier.  Insert the igniters in the engines, then connect four leads (one from each engine) to one clip.  Connect the remaining four leads to the other clip.  Use a strong 12 volt system.

µ §