=================================================================

NTSHAPE V1.10:

Nosecone / Transition Section Shape Generator Software

=================================================================

Copyright (c) 1993, 1994, Gary A. Crowell Sr.

Permission is granted for unrestricted

use and distribution of the program

NTSHAPE.EXE Version 1.10

for non-commercial applications.

=================================================================

INTRODUCTION TO NTSHAPE

=================================================================

The original program NCSHAPE, was developed out of frustration at

the *BORING* nosecone shapes that are the only type available

from manufacturers at this time.  If you want anything other than

a tangent ogive, you have to do it yourself or you're out of

luck.  It's particularly frustrating because there are several

interesting shapes that are reportedly more efficient than the

tangent ogive in the transonic regions.  The simple reason you

don't see them being used is that they have to be generated by

some moderately complex equations, while a tangent ogive curve

can be drawn with a piece of string.  NCSHAPE was updated to

NTSHAPE when the ability to draw transition sections was added.

NTSHAPE will do a whiz-bang job of displaying a variety of

nosecone and transition section shapes on the screen, but its

most important feature is the ability to generate a list of

radii and diameters at specified station points.  This list can

then be used to provide reference points for turning a nosecone,

drawing a template, or machining a mold.  In the case of a

conical transition section, you can also get dimensions for

cutting a shroud.

HARDWARE REQUIREMENTS:

======================

NTSHAPE requires a DOS/IBM'ish computer with VGA display.  It'll

work fine on a '286 or better, probably fine on anything smaller

too.  NTSHAPE is compiled to take advantage of a numeric co-

processor if available; that makes some of the shape draws

faster.

GENERAL DATA ENTRY:

===================

The data entry user interface is the ugly, Ugly, "choose a

letter ..." variety.  But hey, it works; maybe I'll do something

better next time.  Selections that are required are displayed in

red, those not available are in grey, everything else in white.

Anything except the greyed functions can be selected anytime, in

any order.  There is no rodent infestation.

When you select a function that requires data entry, the entry

area will be highlighted.  Numeric fields will not take alpha

characters or negative numbers.  Editing on a field is limited to

backspacing only.  To complete an entry, either <CR> or use the

<UP ARROW> or <DOWN ARROW> cursor keys to jump directly to the

next entry field.

=================================================================

NTSHAPE  V1.10  page 1               (c) 1994 Gary A Crowell, Sr.

The Description fields are just 40-character spaces available to

enter some identifying text.  It will be included as the heading

of a generated listing but otherwise is not required.

Data entry units may be toggled between in-ft-mm-cm any time that

an entry field is not highlighted.  It is perfectly acceptable to

enter a value in one field, then toggle the units and enter a

value in a different field in a different units system.  The

active units system is always evident by the unit labels

displayed.  All values displayed on the screen will be

automatically converted to the selected system.

NOSECONE DATA ENTRY:

====================

Required data entries are simply the diameter and length of the

nosecone.  Alternately, you can enter either one of those and the

aspect ratio of the nosecone.  The aspect ratio is just the

length divided by the diameter, and is also known as the

"caliber."  For example, you might see a nosecone specified as a

"12-inch diameter, 5-caliber ogive," and that means that it is 60

inches long (less any length that might be lost due to rounding

of the nosecone tip).  NTSHAPE does not account for tip

radiusing.  Note that entering an aspect ratio will appropriately

alter the length entry.  When the minimum required data is

entered, the nosecone shape will be displayed.  You can still

change the dimensions, shape, or units, and the display will be

updated.

The shape selection can be made any time that an entry field is

not highlighted, by selecting the function letter associated with

a particular shape, or with the <UP ARROW> or <DOWN ARROW> cursor

keys.  The selected shape is highlighted in blue.  Selecting the

Power Series shape requires entry of an exponent value; values

are accepted between 0 and 1 only. (Play with the exponent value:

try 0 and 1.)  Note that you can also get a hemispherical shape

by selecting a Tangent Ogive shape with an aspect ratio of 0.5.

Selecting the Secant Ogive shape requires entry of an proportion

value; values are accepted between 0.01 and 1 only.  A "1" gives

a Tangent Ogive; smaller values produce a Secant Ogive that is

a portion of a larger Tangent Ogive; i.e., a value of .7 produces

a Secant Ogive fitting the length and diameter specifications,

that is equivalent to the upper 70% of a larger Tangent Ogive.

=================================================================

NTSHAPE  V1.10  page 2               (c) 1994 Gary A Crowell, Sr.

TRANSITION SECTION DATA ENTRY:

==============================

Required data entries are simply the forward and aft diameters

and the length of the transition section (excluding shoulders).

The forward and aft diameters cannot be identical.  When the

minimum required data is entered, the transition section shape

will be displayed.  You can still change the dimensions, shape,

or units, and the display will be updated.

The shape selection can be made any time that an entry field is

not highlighted, by selecting the function letter associated with

a particular shape, or with the <UP ARROW> or <DOWN ARROW> cursor

keys.  The selected shape is highlighted in blue.

CALCULATED VALUES:

==================

When a shape is displayed, the cross section area, surface area,

volume, and Barrowman Center-of-Pressure of that shape will be

calculated and listed in the display area.  The surface area of

a nosecone does not include the base area, and the surface area

of a transition section does not include the area of either end.

Volume measures are exclusive of any shoulders.  These values are

numerical approximations whose accuracy will vary depending upon

the precision value chosen on the first menu screen.  On a 10"

nosecone, a precision value of 1X will result in a CP location

"accurate" to about .1".  A value of 20X will place the CP to

within .001".   The drawback is that choosing a higher precision

will require a correspondingly longer time to calculate and draw

the nosecone.  Choose the precision value based upon the speed

of your computer, and how annoyed you are at the "imprecise"

values.  Note that all CP locations are measured from the aft

end of the component.  Note that the precision selection does

not affect the accuracy of station diameters or shroud

dimensions; they are always accurate to the floating point

resolution of the compiler.

For a tangent ogive shape, the ogive radius will also be

displayed.  The ogive radius is the radius of the arc that forms

the profile shape of an ogive.  The "tangent" part simply means

that the rocket body at the base of the nosecone is tangent to

the ogive radius arc.  For a secant ogive shape, the ogive

radius is also displayed, but this is the radius of the larger

tangent ogive of which the secant ogive is a part.  The base of

the secant ogive is not tangent to the rocket body.

For a conical shape, the cone half angle is displayed.  From a

vertex at the cone tip, the half angle is the angle between the

surface and the center line of the cone.

=================================================================

NTSHAPE  V1.10  page 3               (c) 1994 Gary A Crowell, Sr.

WEIGHT / CG CALCULATIONS

========================

This selection is permanently grayed out, and is not available at

this time.  The CG point shown on the display is accurate

assuming that the shape is a solid of uniform density, with no

shoulders.

FILE MENU:

==========

On this screen you specify the DOS filename that will be used for

the generated station diameter list.  NTSHAPE.LST is the default

but you can change the filename to whatever you want; the file

extention is fixed at .LST.  Only valid DOS filename characters

are allowed in this field.  There is a check to see if the

filename exists and the option is given to overwrite or not.

On this menu you also enter the increment for the station points

on the generated listing.  The base of a nosecone, or the aft end

of a transition section, is the zero reference point, and the

station points will be placed at equal increments starting from

that point.  Both the radius and diameter will be listed at each

station point.

If the shape being generated is a conical transition section,

shroud* dimensions can be generated in the listing file rather

than station diameters, by entering a zero as the value of the

station increment.  Examples of station diameter and shroud

dimension listing files are given in the next sections.

* A "shroud" is the name given a conical transition fabricated

from a flat material such as stiff paper, cardboard, or plywood,

by rolling the material into the proper conical section shape.

STATION DIAMETER LISTING FILE:

==============================

An example of a nosecone file generated by NTSHAPE:

NTSHAPE: Nosecone / Transition Section Shape Generator

(c) 1994 Gary A. Crowell, Sr.

DESCRIPTION:    ASTROBEE D   FULL SCALE 5 CALIBER

SHAPE:          Tangent Ogive NC   OGIVE RADIUS:   303.000  in

BASE DIAMETER:    12.000  in       BASE AREA:      113.097  in2

LENGTH:           60.000  in       SURFACE AREA:  1507.832  in2

ASPECT RATIO:      5.000  calibers VOLUME:        3620.013  in3

CP LOCATION:      29.827  in (from base of nosecone)

STATION         RADIUS         DIAMETER

-------        --------        --------

0.000           6.000          12.000  in

5.000           5.959          11.917

10.000           5.835          11.670

15.000           5.628          11.257

20.000           5.339          10.678

25.000           4.967           9.934

30.000           4.511           9.022

35.000           3.972           7.944

40.000           3.348           6.696

45.000           2.640           5.280

50.000           1.846           3.692

55.000           0.966           1.933

60.000           0.000           0.000

=================================================================

NTSHAPE  V1.10  page 4               (c) 1994 Gary A Crowell, Sr.

SHROUD DIMENSIONS LISTING FILE:

===============================

NTSHAPE does not draw a shroud outline on the printer.  While it

would have been possible to program it that way, it would limit

the shroud to the page size, and introduces several potential

inaccuracies.  First, the printer, even a laser printer, will

introduce distortions, then you'll add more distortions when you

cut out the template, and more again when you transfer the shape

to the shroud material.  Better accuracy can be achieved by

carefully drawing the shroud shape directly on the shroud

material with the proper procedure.  The following is an example

of a shroud dimension listing file, and a detail of a simple

drawing procedure:

NTSHAPE: Nosecone / Transition Shape Generator  V1.10

(c) 1994 Gary A. Crowell, Sr.

DESCRIPTION:

SHAPE:          Conical TS

FORE DIAMETER:     3.000  in

AFT DIAMETER :     4.000  in         C/S AREA :       12.566  in2

LENGTH:            3.000  in         SURFACE AREA:    32.971  in2

ASPECT RATIO:      0.750  Calibers   VOLUME:          29.032  in3

CP LOCATION:       2.310  in (from aft end of transition section)

___----~~~~P~~~~----___          0P =       12.166 in

_.-~~           |           ~~-._     0Q =        9.124 in

_-~                |                ~-_  0W =        7.483 in

.~                   |                   ~.

X ------------------- Y ------------------- Z

\                   |                   /

\                 |                 /   ang X0Z =  22.194 deg

\               |               /     XY = YZ =   4.595 in

\             |             /       AB = BC =   3.447 in

\   __---~~~Q~~~---__   /         0B =        8.448 in

A~------- B -------~C           0Y =       11.264 in

\       W       /             QA = QC =   3.512 in

\     |     /               PX = PZ =   4.683 in

\   |   /                 AX = CZ =   3.041 in

\ | /

0

To accurately draw a shroud only the primary dimensions 0P, 0Q,

and 0W are required.  Other shroud dimensions are listed for

completeness.

=================================================================

NTSHAPE  V1.10  page 5               (c) 1994 Gary A Crowell, Sr.

Shrouds can be drawn quickly and accurately with the following

simplified procedure:  First draw the primary axis 0P, and mark

off the dimensions 0W, 0B, and 0P.  These are the only

measurements required.  Second, with a compass centered at 0,

draw the arcs XPZ and AQC (estimating the endpoints).  Third,

with the compass centered at P, set the compass radiius to the

distance PW, and use this radiius to mark the endpoints X and Z

of the arc XPZ.  Finally, with a straightedge on 0X, draw the

shroud edge AX; similarly draw CZ.

You'll note that this procedure requires only three measurements

taken on the same axis from the same endpoint, and there are no

complications of angular measurements or attempts to measure or

construct perpendicular lines.

NTSHAPE CONFIGURATION FILE

==========================

When exiting NTSHAPE, a configuration file is written with the

filename NTSHAPE.CFG.  This file saves the precision selection,

the station increment, and the units system last used.  Upon

restarting NTSHAPE, the configuration file will be read and the

last selections will be restored.  You may restore the default

selections by deleting the NTSHAPE.CFG file.

HARD COPY PRINTOUT

==================

...of the graphic display is not supported by NTSHAPE.  But if

you must have one, you could run NTSHAPE from a MSDOS prompt

within Windows and press <Alt><PrintScrn> while the nosecone is

displayed.  That copied the VGA screen to the Windows clipboard.

Get back into windows and start PaintBrush (its an accessory that

comes with Windows).  Choose Edit, Paste, and the NTSHAPE screen

will appear.  Choose Pick, Invert, to change the screen to a

white background.  Now, File, Print will print the screen to your

graphics printer.

There are plenty of other screen capture and print options in

both Windows and DOS, in Public Domain, Shareware, and Commercial

versions, that would also accomplish a screen print.

=================================================================

NTSHAPE  V1.10  page 6               (c) 1994 Gary A Crowell, Sr.

NTSHAPE V2.00

=============

If there ever is one, a future version of this program might

include:

* The ability to draw the shoulder too, then add wall thickness

and material density inputs which then permits approximate

calculations of weight and CG.

* Accomodation of nosecone tip radius.

* The ability to output the station list directly to the screen

or printer.

* Data entry improvements.

* Entry of a Conical shape by diameter and cone angle.

* Any other nosecone shapes that might be useful.

* Any other suggestions that *you* might provide.

* I'm not sure that I have the appropriate equations for what

I'm calling the "Power Series" shape.  Any information on

this would be appreciated.

* Gee, if I added fin displays, I could maybe turn this into

a CP/CG program.

Suggestions for improvements, corrections, or bug reports of any

nature would be greatly appreciated.

"Copy-protected software is user- "        Gary A. Crowell, Sr.

offensive"                                6907 Ashland

Boise, ID  83709

208-377-4924 home eves

208-345-4050 work days

208-345-9359 work fax

CIS 71461,1525

garyc@data-cache.com

NCSHAPE / NTSHAPE REVISION HISTORY:

===================================

NCSHAPE:  (nosecone shape generator only)

7/5/93    1.01    Reformatted and updated documentation.

7/10/93   1.02    Added Ellipse shape; CP location display.

11/1/93   1.20    Menu enhancements; allows lower case in

description field.

NTSHAPE:  (nosecone and transition section shapes)

12/30/93  1.00    Initial release to CIS.  Added secant

ogive NC shape and display improvements.

2/17/94   1.10    Added numerical precision selection. Added

NTSHAPE.CFG file.  Added shroud dimension listing

file.  Added prelim CG display.  Corrected data

changed flag problem.

=================================================================

NTSHAPE  V1.10  page 7               (c) 1994 Gary A Crowell, Sr.