DIGITRAK Flight Analyzer Program

BETA TEST VERSION

Author: Larry Curcio

*****         Please report problems to me at curcio@telerama.lm.com       *****

WHAT IT DOES:

The Digitrak Flight Analyzer takes data from onboard accelerometers and reduces them to

three files:

File 1) Flight Summary File

This file prints out you input data, along with output data summarizing the flight. The output

data include maximum altitude, velocity, mach number, and acceleration. There are also motor

statistics such as total impulse, specific impulse, and burning time. The flight summary file

also contains sample-to-sample values for altitude, velocity, acceleration, and drag coefficient.

The flight summary file should be immediately usable by GNUPLOT, a cross-platform

shareware program. GNUPLOT ignores lines in data files that begin with pound signs (#).

Thus, all but the sample-by-sample data begin with this character.

File 2) Drag Coefficient/Velocity Curve file

This file, immediately usable by Digitrak, contains drag coefficients from the rocket's coast

phase. They are listed at 1 m/sec intervals from 0 - 999 m/sec. It is common for this file to

contain some noise. It is therefore smoothed by averaging each point with adjacent points.

File 3) Thrust/Time Curve file

This file is also immediately usable by Digitrak. It is a standard thrust curve, taken from

flight data. It is altitude and temperature adjusted at every point. The cutoff is determined

iteratively, as is the propellant mass expenditure. The smoothed Cd/Velocity curve is used to

create this curve.

In the name of cross platform compatibility, this program does not plot the data. I am going

to try to set up standard arrangements with GNUPLOT on PC, MAC, UNIX and VMS,

however, so that the software will run substantially the same on all those systems. Data are

easily plotted in spreadsheets in the meantime.

VISION:

You put the flight data into the analyzer, and the analyzer gives you three files. You put the

Cd and thrust files into Digitrak, and it reproduces the flight data file. You can then use the

Cd file with other thrust curves, and the thrust curve with other Cd file. You can also vary the

payload weight. Future versions should predict performance when the rocket is scaled up or

down.

Computer networks can be used to share thrust and Cd files for standard commercial products.

CAPABILITIES AND LIMITATIONS:

The Beta Test version accepts Cambridge accelerometer data from dump files, and less

perfectly from Cambridge Print Files. The beta test version cannot accept Adept data, or data

from any other barometric altimeter. The program depends on the data being stable to

differencing, and barometric altimeter data are not stable enough with my present level of

technology. Only single-stage configurations are allowed in this version. I hope to

accommodate multi-stage configurations in the future; however multi-stage treatment cannot

be exact.

The Flight Analyzer accepts data in English units, Metric Units, or a combination (which I

threw in cuz I like to use it) where altitudes are entered in feet, rocket dimensions in

centimeters, and temperatures in degrees F. Output is always in both English and Metric

units.

All flights are assumed to be vertical. For valid results, take data on days with winds less

than  8 MPH.

USING THE DIGITRAK FLIGHT ANALYZER:

I won't describe every input item, but here are a few notes on input:

The program makes a Cd/Velocity table that supplies data up to 999 m/sec. Most flights don't

go that fast, so the program can fake mach effects, according to Mandell, Caporasso, and

Bengen. This option is chosen at input time by saying that you want simulated Mach added to

the Cd Table. Do *not* choose this option if your flight has mach effects in it. You will lose

them in the Cd table. (Not in the input, of course.)

The Nose Type (Sharp or Round) option is used only if you want the simulated Mach effects

added.

Rockets never rise perfectly vertically, but on calm days, stable rockets can approximate a

vertical flight very closely. If a rocket goes significantly off-vertical, then the rocket will be

going at a significant rate of speed at apogee, and lower velocities will be missing from the

coast phase data. In this case, the program takes the Cd for the lowest velocity in the coast

phase, and propagates it back to zero.

See the sample flight summary file for sample input.

SOME POSSIBLE PROBLEMS:

1) DIGITRAK gets a significantly higher altitude than the Flight Analyzer.

This generally happens with off-vertical flights that are entered as Cambridge hex dump files.

Check the last line of the flight summary file that begins with a pound sign(#). This line

contains the lowest velocity found in the coast phase. If this number is high, an off-vertical

trajectory is likely. DIGITRAK will compute the extra altitude that the rocket would attain if

it were to coast vertically from that speed.

2) DIGITRAK gets a significantly lower altitude than the flight Analyzer.

This may happen with Cambridge print files that have an extra long first interval. The flight

analyzer doesn't yet deal well with such data. (The Cambridge software seems to fudge such

data, at least for the thrust curves.)

3) The derived impulse is a bit off.

Impulse computations are very dependent on the launch mass and propellant mass values

entered. If these are inexact, then the impulse will be as well. Low impulses may also result

from off-vertical trajectories. Check the lowest coast phase velocity. This should be below 5

m/sec.

4) The drag coefficients seem low.

Larger rockets, like those capable of carrying the Cambridge unit do have lower Cd's. Low

Cd values can also result from off-vertical trajectories. Check the lowest coast phase velocity.

This should be below 5 m/sec.

COMPILING THE SOURCE FILE:

The Flight Analyzer will compile on most systems. It supports true screen clearing in TURBO

C for DOS, and for UNIX. On other systems, it supports screen clearing through successive

line feeds. It knows how to compile its screen clearing code by the SYSTEM_TYPE

parameter. Search the code for SYSTEM_TYPE. Define this as UNIX for unix systems,

TURBODOS for DOS Turbo systems, and GENERIC for anything else.

A typical UNIX command for compiling the source is:

cc analyze.c -o analyze -lm

On HPUX systems, use the following

cc analyze.c -o analyze -lm -Aa

On MAC systems and on C++ systems that like to have each function prototyped, choose

whatever options allow you to compile the code without prototypes. Note that the interface is

text. The "exit" function, which is standard (damnit!), is not recognized by some visual

systems that eschew standard input and output. They are only pretending. If worse comes to

worst, delete the whole if-statement curly-brace unit associated with the exit function.

If you run out of space, you're either on a commodore 64 or you're on one of those charming

C++ systems where hello_world takes 255K. Write Bill not me. No. Write me too... (grumble

grumble:-)

SAMPLE FLIGHT SUMMARY FILE::

#Units are: Combination

#Enter Launch Site Altitude (ft) : 5840.000000

#Enter Launch Temperature (F) : 50.000000

#Nose Type:  SHARP

#

#Stage Number = 1

#Kind of Input for stage:  Cambridge Dump

#Add Simulated Mach to Cd Table ? : Yes

#Maximum Low-Velocity Cd: 999.000000

#Enter Launch Mass (g) :  1504.000000

#Enter Propellant Mass (g) :  92.000000

#Enter Maximum Diameter (cm) :  10.000000

#Time Interval in Seconds 0.065536

#Input File Name  tom.dat

#

#Total Impulse = 154.963181 ns           34.823188 lb Sec

#Max Altitude = 341.596985 Meters        1120.725025 Feet

#Max Velocity = 85.246658 Meters/Sec     279.680640 Feet/Sec

#Max Acceleration = 8.040497 gs       Min = -1.670349 gs

#

#Isp = 171.788101 Sec           Max Mach Number = 0.252611

#Flight Time = 8.454144 Sec     Burning Time = 1.638400 Sec

#

#Last Velocity Registered in Coast Phase = 0.761640 m/sec

# Time     Altitude(ft)    Velocity(ft/s)    Accel(ft/s^2)    Cd

0.000000    0.000000         0.000000       0.000000     35.311707

0.065536    0.192078         5.861767       178.886938     35.311707

0.131072    1.004470         18.930446       219.937499     5.055681

0.196608    2.758977         34.612862       258.651820     1.682570

0.262144    5.544114         50.382765       222.607437     0.993398

0.327680    9.326911         65.059055       225.277399     0.754516

0.393216    14.079776         79.986881       230.283589     0.590986

0.458752    19.819194         95.166235       232.953526     0.558981

0.524288    26.553763         110.356527       230.617400     0.515795

0.589824    34.278469         125.382767       227.947162     0.450594

0.655360    42.976823         140.069990       220.271310     0.445628

0.720896    52.626252         154.407257       217.267536     0.426822

0.786432    63.209559         168.569546       214.931410     0.427775

0.851968    74.715627         182.567796       212.261272     0.405377

0.917504    87.130840         196.314510       207.255182     0.406029

0.983040    100.435855         209.722220       201.915470     0.411531

1.048576    114.605572         222.703405       194.239042     0.385264

1.114112    129.612050         235.258089       188.899142     0.398903

1.179648    145.427337         247.386273       181.223290     0.374511

1.245184    162.015238         258.836398       168.207150     0.362535

1.310720    179.306229         268.842944       137.168793     0.343586

1.376256    197.161628         276.060782       83.102511     0.340592

1.441792    215.369299         279.593157       24.697006     0.340592

1.507328    233.695590         279.680640       -22.026864     0.340592

1.572864    251.953422         277.504335       -44.388308     0.340592

1.638400    270.035663         274.321916       -52.731407     0.340592

1.703936    287.899310         270.833295       -53.732852     0.340775

1.769472    305.533249         267.311859       -53.732852     0.348803

1.835008    322.936779         263.801361       -53.399035     0.356580

1.900544    340.110226         260.290864       -53.732852     0.362535

1.966080    357.053614         256.780366       -53.399035     0.366355

2.031616    373.767645         253.291720       -53.065606     0.368245

2.097152    390.253771         249.824951       -52.731407     0.368695

2.162688    406.513768         246.390998       -52.064157     0.374511

2.228225    422.549814         242.989835       -51.730721     0.382567

2.293761    438.364037         239.621512       -51.062708     0.394718

2.359297    453.957489         236.253190       -51.730340     0.398903

2.424833    469.329868         232.873928       -51.396526     0.398074

2.490369    484.481825         229.527458       -50.729276     0.391961

2.555905    499.416265         226.235679       -49.727830     0.387930

2.621441    514.136791         222.998593       -49.060199     0.385264

2.686977    528.645857         219.783359       -49.060580     0.385383

2.752513    542.944915         216.590002       -48.392949     0.389415

2.818049    557.036166         213.440399       -47.725317     0.399797

2.883585    570.922065         210.323611       -47.391694     0.411531

2.949121    584.603712         207.206810       -47.725508     0.415381

3.014657    598.080707         204.079071       -47.725508     0.410443

3.080193    611.353451         200.973209       -47.057877     0.404410

3.145729    624.424446         197.922027       -46.056813     0.406029

3.211265    637.296897         194.914599       -45.722805     0.412630

3.276801    649.972655         191.918109       -45.722996     0.415097

3.342337    662.452372         188.932545       -45.389181     0.415097

3.407873    674.737149         185.968859       -45.055555     0.412234

3.473409    686.829139         183.048913       -44.054300     0.405377

3.538945    698.731195         180.172709       -43.720486     0.403485

3.604481    710.446170         177.340259       -42.719228     0.408513

3.670017    721.976619         174.540611       -42.719419     0.418911

3.735553    733.323591         171.740964       -42.719228     0.427420

3.801089    744.486687         168.930390       -43.052855     0.427775

3.866625    755.465606         166.119817       -42.719228     0.427775

3.932161    766.261399         163.342047       -42.051787     0.424446

3.997697    776.876220         160.597079       -41.718164     0.420921

4.063233    787.312221         157.884914       -41.050533     0.422565

4.128769    797.571203         155.194627       -41.050533     0.426822

4.194305    807.654219         152.515277       -40.716718     0.438123

4.259841    817.562019         149.846854       -40.716909     0.438123

4.325377    827.294954         147.178431       -40.716909     0.446193

4.390913    836.853374         144.520946       -40.382901     0.450546

4.456449    846.237930         141.874412       -40.383092     0.445628

4.521985    855.449773         139.249743       -39.715651     0.443690

4.587521    864.490706         136.657876       -39.381837     0.443690

4.653057    873.362130         134.076948       -39.381837     0.443349

4.718593    882.064746         131.506958       -39.048023     0.446015

4.784129    890.599656         128.958833       -38.714397     0.443546

4.849665    898.968361         126.432585       -38.380582     0.443546

4.915201    907.171862         123.917275       -38.380582     0.450594

4.980737    915.210461         121.401965       -38.380582     0.458881

5.046273    923.084957         118.908520       -37.713142     0.484639

5.111809    930.796352         116.426013       -38.046765     0.499790

5.177345    938.345148         113.943506       -37.713142     0.499790

5.242881    945.731243         111.460999       -38.046765     0.515795

5.308417    952.954637         108.978493       -37.713142     0.509127

5.373953    960.016033         106.517863       -37.379325     0.509170

5.439489    966.916830         104.079110       -37.045604     0.509170

5.505025    973.658932         101.673160       -36.378164     0.527395

5.570561    980.243740         99.278148       -36.711884     0.557401

5.636097    986.671155         96.872198       -36.711884     0.557401

5.701633    992.940174         94.444377       -37.379325     0.558981

5.767169    999.050098         92.016556       -36.711884     0.526058

5.832705    1005.002327         89.632477       -36.044446     0.514563

5.898242    1010.800167         87.303077       -35.043095     0.514563

5.963778    1016.445718         84.984616       -35.710629     0.548977

6.029314    1021.938182         82.633346       -36.044446     0.589145

6.094850    1027.276255         80.271137       -36.044349     0.590986

6.160386    1032.460238         77.930805       -35.376909     0.590986

6.225922    1037.491934         75.623283       -35.043095     0.564780

6.291458    1042.373044         73.337631       -34.709468     0.571073

6.356994    1047.104771         71.062917       -34.709374     0.571073

6.422530    1051.687814         68.799136       -34.375654     0.635737

6.488066    1056.122475         66.535354       -34.709374     0.754516

6.553602    1060.408353         64.260641       -34.709468     0.754516

6.619138    1064.544446         61.964051       -35.376909     0.853898

6.684674    1068.529352         59.645589       -35.376815     0.915499

6.750210    1072.362673         57.338067       -35.043189     0.943673

6.815746    1076.045808         55.063353       -34.375654     0.943673

6.881282    1079.580661         52.810510       -34.375654     0.971793

6.946818    1082.967832         50.557664       -34.375701     0.993398

7.012354    1086.207321         48.304818       -34.375654     0.993398

7.077890    1089.299528         46.062911       -34.041934     1.065296

7.143426    1092.245556         43.842874       -33.708167     1.245968

7.208962    1095.046405         41.633776       -33.708167     1.245968

7.274498    1097.702175         39.413739       -34.041934     1.490270

7.340034    1100.212066         37.182767       -34.041934     1.682570

7.405570    1102.575776         34.951795       -34.041887     1.682570

7.471106    1104.793607         32.731761       -33.708167     1.779586

7.536642    1106.866660         30.533598       -33.374446     2.043692

7.602178    1108.796036         28.346370       -33.374446     2.043692

7.667714    1110.582035         26.159143       -33.374421     2.608582

7.733250    1112.224057         23.950044       -34.041909     3.688597

7.798786    1113.720900         21.730009       -33.708167     3.688597

7.864322    1115.072564         19.520911       -33.708188     5.055681

7.929858    1116.279851         17.322748       -33.374421     7.913355

7.995394    1117.343461         15.135522       -33.374421     7.913355

8.060930    1118.263694         12.948296       -33.374434     13.402117

8.126466    1119.040250         10.750133       -33.708179     22.964018

8.192001    1119.672729         8.551970       -33.374434     22.964018

8.257537    1120.161531         6.364743       -33.374427     35.311707

8.323072    1120.506656         4.166581       -33.708176     35.311707

8.388608    1120.725025         2.498820       -17.187833     35.311707