DIGITRAK 5.1 DOCUMENTATION

Copyright Larry Curcio

29 December 1994

Including Program Updates by Warren Massey

17 December 1994

SEE ** GETTING STARTED FAST ** BELOW

[See COMPILER NOTES, RUNNING THE PROGRAM (try the exercises), and FIELD

INSTRUCTIONS below]

DESCRIPTION: DIGITRAK is a vertical rocket altitude simulation program (RASP

program) with the following features:

1) Finds altitude and ascent time, and other trajectory statistics from drag

coefficient, diameter, launch mass and engine (i.e.; standard RASP

functions);

2) Finds altitude and drag coefficient from observed ascent time (Temporal

Backtracking);

3) Finds drag coefficient and ascent time from observed altitude (Optical

backtracking);

4) Calculates optimal mass;

5) Supports up to three stages;

6) Supports transonic and supersonic rockets;

7) Uses a configuration file to specify user preferences;

8) Depreciates air density with altitude;

9) Can use drag profiles in prediction and backtracking;

10) Supports staging delays and air starts;

11) Can output GNUPLOT-compatible file with altitude, velocity, acceleration,

and drag coefficient versus time.

12) Input/output in English or metric units.

Thus, DIGITRAK is designed to be used as a practical tool for model and high

power rocket development.

===========================================================================

** GETTING STARTED FAST **

1) Compile (and link, if necessary) the program. As it is, it should compile

straightforwardly on most platforms. You may wish to tweak a platform-

dependent switch or two for extra function on your machine. (See Compiler

notes below)

2) Edit the file, digitrak.cfg, and follow instructions. Change settings as

appropriate. In particular, you may wish to specify separate directories

for motor files and/or drag coefficient files. These are the last two

items in the file. (If these files are to share the same directory as

the program, you may specify '' for each of them.)

3) Secure the file, RASP.eng. Put it into the motor file directory. Also,

put the file Mark.c into this directory. Edit Mark.c. If you are on a

VMS system (or any other system that takes long file names with no

embedded blanks) set

#define VAX 1

#define MAC 0

If you are on a MAC system, or any other system that takes long file

names *with* embedded blanks, set

#define VMS 0

#define MAC 1

If you are on a system that takes 8-character (or fewer) file names,

set

#define VMS 0

#define MAC 0

Compile (and link, if necessary) Mark.c, then run it. Specify RASP.eng

as your input file. Mark.c (named for Mark Johnson and Mark Speigel,

who created the thrust curves in that file). The program produces over

a hundred separate motor files - so make sure you use a clean directory!

4) Go back to the program directory and run the program. Two tips:

a) When specifying a motor, enter a FILE NAME, *without* the extension,

.eng; and

b) Masses are *LAUNCH* masses. That is they *DO* include the motor mass.

Many simulation programs require that motor-less masses be entered.

(Because DIGITRAK is designed to be used to calculate altitudes and

Cd's for *particular* rockets, it requires that rocket's *actual*

launch mass, rather than an estimate.)

Note that earlier versions of DIGITRAK required that the input units (Metric

or English) be set at compile time. Thanks to Warren Massey's updates, this

option is set at run time. One may also set the elevation altitude of the

field in Warren's configuration file, rather than in the program. (Your

club's launch waiver is a good source for this information.)

=============================================================================

***  DIRECTORY VALIDITY CHECKING ***

DIGITRAK can be made to check the validity of motor and Cd directory names.

Since the syntax of such names varies from platform to platform, this feature

is set at compile time. The appropriate #define to set is:

#define DEVDIR_DELIMITER '<string>'

where <string> is a string of delimiters used in directories on the

platform of your choice. For example, in DOS,

#define DEVDIR_DELIMITER ':\' /* Setting For DOS Systems */

The program will look for at least one instance of such a delimiter in the

directories specified in the .CFG file. It will then try to read a file,

temp.dat from that directory. In the (unlikely) event that the read succeeds,

the directory will be accepted as valid. Otherwise, the program writes

temp.dat. If the write succeeds, the program deletes the file and declares

the directory valid.

To disable directory validity checking, set

#define DEVDIR_DELIMITER '' /* Setting to Disable Validity Checking */

The above setting is the default. Note that if your copy has a different

setting, then the program has been edited by a third party. Because the

extent of such editing is unknown, the program may not conform to this

documentation file.

=============================================================================

*** COMPILER NOTES ***

DIGITRAK should compile straightforwardly on any system that conforms to

ANSI standard C. Special instructions for some particular operating systems

follow.

TO COMPILE IN UNIX:

Look for a statement near the beginning of the code like the following:

#define UNIX 1

or

#define UNIX 0

Make sure it says,

#define UNIX 1

This makes the file names case-sensitive. Otherwise, the names default to upper

case. As delivered, the UNIX definition should be in the correct state. Then

enter the following:

cc digitrak.c -lm -o digitrak

This version requires the presence of the system 'clear' command. If this

doesn't work on your system (though it usually does), you can use the curses

library to effect a clear with the corresponding settings and commands.

#define UNIX 2

cc digitrak.c -lm -lcurses -o digitrak

DIGITRAK is compatible with ANSI C; however on some UNIX systems, an

additional compiler switch must be set to invoke this standard. (Two

examples of compiler switches appear in the example: -lm and -o) The

compiler switch itself is not standard in UNIX. (As if anything *were*

standard in UNIX! On HP platforms, the switch is -Aa) Because DIGITRAK has

become large of late, I have decided to conform to ANSI, rather than to K&R

because the ANSI standard checks function calls at compile time. (As it

happens, even the latest edition of K&R supports ANSI, so the association

of K&R with the old standard is arguably inappropriate.)

Anyway, on *HP* platforms try  (with #define UNIX 1)

cc digitrak.c -lm -Aa -o digitrak

or (with #define UNIX 2)

cc digitrak.c -lm -lcurses -Aa -o digitrak

I did find one UNIX compiler that didn't support the alarm character, \a. This

compiler produced a warning message, and produced a functional program without

the alarm character. Should you get a warning message of an unsupported

character, try the code and (as Douglas Adams would advise) DON'T PANIC!

THINK C on the MAC:

DIGITRAK should compile without errors.

If you wish to make the program case sensitive, then find the

#define SENSITIVE

statement, and set it

#define SENSITIVE 1

Set the UNIX and ANSI switches to 0 (They are like the SENSITIVE switch).

TURBO C For DOS

DIGITRAK will give special function in this compiler, including color and

true screen clearing. To avail yourself of this function, look for the

definition of TURBODOS, and set it to:

#define TURBODOS 1

DIGITRAK compiles with one warning in DOS TURBO C; however version 5.1 is

too big for the program editor. The easiest way to compile DIGITRAK in

TURBO C is to use the command line compiler, TCC. If you have not installed

TCC, then split the program into two pieces, DIGIT1.c and DIGIT2.c. The

beginning statement of DIGIT2.c should be:

#include "DIGIT1.c"

(Assuming that DIGIT1 is in the same directory, of course).

TURBO C++ For Windows.

Compiles with many warnings. DO NOT set TURBODOS to 1. You may, if you wish,

set

#DEFINE CLS clrscr()

for true screen clearing.

MICROSOFT VISUAL C++ for WINDOWS:

In project options, specify your project as a QuickWin application. In

compiler options, set the memory model as HUGE, set the code as 80386 (if

you have a 386 or better), specify inline 8087 instructions, and optimize for

speed. The program compiles with a large number of warnings, since it uses

C++ standards; however it should have no errors, and should run directly.

This method produces a module that won't run on 386 and below processors without

math coprocessors. Also, you will have to maximize two windows before you can

run, and you won't get true screen clearing. I prefer the following method:

Specify a DOS application in your project options (making sure that stdio is

included among the libraries, otherwise the compiler won't find printf).

Choose full optimization and, huge memory model and emulation. Include:

DEVICEHIGH \dos\ANSI.SYS

in your config.sys, and compile DIGITRAK with the ANSI switch set to 1. This

will give you true screen clearing, and a robust module that fills an entire

screen.

VMS VAX

The Program should compile and link directly.

Most VAX terminals are ANSI compatible. Setting

#define ANSI 1

will cause DIGITRAK to deliver true screen clearing on such terminals.

OPEN VMS ALPHA

On some versions of the ALPHA C compiler, the function, fopen() is not

defined in the standard way. (i.e.; the compiler flags the declaration

of FILE * fopen();) Delete or comment out the two declarations of fopen().

More recent versions of ALPHA C define fopen() correctly. For screen

clearing

#define ANSI 1

as in VAX C.

===========================================================================

****** GENERAL INFORMATION ******

STAGING DELAYS

Undelayed staging is a technique designed to achieve great velocities. In the

lower atmosphere, this is not the best way to achieve altitude, because

rockets realize diminishing altitude returns to added velocity there. (This

is after a certain velocity has been reached. At low velocities, rockets

actually realize increased returns to added velocity.) It is therefore often

advantageous (in principle) to delay the ignition of an upper stage until the

rocket has slowed down somewhat, but not so long that the rocket comes to a

halt. See warning below!

There are two kinds of delayed staging. In a simple staging delay, the

burned-out lower stage remains attached while the rocket coasts for a time

interval before the next stage is ignited. In what I call an air start delay,

separation occurs at lower stage cutoff, but there is a delay before the next

stage is ignited. The choice of delay type will depend on the relative

ballistic coefficients of the lower and upper stage.

WARNING! *** READ THIS PARAGRAPH *** The actual effects of staging delays of

either kind cannot be adequately predicted by any simulation program, least

of all a vertical trajectory program. Vertical trajectory RASP programs

assume that all motion is either up or down - this is tantamount to the

assumption of sophisticated active guidance. Staging delays, particularly

those that result in low staging velocities, may cause rockets to veer

radically from vertical trajectories. They may even cause powerdives. This

should be obvious because at low staging velocities, one is effectively

setting off a rocket in the sky without a launch rod. Such untoward results

can be simulated in off-vertical trajectory programs, but they cannot be

reliably predicted by any simulation, since they may be caused or worsened by

unpredictable turbulence or a capricious puff of wind. DIGITRAK prints out

staging velocities to give the user an intuitive idea of a rocket's stability

at staging, however the authors cannot assume any liability in any case.

Staging delays work only in vertical trajectories. In off-vertical flights,

staging delays often result in lost altitude compared to no delays. More

importantly, they usually cause vast increases in the horizontal distances

covered by rockets.

OPTIMAL MASS CALCULATION

In a single-stage rocket, DIGITRAK finds the optimal mass straightforwardly.

In a multi-stage context, DIGITRAK assumes that the lower stages are as light

as possible. It then finds the best mass adjustment (positive or negative)

for the highest stage, and adds this adjustment to the mass of each stage

(since the mass of every stage includes the mass of the top stage). This mass

will always be the best possible upper stage adjustment. An undelayed staged

configuration has no feasible optimal launch mass, because the optimal amount

of non-propellant mass in a lower stage (exclusive of upper-stage mass) is

zero. The mass calculated by DIGITRAK will, however, be the *best feasible*

mass for:

1) All undelayed staged configurations;

2) All 2-stage configurations where an air start delay is used;

3) All N-stage configurations with only one delay that is situated before the

last stage, and is an air start delay;

4) Virtually all practical 2-stage configurations with simple staging delays,

as long as the top stage is not dwarfed by the bottom stage.

The value may be sub-optimal for some odd configurations with simple staging

delays. That is, there may be an attainable non-zero optimal lower stage dead

mass for a rocket with simple staging delays - if the delay is designed to

coincide with a non-negative staging velocity. When this is so, then optimal

mass for the lower stage is less than or equal to what it would have been if

that stage had been the ultimate stage. This is so only when the upper stages

are very small compared to the next lower stage. If the upper stages are

larger than, as large as, or only moderately smaller than the next lower

stage, then the lower stage should be made as light as possible. This is

usually the case in practice.

Globally optimal mass distribution for all delayed configurations is a linear

programming problem, and is not supported in this version, nor are optimal

delay/stage mass combinations. DIGITRAK should be adequate for most

real-world applications.

It may be observed that calculated optimal mass differs somewhat from one

program to another. When this is true, however, it should also be observed

that altitude differences are trivial when the other program's mass value is

substituted. Optimal mass values may also vary slightly with variations in

engine performance, and variations in air density (temperature and humidity).

They are not to be taken as gospel truth, but as good working values. Optimal

mass occurs as a "Knee" in a rocket's altitude/mass curve.

SPECIAL NOTES ON OPTIMAL MASS:

o The optimal mass for rockets with Cd=0 (or very small) is the propellant

mass. In other words, the smaller, the better. DIGITRAK will not calculate an

optimal mass for drag coefficients less than .015.

o When optimal mass occurs in the transonic region, there may be more than

one local maximum in the altitude/velocity curve. DIGITRAK may then display

a local maximum that is not the global maximum. Calculations for transonic

and supersonic rockets have limited accuracy anyway. See below.

TRANSONIC AND SUPERSONIC SUPPORT

Supersonic support is accomplished through the semi-empirical method

described in Mandell, Caporaso, and Bengen Topics in Advanced Model Rocketry.

MC&B use a critical mach number of 0.9. The program as delivered uses an

adjusted formula for a critical mach number of 0.8. To get MC&B's original

formulas, change MCRIT in the function, machmaker to 9. (The compiler option

flags will lead you there. See COMPILER NOTES below.) Unfortunately, the

model assumes zero angle of attack. As rockets approach the speed of sound,

angle of attack oscillates unpredictably. This difficulty is present in every

RASP program supporting transonic and supersonic speeds. If the barrier is

traversed quickly, the errors encountered are tolerable. As of this writing,

there is no totally adequate drag divergence model that can predict angle of

attack variations. (Though the effect of such variations can be modeled

pretty well.)

*** MORE COMPILER NOTES ***

The source text contains flags that lead the user to places where options may

be tweaked. Look for /*COMPILE OPTION*/ in the text.  There is also a

"Hackers' area," where some things may be changed that most people won't want

to change. Most #defines are documented or otherwise obvious (e.g.; PI).

o TURBO C for DOS Adjustment: set #define TURBODOS to 1, otherwise set it to

0. This switch invokes color, screen clearing and screen positioning in TURBO

C for DOS.

or, more generally:

o SCREEN CLEARING, the #define CLS STATEMENT: CLS should be #defined in the

beginning to be the clear screen command for your system. In TURBO C, this is

clrscr(). On UNIX systems this may be defined as whatever clear screen

function prevails. Alternatively you can use printf("\n \n \n \n \n"). You

can use the number of CR/LF's that will actually clear the screen. Version IV

allows screen clearing for ANSI displays through the printing of an escape

sequence. ANSI displays include VTxxx terminals, and PC's with ANSI.SYS

installed in the CONFIG.SYS file.

o COLOR: If your system has color, DIGITRAK has a function called "Color()",

in which you can write a machine-dependent routine.

THRUST CURVES IN DIGITRAK

DIGITRAK can be used with thrust curves from SIMLAB, or with thrust curves

converted from the RASP program. The engines are called by the thrust curve

file names. They must all have the type, ".eng", which is NOT to be entered

into DIGITRAK. I made a small program to cut the RASP motor file (in whatever

incarnation) into individual thrust curves. The thrust curves in the motor

file I was using were prepared by Mark Johnson. They are piecewise linear and

made to conform to shape, impulse, and maximum thrust of over 100 motors. I

call the conversion program, MARKURVE.c, for Mark. In addition, all the

individual files put out begin with the letters, mj. (Note, Mark Speigel

has recently contributed to this file as well.)

The program should work directly. Simply compile it, execute it, and give it

the name of the motor file. It will create MANY thrust curves, THEREFORE IT

SHOULD BE RUN IN A CLEAN DIRECTORY. The files all have the type, ".eng". The

letter after the prefix, mj, is the manufacturer ala RASP conventions. The

remaining letters describe the motor itself. Thus:

mjae15.eng is Aerotech E15

mjee15.eng is the Estes E15.

RUNNING THE PROGRAM

[SPECIAL NOTE ON MASS ENTRY:

ALL MASSES IN DIGITRAK ARE LAUNCH MASSES. THEY *INCLUDE* MOTOR MASSES

Rocket masses smaller than motor masses will cause abnormal terminations.

THE MASS OF A STAGE *INCLUDES* THE MASS OF ALL UPPER STAGES

Thus, for example, the mass of a first stage is the mass of the entire

rocket.]

Invoke the program by entering:

DIGITRAK

Choose T for temporal backtracking, O for optical backtracking, and P for

prediction (the default - direct simulation from drag coefficient). You may

use upper-case or lower-case letters as you wish. If you wish to backtrack

with using a profile instead of a drag coefficient or shape constant, enter

TP or OP instead of T or O for (respectively) temporal or optical

backtracking with a profile. (Profile backtracking alters trim factor instead

of drag coefficient. Trim factor is a factor by which the entire profile is

multiplied to make it consistent with flight data.)

If T is chosen, then enter the launch mass, engine name, diameter and ascent

time.  If TP is chosen, you must enter the same data and a profile file name

besides. If O is chosen, then enter launch mass, engine name, diameter and

altitude.  If OP is chosen, you must enter these data and the name of a

profile as well. If P is chosen, then enter launch mass, engine file name,

diameter and drag entry.

A drag entry is one of the following:

1) A drag coefficient. DIGITRAK assumes a drag coefficient has been entered

if the drag entry is a number, and if a non-zero diameter is entered;

2) A shape constant (the sea-level constant by which the square of velocity

must be multiplied to give the drag force). DIGITRAK assumes a shape constant

if the drag entry is a number and if the diameter value is zero; or

3) A drag profile. DIGITRAK assumes a drag profile if the first character of

the entry is non-numeric. A profile is a file of drag constants for each

consecutive integral velocity from 0 m/sec to some large number (<1000)

m/sec. Drag profiles also have information that went into Cd calculation -

including diameter. At this writing, there is no C program for creating these

files. SIMLABV5 has two profile generators that run on IBM clones. These are

being converted. Profiles may take mach effects into account or they may not.

Current profiles do not. They may be corrected for mach effects by DIGITRAK

by concluding the profile entry with /c or /C. Profile file names and file

types must both be entered.

Engine names are the file names (not file types) of engine thrust curve

files. Engine files must have a file type of ".eng".

When the data are entered, DIGITRAK produces trajectory statistics.  Enter N

to quit, or Y to perform another run.  The values entered on the last run

become the defaults for the next.  This feature is convenient for "What If"

questions.   At the end of each run, the program allows you to compute your

rocket's OPTIMAL MASS, the mass that yields the highest altitude for the

rocket's impulse and drag characteristics.  If you enter 'O' at the prompt,

you will see estimates of your rocket's performance at its optimal mass

Entering N at the end of the run terminates the program.

Try these configurations for practice:

Configuration #1

1 stage, 100 grams (3.53 oz), mjec6 motor, Diameter=4.16 cm (1.64 inches),

drag coefficient = 0.6.  Altitude will be about 523 feet. Optimal mass is

about 52 grams. Allow the computer to do another calculation. Choose temporal

or optical backtracking. <RETURN> through the input, choosing all the

previous values. The ascent time or altitude from the last run is the default

entry for the backtracking data. Your drag coefficient and altitude should be

extremely close to their previous values.

Configuration #2

2 stages.

Stage #1: 5100 grams (179.9 oz) launch mass, mjik550 motor, diameter=6.5 cm

(2.56 inches), Cd = 0.6

Stage #2: 100 grams (3.53 oz) launch mass, mjec6 motor, diameter=2.48 cm

(.976 inches), Cd = 0.5

Altitude should be about 5163 feet. Optimal mass of first stage is about 5885

grams. For the second stage, optimal mass is about 885 grams (Yes. Because of

the very large first stage, the weight of the upper stage at its optima mass

exceeds its thrust.) Allow the computer to do another calculation, and answer

'yes' when the program asks if you want to keep the optimal mass statistics.

Choose temporal or optical backtracking. <RETURN> through the input, choosing

all the previous values. The ascent time or altitude from the last run is the

default entry for the backtracking data. Your drag coefficient and altitude

should be extremely close to their previous values. Try backtracking each

stage.

Multi-stage Sensitivity Exercises:

o Try changing the drag coefficient of the first stage in the last problem to

1. Does this change make much difference in altitude of the original

configuration - of the optimal mass configuration - of the drag coefficient?

What are the % errors? Does it change optimal mass much?

o The first stage in this configuration is *much* more powerful than the

second stage. Would you expect a more balanced (and realistic) configuration

to be more or less sensitive to first stage drag coefficient?

o Make up a realistic configuration to test your answer. Try backtracking

this configuration, using widely varying drag coefficients. What % altitude,

drag coefficient, and optimal mass error do you encounter in the new

configuration? At this point, you should be able to appreciate the note

below.

MULTI-STAGE NOTE: 3-stage rockets are virtually impossible to follow well

enough to backtrack. In practice, most 2-stage rockets can be backtracked by

eyeballing a Cd for the first stage, and backtracking the second stage. Most

such configurations are extremely insensitive to changes in first-stage Cd.

It is wise to check the altitude and drag coefficient sensitivity to the

value of first stage Cd. If the diameters of the stages differ widely then

use an reasonably small value for first stage Cd as a lower bound. To get a

safe lower bound for first stage Cd, and safe upper bounds for second stage

Cd and altitude when diameter is constant, do the following. Backtrack the

second stage with any with any first stage Cd value, then substitute the

backtracked Cd of the top stage for that of the bottom stage, and

rebacktrack. Repeat the procedure until self-consistency is reached. (This is

often reached on the second try, and confirmed on the third.) A safe upper

bound for lower stage Cd is usually unity. The corresponding altitude and

upper stage Cd are good lower bounds for both. The range should be narrow, so

split the difference. The only way to get a good lower stage Cd value is to

launch that stage as a single-stage rocket.

FIELD INSTRUCTIONS FOR TEMPORAL BACKTRACKING

Temporal backtracking is easy to do, but there are many things that one can

do wrong. I myself spent several months believing that temporal backtracking

and RASP programs in general did not work. The belief was based on errors I

had made in the experiments. I have prepared this guide so that others will

not commit the same errors, and reach the same false conclusions about the

method.

0) TAKE TIME TO LEARN THE BACKTRACKING PROCEDURE. Practice on rockets that go

500 feet or less until you have the knack. It helps to verify your learning

by comparing backtracked altitudes with single-station tracking. Altitudes

should compare very well on average. The tendency is to record short times,

at first. Even without tracking verification, this sort of error usually

causes conspicuous underestimates.

1) Always backtrack in calm weather, that is in winds of 7MPH or less. This

is extremely important since high winds cause strong violations of the

mathematical model. Drag coefficients measured on calm days may be used to

predict altitudes in winds up to about 10 MPH (provided weathercocking is

minimal); however they cannot be used to predict parachute delays except in

calm weather.

2) For best results, use large, easily-tracked models. Though experienced

backtrackers can often time highly-visible rockets from near the pad, it is

wise to have timers stationed at distances of at least half the anticipated

altitude of the rocket. Ironically, the farther you are from the launch pad,

the more likely it is that you will not lose sight of the rocket. The more

timers you have, the more chance that at least one of them will be able to

track the model. Since digital stopwatches are inexpensive, there is no

reason why everyone at a launch should not have one.

3) Rocket engines must have the longest delays available. That is, use

upper-stage engines in your single-stage rockets. This is vital. Long delays

guarantee that your rockets will reach their highest points.

4) Make sure you weigh your rockets on a gram scale. Most schools have

triple-beam balances, which are ideal. Postal scales are not accurate enough.

You may weigh your rockets on the field before launch, or (if they are large

enough so that a gram or two makes negligible altitude difference) you may

weigh them at home or at school. If you do not weigh your rockets immediately

before launch, make certain that they are loaded consistently, so that you

can get an accurate estimate of launch weight.

5) Make certain your launcher is vertical. To do this, hang a beaded metal

chain, of the type used on old electrical pull switches, next to the launch

rod. Adjust the rod until it is parallel with the chain. (Beaded chain is

available at most hardware stores.)

Alternatively, you may wish to tilt your launcher by two degrees. This is

useful when you launch alone, as it causes the rocket to reach apogee some

distance from you while introducing very little error into the backtracking

process. However, though your calculated altitude is likely to be correct, an

off-vertical launch may give you an inflated value for drag coefficient,

since gravity causes the rocket to interact with the air at a non-zero angle

of attack. Also, this method is not suitable should you decide to verify your

backtracking with single-observer optical tracking.

6) Have the person who is launching the rocket give an audible countdown.

Hand signals or whistles are helpful to distant observers. Understand that

there will likely be a short pause between the button push and ignition.

7) Time the rocket's ascent from first *blast* of smoke to the point of

apogee. FOLLOW THE INSTRUCTIONS IN THE SECTION BELOW CALLER _DETERMINING THE

MOMENT OF APOGEE_

8) Average the times from everyone who declares an accurate measurement,

though you should discard wildly inconsistent times from inexperienced

timers.

9) Enter the data in the DIGITRAK program to calculate your altitude.

10) Use average altitude over several flights to calculate your drag

coefficient.

*** DETERMINING THE MOMENT OF APOGEE ***

The hardest part of temporal backtracking is the determination of apogee. You

cannot just pick an arbitrary moment in time when the rocket is spatially

near apogee, because it is traveling slowly there, and it spends a lot of

time in the region. Small timing errors translate into altitude errors by

multiplying them times the average speed of the rocket throughout its ascent.

The effects of larger errors are worse. Thus, a second's error typically

translates into about 200 feet of altitude.

The key to judging apogee is to step back at least half of the rocket's

anticipated altitude. FARTHER IS BETTER. If you are back far enough, you can

see the tracking smoke form an arc at apogee. The successful backtracker

learns to anticipate the formation of this arc and to extrapolate its high

point, so that (s)he can clock the precise moment of apogee. Sometimes the

smoke trail of a rocket will disappear immediately after cutoff. This means

that the rocket is traveling so quickly that its tracking smoke disperses.

The smoke will reappear much higher in the sky, when the rocket slows down.

When you lose a rocket at burnout don't panic. Remember to look for the smoke

trail much higher in the sky. If you still don't see the trail, then look

higher than you had dared. As soon as you see the trail again, you must

extrapolate the arc so you can clock accurately. That is, don't perceive the

smoke trail as a straight line, rather perceive it as the leg of an arc about

to form.

Rockets that fly directly toward you or directly away will sometimes not

reveal an arc; your timing of these will be slightly less accurate. Clock the

point when you just notice that the rocket has nosed over beyond horizontal

attitude. The idea is that, if the rocket goes high enough, it will stray

enough from the vertical to have horizontal velocity at apogee. It is

presumably stable, so it should follow its nose. At its peak, it should be

horizontal, but if you try to clock the point when the rocket is horizontal,

you may be struck by how much "more horizontal" the rocket becomes a moment

later. The moment when the rocket just passes this point is least ambiguous.

Very low altitude rockets will often not arc over, because they don't acquire

enough horizontal speed. You can usually see such rockets begin to drop, so

clock that moment. There is an additional problem with low altitude rockets,

though. Since they travel slowly, drag has a relatively small influence on

them. I have seen cases when doubling the drag coefficient changes the

altitude no more than 30 feet. Ascent time is also weakly affected. As a

result, drag coefficient is even more sensitive than usual to measurement

errors. Six or seven flights may be required to determine the drag

coefficients of such rockets.

PITFALLS

1) Rockets travel very slowly near apogee, which is to say they stay around

apogee for a long time. It is not sufficient to time a rocket until it gets

around it's apogee, because it is going to stay within 16 feet of that point

for about 2 seconds. Timing errors translate into altitude by multiplying

them by the average speed of a rocket during its entire flight, not near its

apogee. Nevertheless, there is a tendency to be glib about clocking. CLOCKING

MUST BE PERFORMED RIGOROUSLY, ACCORDING TO THE INSTRUCTIONS ABOVE.

2) Light rockets will often eject just before apogee - even with engines

intended for second stages, as delay times are extremely variable. There is

a tendency to accept data from flights like this because (once again), the

rocket seemed "Almost horizontal." These flights cannot be temporally

backtracked. If you wish to compute a drag coefficient, then weigh these

rockets down with tracking powder.

3) Often a rocket will disappear as it jumps off the pad, only to reappear

again at ejection. There is a tendency to clock this point. There is even a

tendency to perceive this point as apogee. The data in such cases, are

invalid. Discard them. Remember that it is easiest to clock large rockets.

MEASURING DRAG COEFFICIENTS (Cd)

Drag coefficient is often sensitive to errors in backtracking parameters

(time, altitude, engine performance). This is true for both optical and

temporal backtracking. The object is to produce a Cd that delivers an average

altitude for the rocket. In practice, this will vary only slightly with the

backtracking program.

Fly each rocket several times, and compute a mean altitude.  Then backtrack

optically on the mean altitude to get the drag coefficient.  The more flights

you use, the more accurate your Cd is, on average.  Expect performance to

vary from flight to flight, since engine performance varies. Also, random

variations in launch angle, and stability corrections affect your rocket's

angle of attack, and hence its drag characteristics.

Sometimes rocket masses will vary slightly from flight to flight. In this

case, decide on a standard weight. Backtrack working drag coefficients for

each flight. Using the working drag coefficient, predict an altitude at the

standard launch mass. Use this predicted altitude in your average. Compute

the drag coefficient for the standard launch mass from the average altitude.

Do not use widely different configurations in the computation of a single

drag coefficient, as Cd varies somewhat with speed.

See the MULTI-STAGE NOTE in the exercises at the end of RUNNING THE PROGRAM

for multi-stage backtracking procedures.

MEASURING ALTITUDE

Backtracked altitudes are much more robust than backtracked Cd's are. Average

altitudes are more accurate still. In my own experiments, the average

altitudes of 11 backtracked rockets were correlated at a level of .99 with

average directly-measured altitudes. See the MULTI-STAGE NOTE in the

exercises at the end of RUNNING THE PROGRAM for multi-stage backtracking

procedures.

LIMITATIONS

BACKTRACKING CANNOT BE PERFORMED IN WINDS GREATER THAN 7MPH, because strong

violations of the simulation model occur in the wind. Temporally-computed

altitudes will be dramatically short in windy weather.  Wind, thus, affects

ascent time more than it affects altitude, taking time preferentially from

low-velocity portions of the trajectory. Fortunately, therefore, drag

coefficients computed temporally in calm weather will predict ALTITUDES

fairly well for winds up to about 10-12MPH; however they will not predict

required parachute delays except for calm weather launches.  No simulation

program will predict performance on windy days.

Rockets must be clearly visible at their apogee. Visibility is usually best

in the morning or late afternoon, and naturally large rockets are easier to

see than small rockets. Visibility is enhanced when viewing from a distance.

Timers should be stationed at least half the rocket's expected altitude from

the launch pad. Longer distances are better. If several individuals are

viewing the launch, and each of them carries a stopwatch, chances are

excellent that at least one of them will follow the rocket to apogee.

Redundant timings should be averaged.

Backtracked rockets should have maximum speeds below mach 0.75.

FURTHER INFORMATION

Much work has gone into the development of temporal backtracking, and into

the testing and verification of RASP programs. The important results are

merely summarized here. I cannot, however, present an extensive sensitivity

analysis in this document. A detailed account of my experiments is a 93-page

(double-spaced) document. This document will soon be available through the

National Association of Rocketry Technical Services. A summary of that report

appears in _High Power Rocketry_ magazine, December 1994 issue.

=============================================================================

UPDATE HISTORY

****** CHANGES IN VERSION 2.0 ******

o Runge-Kutta method employed. (Slightly faster. Answers are the same.)

o Line deleted from boost procedure, which cause altitude    over-estimation

(Previously announced)

o MAXITMASS changed to MAXITMAS, and DELTAMASS changed to DELTAMAS   (Some

compilers cannot take #define's > 8 characters)

o Tabs removed before one set of #if/#else statements.    (Helps keep

compilers happy. My editor snuck them in, honest!)

o Advice to subtract .06 seconds for digital backtracking is deleted.   This

was the result of a bug, the deletion of which is documented    above.

****** CHANGES IN VERSION 2.1 ******

o CONTINUOUS TIME: Uses closed-form solutions on last coast time interval.

Without this feature, time is discrete, and intervals must be short. With

this, intervals can be wider, taking better advantage of the R-K algorithm.

o Default coast time interval changed from .005 seconds to .01 seconds for

faster run-time at virtually no accuracy cost. (DELTAT0 can be changed

back by the user, if desired.)

o Default tolerance (NEWTTOL) for backtracking is 100 times more exacting.

this is made possible by continuous time.

****** CHANGES IN VERSION 2.2 ******

o FIXED INSUBSTANTIAL BUGS IN CONTINUOUS TIME ROUTINE AND IN MASS

INITIALIZATION.

o IMPLEMENTED ENGLISH INPUT OPTION (#define INUNITS).

o ALLOWED LAUNCHER > SEA LEVEL (Set #define ALT0 to launch site altitude in

meters).

o Fixed bug in BACKTRAK_STAGE routine that would bomb on a single stage

rocket

after an upper stage in a multistage rocket was backtracked.

o Header records in the output file are preceded by '#' to make them

gnuplot compatible.

****** CHANGES IN VERSION 3.0 ******

o Full Fehskens-Malewicki treatment at each interval

o Variable intervals in coast phase.

o Coast phase interval set to 0.1 seconds if SPEED < MACH 0.8  or

SPEED > MACH 2.0. (Speed out of divergent range)

o Coast phase interval set to .01 seconds otherwise.

(Note that with FM, intervals should not be too short, since machine

precision is more limiting to accuracy than calculational precision after a

point.)

** VERSION 3.1 **

CONTINUOUS TIME EXTENDED TO STAGING DELAYS. THE DEFAULT NUMBER

OF INTERVALS IN THE BOOST PHASE REDUCED FROM 500 TO 400.

** VERSION 3.2 **

FIXED BUG IN DRAG PROFILE HANDLING PROCEDURE. IMPLEMENTED COLOR

FOR TURBO DOS. ALSO IMPLEMENTED TURBODOS SWITCH, WHICH SWITCHES

BETWEEN DOS (#define TURBODOS 1) AND UNIX OR GENERIC

(#define TURBODOS 0)

** VERSION 3.3 **

OUTPUT FILE FIELDS ARE LABELED. OUTPUT FILE UNITS ARE DETERMINED

BY INUNITS OPTION. PROGRAM IS BULLET-PROOFED.

** VERSION 4.0 **

Compiles in ANSI C standard. (Since there are so many C's out there,

and since UNIX has no standards, and isn't unique as an operation

system, I had to do *something*.)  If you have trouble compiling,

look for a switch on your compiler that supports the ANSI standard.

Supports screen clearing on screens/terminals that obey ANSI escape

sequences. This includes VTxxx terminals, where xxx >= 100; it also

includes IBM PC compatibles with ANSI.SYS installed in the config.sys

To try this, set ANSI to 1, and set TURBODOS and UNIX to 0.

Previous versions did not work correctly when a form factor was input

instead of a drag coefficient (a widely ignored feature). That bug is

fixed in 4.0. (Metric form factors (shape constants) computed by the

system were correct.)

The mass optimization routine is tuned and tweaked. Note that for

many rockets, the altitude/mass curve is very flat around the maximum.

Noticible changes in mass will often produce negligible changes in

altitude. Mass optimization is done through a digital approximation.

Version 5.0

Contains Warren Massey's updates to include a configuration file, and

to set English/metric units at run time.

Version 5.1

Contains a fix to make file names case-sensitive if desired.

Contains a fix for an re-entry anomaly with backtracked Cd's.

Contains true screen clearing for UNIX machines through the

system clear function or through the curses library.