AIRSTART.C

Version 1.1

30 July 1995

AUTHOR:                         Larry Curcio

(Currently: Curcio@Telerama.lm.com)

PURPOSE:

Produces a single interpolated thrust curve from 1 or more

piecewise-linear, coarse thrust curves. If more than 1 curve is

specified, then the interpolated curve is the sum of the input

curves. Each input curve may be assigned a time offset (ignition

delay), to represent airstart configurations. Each input curve

may also be assigned a multiplier, to represent simultaneous

ignition of several thrust curves. The multiplier need not be

integral. This feature may be used to mimic thrust curves from

motors with impulses varying from the mean - perhaps by one or

two standard deviations. To enhance the usefulness of airstart for

this purpose, version 1.1 allows the entry of a time multiplier, which

may also be non-integral. This feature allows you to stretch or shrink

the thrust duration to model documented variabilitiy.

Resulting thrust curves are compatible with DIGITRAK or SIMLABV5.

Suitable curves for RASP.C may be obtained by transcribing, in

compatible format, points at the time values in the component

curves. Since the number of intervals in the component curves is

small, the transcription should be easily done. The resulting

curves can also serve as piecewise linear (uninterpolated)

DIGITRAK curves. Future versions of this program will generate

such curves automatically upon request.

EXAMPLE PROBLEM #1:

Represent an ensemble of motors consisting of

1) A cluster of 3 D12's and 3 C6's ignited at liftoff; and

2) A cluster of 3 D12's ignited 1.1 seconds after liftoff

In the interaction below, answers from the keyboard are enclosed

in square brackets [].

Output File Name:[fred.eng]

A Homogeneous Cluster is a Collection of Identical Motors Ignited

Simultaneously

How Many Homogeneous Clusters? [3]

Enter Motor File Name <1> [MJED12.ENG]

Enter Number of Motors in Cluster[3]

Enter Ignition Delay [0]

Enter Motor File Name <2> [MJEC6.ENG]

Enter Number of Motors in Cluster[3]

Enter Ignition Delay [0]

Time Multiplier for Cluster [1]

Enter Motor File Name <3> [MJED12.ENG]

Enter Number of Motors in Cluster[3]

Enter Ignition Delay [1.1]

Time Multiplier for Cluster [1.0]

The program writes a file called "fred.eng," which contains an

interpolated curve that is the sum of the component curves,

displaced by the specified ignition delays.

EXAMPLE PROBLEM #2

Create a SIMLAB - compatible thrust curve from the coarse

DIGITRAK curve, MJEC6.ENG.

Since AIRSTART always produces interpolated curves, we need enter

only a single homogeneous cluster consisting of 1 C6 curve. Thus:

Output File Name:[INTERPC6.eng]

A Homogeneous Cluster is a Collection of Identical Motors Ignited

Simultaneously

How Many Homogeneous Clusters? [1]

Enter Motor File Name <1> [MJEC6.ENG]

Enter Number of Motors in Cluster[1]

Enter Ignition Delay [0]

Time Multiplier for Cluster [1.0]

The program writes a file called "INTERP.C6," which contains an

interpolated C6 curve.

EXAMPLE #3:

Create a curve to represent an MRCB6 that delivers impulse

2 standard deviations above its mean.

NOTE: This is not a perfect representation of such a scenario,

because the exact process is not uniquely specified in the

problem. The extra impulse could be a result of too much

propellant, for example. In this case, the burning time would be

extended by an approximately proportional amount. The extra

impulse could also be the result of a motor's being left in the

sun before launch, or it could come from a slightly small (or

fouled) nozzle throat. In either of these least two cases, the

burning time would be below average. This method doesn't change

burning time at all, but rather changes thrust. For motors with

short burning times, there is generally little difference in

resulting altitude among the various scenarios. Exceptions to

this generalization can occur when the rocket very light(well

below optimal mass) or when the rocket is very heavy (well above

optimal mass - but only if the difference in burning times among

the various scenarios is substantial. In most cases, this is a

reasonable representation for altitude estimates.

The Standard deviation of the impulse for any NAR - certified

motor can be looked up in the NAR S&T tables. In the case of the

MRC B6, 2 standard deviations represent about 6.1% of the mean

impulse. The standard curve delivers mean impulse, so we can

represent the new motor as a cluster of 1.061 standard MRC B6

motors.

Output File Name:[INTERMB6.eng]

A Homogeneous Cluster is a Collection of Identical Motors Ignited

Simultaneously

How Many Homogeneous Clusters? [1]

Enter Motor File Name <1> [MJMB6.ENG]

Enter Number of Motors in Cluster[1.0601]

Enter Ignition Delay [0]

Time Multiplier for Cluster [1.0]

PITFALLS:

It is easy to conclude, erroneously, that the program is

compressing some of the component curves. To see this, create a

curve from two D12's, separated by a 1- second ignition delay. A

graph of the resulting curve seems to show a compressed D12 added

to an elongated D12. This is an illusion. The second curve begins

before the first curve has terminated. While both motors are

firing, the resultant curve contains the sum of their thrusts. A

dropoff occurs when the *first* curve ends; however, the pattern

in the resulting curve suggests to the eye that the *second*

curve has terminated. The impression is illusory.

There is a danger of accidentally specifying an input file when

the program requests an output file. The program does not open

the output file until all input files are read; therefore, there

is much opportunity for the user to correct the problem. It is

wise, however, to have backup files for your input curves or to

reference them from a separate directory, while writing to the

current directory.