DESCRIPTION:

DPROFILE.C generates a disk file of predicted drag coefficients

for speeds varying between 0 and 1000 m/sec at 1 m/sec intervals.

These files, which I call drag profiles, can be used by DIGITRAK

to generate performance predictions for model rocket

configurations. The program also generates and displays analyzed

Cd predictions for a number of velocities.  In addition to drag

coefficients, DPROFILE.C generates center of pressure (CP)

locations, and handles elliptical fins. Cd predictions are

adjusted for transonic/supersonic speeds by methods described in

Mandell, Caporasso and Bengen, _Topics in Advanced Model

Rocketry_. CP values are purely subsonic.

DPROFILE will accept rocket measurements in inches or in

centimeters. DIGITRAK, though it requires drag profiles in m/sec,

will accept all other data in English or metric units. DIGITRAK

will also display predictions in English or metric units.

GENERAL DISCLAIMER:

This program is supplied as-is. No warranty of any kind is made,

including but not limited to the implied warranties of

merchantability and suitability for a particular purpose. Most

importantly, it is unwise to base decisions concerning life, limb

or property on simulation programs like DPROFILE or DIGITRAK. The

author cannot be responsible for damages incurred through the use

of this program.

More specifically, semi-empirical Cd prediction is sometimes very

accurate, and sometimes very inaccurate. In addition, drag

coefficients can change abruptly, even discontinuously, under

circumstances unforseen by the theorist, the programmer, the

user, any two of the above, or all three.

Also, it is difficult to verify the accuracy of Cd computations

without a wind tunnel because:

1) Measured Cd values must be collected from several flights of

the same kind of rocket, since Cd values are very sensitive to

measurement errors and motor performance variation. Values from

individual flights are virtually useless.

2) The range of subsonic Cd values for most streamlined model

rockets is very narrow: generally between 0.4 and 0.85. Since

most predicted Cd's fall in this range too, predictions may often

be close by accident;

3) Because of point 2), many different kinds of rockets must be

constructed to establish a reliable correlation between predicted

and measured values. For example, notice that the value 0.6 is

going to be close most of the time, but it will be totally

uncorrelated with measured values, since higher measurements will

not be attended on average by higher predictions; and

4) Data collection must take place during moments when calm

weather, free time, disposable income, available assistance, and

massive amounts of good luck all coincide.

I have done a provisional analysis, however. I present it here to

characterize the envisioned degree of effectiveness of the

program, and also its limitations.

PROVISIONAL ANALYSIS

CASE ROCKET                      MEASURED  PREDICTED

1 Airwalker                    0.6598    0.6966

2 Magnum                       0.5825    0.6472

3 Alpha                        0.5264    0.4950

4 Bertha                       0.4777    0.4951

5 Dagger                       0.8025    0.7709

6 Orange                       0.6291    0.7350

7 Nova                         0.3981    0.7087 (*)

8 Ranger                       0.5700    0.5714

9 Maniac                       0.6200    0.6087

10 Wizard                       0.6857    0.6945 (**)

(*) Outlier

(**) Used A8/A3 motors, which have high impulse variability.

Values were 0.87 on A8 and 0.54 on A3's. Combined data yielded

above statistic.

NOTE: Nova omitted in analysis below:

UNWEIGHTED LEAST SQUARES LINEAR REGRESSION OF PREDICTED

PREDICTOR

VARIABLES    COEFFICIENT    STD ERROR     STUDENT'S T       P

---------    -----------    ---------     -----------    ------

CONSTANT        0.05484       0.11141         0.49       0.6376

MEASURED        0.94005       0.17868         5.26       0.0012

R-SQUARED = 0.7981; SIMPLE CORRELATION = 0.89

NOTE: MODEL FORCED THROUGH ORIGIN

PREDICTOR

VARIABLES    COEFFICIENT    STD ERROR     STUDENT'S T       P

---------    -----------    ---------     -----------    ------

MEASURED        1.02710       0.02440        42.08       0.0000

PREDICTED VS MEASURED Cd'S

PREDICTED +--------------------------------------------------+

0.82 +                                               B  |

|                                                  |

0.77 +                                               A  |

|                      A                           |

0.72 +                                                  |

|                           A  2                   |

0.67 +                           B                      |

|                A    BB                           |

0.62 +                     A                            |

|              B B                                 |

0.57 +              A                                   |

|        B                                         |

0.52 +                                                  |

| 2      A                                         |

0.47 +                                                  |

++------+------+------+------+------+------+------++

0.47   0.52   0.57   0.62   0.67   0.72   0.77   0.82

MEASURED

A = DATA POINTS

B = FITTED LINE (Stairstepping Reference)

REMARKS:

The dataset comprises 50 points on 10 rockets, of which one was a

conspicuous outlier. Cd determination from flight tests is itself

inaccurate when done with motors below C class; however errors in

variables analysis (not presented here) demonstrates that the

predicted Cd's have more noise than the measured Cd's.

All Cd's were computed with the "PAR" surface granularity, which

I picked to represent my own modeling skill because it predicted

an observed jump in Cd for my Alpha when it is flown on a B motor

instead of an A motor. NOTE THAT EXAMPLES AT THE END ARE DONE

WITH POLISHED SURFACES.

The program performed significantly better than the mean Cd in

predicting altitudes; however it is far from perfect. (Witness

the grand miss on the NOVA.) A recent discussion of Cd prediction

on netnews.sci.aeronautics included several comments from

computaional fluid dynamics professionals that Cd prediction is

fraught with uncertainty. I agree. Nevertheless, this program is

clearly better than alternative methods.

Altitude predictions are generally much better than Cd

predicitions because drag is but one of three major forces acting

on a rocket. (Lift's effect is small in vertical flights through

calm air.) Of these, thrust and weight are reasonably

predictable, and contribute greatly to the correlation between

predicted and measured altitudes.

NEW APPROACH TO FINDING A UNIQUE CD TO SUMMARIZE A CONFIGURATION:

With most drag coefficient prediction programs, one is asked to

estimate a representative velocity beforehand. One must then find

a Cd corresponding to that velocity, and hope that the Cd will

actually characterize a flight. DPROFILE, on the other hand,

generates a drag profile, which is used by DIGITRAK to predict

flight performance. If a unique Cd is required, DIGITRAK can

backtrack the figure based on profile-predicted altitude (optical

backtracking) or on profile-predicted ascent time (temporal

backtracking). Note that the difference between optical and

temporal Cd's is a measure of how well a unique Cd summarizes a

configuration. Alternative measures are the difference in

altitude between optical and temporal backtracks, or the

difference in ascent times. The latter two measures will be small

for subsonic flights, but non-zero, and they arise from

inaccuracies in the constant-Cd model.

An example of differences between optical and temporal Cd's (in

my experience, a representative one) is the computation for the

Alpha. My optically backtracked Prediction, using a catalog-spec

A8 thrust curve (to represent the specially-made motors used by

Gregorek) is 0.446. The corresponding temporally backtracked Cd

is 0.459. In general, the optical & temporally predicted values

from profiles are within 0.02 of each other. Note that this

doesn't show the superiority of one value over another, rather it

shows the limits of the unique-Cd model. The predicted altitude

difference from the two values is about seven feet in 430 or 423:

about 1.6%  to 1.7% error.

USE OF DRAG PROFILES IN BACKTRACKING:

If a profile is available, ascent time data from an actual flight

can be backtracked in DIGITRAK using the (presently undocumented)

option, TP (temporal-profile). Optical data can be backtracked

with the option, OP. Instead of computing a drag coefficient,

DIGITRAK will compute a unique factor (I call it a trim factor)

by which all the Cd's in a drag profile must be multiplied to

summarize a flight. If the profile is reasonably accurate, the

trim factor will compensate for flight-to-flight variations in

motor performance, and in stability corrections. Altitude

estimates from ascent times should be slightly improved when high

performance rockets are being tested. Altitude predictions from

flight data of performance by the same rocket with different

payloads and motors should also be improved. As of this writing,

I have not tested these improvements. I hope to do such testing

by the next revision.

GLOSSARY:

Average Chord: The fin's area divided by its span right angle.

For trapezoidal fins, the average of the root and tip chords. See

Chord, Root Chord, and Tip Chord.

Chord: The width of the fin parallel to the rocket body. Because

fins often taper, the chord is not usually unique. We therefore

speak of various kinds of chords. See Average Chord, Root Chord

and Tip Chord.

Root Chord: The length of the fin/body joint. See Chord.

Section: A part of the rocket body with a consistent diameter. A

rocket has one or more sections. A section may have zero or more

associated sets of fins. Adjacent sections often have different

diameters, but this is not necessarily the case. (E.g.; Sections

should be defined such that they end at stage boundaries so that

you can run the configuration for each stage conveniently - see

Input Tips. Two adjacent stages with the same body diameter will

be have at least two different sections.) If adjacent sections do

have different diameters, then the upper section has an

associated transition.

Semi-Span, or Span Right Angle: This is a term from G. Harry

Stine's STABCALC program. It's just the greatest width of a fin,

measured perpendicularly from the body wall to the farthest tip.

(Even though the word, "Span," is used, no part of the body wall

is included in the measurement.)

Sweep (Sweepback): Another term from Stine: A measure of how far

the leading corner of the root is ahead of the leading corner of

the tip. The distance, parallel to the rocket body between the

leading corners of the two chords.

Tip Chord: The length of the chord at a fin's tip. If the tip is

slightly oblique, then this is the length parallel to the body

where the leading or trailing (the closer to the body) edge

corners. For elliptical fins, this is neither required nor even

defined. See Chord.

Tip Type: Read "Fin Tip Edge Type" If the fin tip edge is

rounded, sharp, or square, so-indicate. (Hoerner likes rounded

tip edges; sharp edges are virtually as good. Square edges,

recommended by Estes, are worst.) The tip you enter may not be a

real tip, however, if you are entering an irregular fin

piecewise. Choose "No Tip Effect" in this case, and see Include

Fin/Body Interference below.

Transition: A tapered body joint that connects adjacent sections

of different diameters. Transitions may taper in either

direction. A boattail is entered as a transition on the last

(lowermost) section.

INPUT TIPS:

Enter data for sections from the top section down. If your rocket

has N sections, then the top section is called the Nth. Thus, we

work from the Nth to the first.

s

The top section length includes the nose cone.

Every section contains the transition or boattail beneath it if

either exists. Therefore, include the transition length in the

length of the section above it. If your rocket has a boattail,

then enter it as the transition for the first section (the

section entered last).

Triangular fins are entered as trapezoids with tip chords of

zero.

Refer to the glossary above if you have trouble interpreting a

prompt.

Multi-Stage Rockets: There is no concept of a stage coded into

version 1.0 of DPROFILE; however multiple stage rockets can be

entered conveniently as follows:

Enter data for the entire rocket, defining sections in such a way

that  stage boundaries coincide with section boundaries. Note

that each stage may have several sections. Note also that it is

perfectly permissible for adjacent sections to have identical

diameters.

Make a drag profile for the entire configuration. Then do another

run without exiting the program. Enter the number of sections in

the configuration without the first stage. Then <Return> through

the data entry section. If a transition doesn't travel with the

stage above it, or if a previously-concealed boattail emerges

from a stage joint, you may have to change the body length and

transition entries of the last section. Otherwise, no more data

entry is required. Repeat this procedure until all stages are

represented. A separate profile is created for each stage.

E2X LAUNCH LUGS: Enter these as flat plates, with diameters of

0.8 times the actual diameter. This corresponds to Gregorek's

model for perforated disks with comparable perforation area

percentage.

ASSUMPTIONS:

Fins are rounded at the leading edge and sharp at the trailing

edge. Drag for fins with rectangular cross-sections will be

under-estimated, unless the fins are very thin. Note that flat

plastic fins for model rockets are usually thin enough to be so

represented. Rectangular cross-sections and laminar flow airfoils

are tentatively scheduled for the next release.

Neither nose cones nor transition are angled abruptly. Ogive nose

cones and elliptical nose cones are tangent to the rocket body at

the fin body joint. Secant ogive nose cones are not allowed in

this version.

Fins on supersonic models are very thin.

NOTE ON SURFACE EFFECTS:

Drag profiles for rockets with all but polished surfaces display

two curious features. Cd rises abruptly, and it remains constant

thereafter until the transonic peak. In fact, drag coefficients

can (and often do) change just this quickly. The particular

effect being modeled does not really corner, but it almost does.

The flat Cd curve after the surface effect is observed just as

strongly as it is represented, however. Teachers who wish to

demonstrate the squared velocity drag law by comparing tracked

and backtracked altitudes may wish, therefore, to cheat a bit and

to use unfinished models =:-)

I've seen this effect with one of my Alpha models. On an A8, it

yielded Cd's in the range of 0.5. On a B6, it yielded Cd's of

about 0.6. I had thought is was fin flutter. Subsequent models

didn't show the Cd jump. (My modeling technique improved...)

DEPARTURES FROM OTHER MODELS:

1) Gregorek's TR11 Gregorek drops square and cube terms in

Hoerner's formulas. For the most part, these omissions are

immaterial; however in his upgraded Alpha, the fin chord is small

enough that the a squared term in the thickness to chord ratio

makes a noticeable difference. Moreover, Gregorek uses the root

chord in his fin drag calculations; he should be using the

average chord. (Hoerner is clear on this point.) Also, Gregorek,

who was likely using a slide rule, rounded his calculations at

each stage; obviously, the computer doesn't do this. Finally, I

calculate the skin friction coefficient for the launch lug,

instead of assuming that it is constant, as gregorek does.

As a result of these differences, my Cd value for the upgraded

Alpha at 100 feet per second is 0.451, and not 0.391, as Gregorek

calculates. HOWEVER this Cd value occurs at a steep descent in

the cd/velocity curve. When the profile representing Gregorek's

configuration is run through DIGITRAK (using the GREGOREK.ENG

motor, which corresponds to A8 catalog specs, as Gregorek's

specially-made motors did, and using 38.8 grams as a launch

mass), the optically-backtracked equivalent unique Cd is 0.412.

This prediction is closer to Gregorek's measured  value (0.421)

than Gregorek's prediction (0.391) was. This example illustrates

the differences between predicted Cd's that are backtracked from

profiles, and those that are not.

My Cd value for the traditional Alpha at 100 feet/second is 0.481

compared to Gregorek's value of 0.445. Again, the difference

results mostly from Gregorek's use of root chord instead of

average chord. My optical prediction from the profile is for the

Alpha flight is .446, which is virtually identical to Gregorek's

prediction.

2) John Kallend's ROCKET4A.BAS Program  Values are very close

when launch lugs and tip effects are omitted. Indeed, I confess I

translated some of John's code into C for Dprofile. (He's

therefore entitled to a cut of all the money I get from this

project.:) John and I used different models for nose/body Cd.

Both models are from Hoerner, and both give very similar results.

John's code assumes (to be anthropomorphic for a moment) that the

rocket's nose is a tangent ogive. I allow conical or ellipsoidal

noses as well. The extra choice yields a small but substantial

difference when an ellipsoid is used; it yields an

insubstantially small difference between a cone and an ogive.

John and I use two different formulas for interference drag

between fins and body. I compensate, according to Hoerner and

Gregorek by adding to the area of each each fin an extra area

equal to the body radius times the root chord. John uses a

formula from Hoerner that predicts less of an interference

effect.

I also differ with John on the treatment of fin tips. Hoerner's

description of the fin tip edge effect is ambiguous in his latest

edition. Indeed, it is hard to determine if he is writing about

the fin tip or its edge. (English was not Hoerner's native

language.) In the previous version of his book, called

Aerodynamic Drag, he has illustrations, and there it is clear

that an edge effect is being described.

John's fin model assumes that flow is either all turbulent or all

laminar. I have extended this model in two ways.

1)If flow is partially turbulent at the maximum chord length and

uniformly laminar at the minimum chord length, I split a

triangular or trapezoidal fin laterally into a segment that has

uniformly laminar flow, and a segment where all flow is at least

partially turbulent. This gives better results than those

obtained from a model based on flow at the average chord for the

entire fin.

2)I adjust for flow that is partially turbulent, instead of

representing all the flow as turbulent.

My Reynolds numbers and kinematic viscosities are adjusted for

temperature and altitude (though the altitude is constant for the

entire profile). John's are calculated at 20 degrees centigrade

and at sea level. Differences are small.

I use Gregorek's launch lug model, and John doesn't address

launch lugs specifically, but uses Hoerner's model for

protuberances. As of this writing, the word on launch lug models

is still out. (See below)

Finally, C's floating point model is often different from BASIC's

floating point. Literal translations from one language to the

other would yield slightly different answers.

Our programs differ in many other ways (e.g.; mine accepts more

complex configurations, but John's is much easier to use), but I

list only the differences in our underlying models. For all this,

our estimates for two versions of the Alpha and for Big Bertha

(calculated without launch lugs - haven't tried with launch lugs)

all differed by less than 0.01. The uncertainty in the unique Cd

model is on the order of 0.02 for these rockets.

Launch Lug Models:

Depending on whom you read, launch lugs account for a lot of

model rocket drag or a little. Bengen is a proponent of the

former position. Gregorek seems to be in the latter position.

Hoerner has a chapter on internal flow through diffusers, which

is partially applicable; unfortunately, to get the drag

coefficient, you need a fudge factor that tells you how much

fluid flows though and how much flows around. Hoerner also has a

section on protuberances from flat or rounded bodies, and their

effect on drag. If a launch lug were solid instead of hollow,

then its total effect would be equivalent to twice the outer skin

friction drag to be expected if it were not attached to anything.

Curiously, this figure is very close to Gregorek's launch lug

model, which uses skin friction on the inner and outer surfaces

and plate drag on the frontal area. (Gregorek assumes free flow

inside and out, in contrast to Hoerner, who invokes interference

drag inside and out.)

Gregorek's model seems to work reasonably well, delivering his

stated objective, an order-of-magnitude prediction. I believe

Hoerner's interference arguments, however, and I tend toward the

belief that Gregorek's success comes from the resemblance of his

model to the solid protuberance model of Hoerner. This would

imply that the amount of internal flow in a common launch lug is

almost negligible for the purpose of Cd computation. I support

this argument with the observation that Estes E2X (tm) series

lugs, which are formed from two perforated plastic plates, are

fairly well modeled as solid plates. (As noted above, a factor of

0.8 times the diameter does better.) The resulting estimate is

only very slightly high, even though the inside diameter of the

plates is greater than that of common lugs.(An additional nicety

of Gregorek's model is that it should represent a very wide lug

reasonably well, so that the model fits at the two extremes.)

My own experiments with launch lug drag are inconclusive. I tried

measuring their effects on small rockets, powered by Estes A3

motors. A3's have such variable performance, though, that the

launch lug drag effect could not be isolated. The same is true

for B6 motors. Indeed, if you believe the NAR figure for impulse

deviation on the B6, it is worse than the A3. C6 motors are

adequate for the purpose, but they require either a larger

rocket, which would itself dwarf the lug effect, or a higher one,

which would be impossible to track. At the moment, I await good

weather to test a middlesize model with Apogee/Aerotech B7

motors. The experiments have, however, already rejected the

notion that launch lug drag is very great.

REMARKS ON EXAMPLES:

Here is some actual output from DPROFILE.C. (Note that my

modeling ability is limited, and I'm comparing modeling to

predictions for perfect models.)

The ESTES Alpha is a straightforward example. The upgraded Alpha

from ESTES TR11 shows how a boattail is handled - as a transition

in the first (last-entered) section.

The Big Bertha shows that metric input can be used, and is an

example of the ellipsoidal nosecone.  (A fin from this rocket can

be broken into two trapezoids by drawing a line parallel to the

rocket body. The area is then easily computed, and the average

chord can be obtained from it by diving by the semi-span.)

The ARCON rocket is a multi-stage example from Barrowman.

The Recruit is an example of a 6-finned, multi-transition rocket,

also from Barrowman.

I have no Cd data for the last two rockets, and I have no

independent verification of CP *except* for the last two rockets.

Notes:

Since the examples were done, I added a roundness adjustment to

the body Cd computation. Cd values may differ slightly in the

third decimal place.

The print feature used here is primarily to document your input.

I'll pretty it up in the next release.

Also since the examples were done, I added a surface type, which

I call "Par". This surface predicts my measured Cd's pretty

accurately - but it represents the finishes *I* put on *my*

rockets. The program does significantly better than a constant Cd

estimate equal to the average measured value. (See analysis

above.)

Alpha from Gregorek

(Temperature is 68.000000 degrees Fahrenheit, Input Units are in Inches)

Total Length = 10.500000, Max Diameter = 0.976000, # of Sections = 1

Nose Type= Ogive, Sharp Nose Length = 2.750000, NoseDiameter = 0.976000

For Section #1:

Diameter = 0.976000, Length = 10.500000, Surface Type = Polished

Number of Lugs = 1Lug OD = 0.170000, Lug ID = 0.150000, Lug Length = 1.000000

Number of Fin Sets for Section #1 is 1

3 Fins of Type Trapezoidal, with Polished Surface

Root = 2.250000, Tip = 1.200000, Mean Chord = 1.725000, Semi-Span = 1.600000

Thickness = 0.093000, SweepBack = 2.300000 Dist From Nose = 8.500000

Fin Tip Type = Round

Alpha from Gregorek

(Temperature is 68.000000 degrees Fahrenheit)

Cd at 100 ft/sec = 0.481383

Nose-BodyCd= 0.271038 (BaseCd= 0.063686), Fin Cd= 0.190544, Lug Cd = 0.019800

Cd at 150 ft/sec = 0.438229

Nose-BodyCd= 0.257092 (BaseCd= 0.066412), Fin Cd= 0.162567, Lug Cd = 0.018570

Cd at 200 ft/sec = 0.418245

Nose-BodyCd= 0.248387 (BaseCd= 0.068346), Fin Cd= 0.152048, Lug Cd = 0.017810

Cd at 250 ft/sec = 0.408201

Nose-BodyCd= 0.242236 (BaseCd= 0.069846), Fin Cd= 0.148688, Lug Cd = 0.017277

Overall CP = 8.935744 Inches From Nose Tip

Cd predicted from Profile with DIGITRAK is .446 at mass = 50 grams, and

CATALOG SPEC motor Gregorek's prediction = .448

Gregorek's measurement was .650. - an outlier in his data.

Cd predicted from Profile with DIGITRAK is .469 at 50 grams, STANDARD A8

My measurement: Temporal data alone yield       .526.

Optical/Temporal combined yield .480

(A8's give very variable performance)

Upgraded Alpha From Gregorek

(Temperature is 68.000000 degrees Fahrenheit, Input Units are in Inches)

Total Length = 12.000000, Max Diameter = 0.976000, # of Sections = 1

Nose Type= Ogive, Sharp Nose Length = 2.750000, NoseDiameter = 0.976000

Boat Tail Length = 1.500000, Base Diameter = 0.736000

For Section #1:

Diameter = 0.976000, Length = 12.000000, Surface Type = Polished

TransitionLength = 1.500000, Transition Base = 0.736000

Number of Lugs = 1Lug OD = 0.170000, Lug ID = 0.150000, Lug Length = 1.000000

Number of Fin Sets for Section #1 is 1

3 Fins of Type Trapezoidal, with Polished Surface

Root = 1.500000, Tip = 0.600000, Mean Chord = 1.050000, Semi-Span = 2.000000

Thickness = 0.062500, SweepBack = 0.450000 Dist From Nose = 10.000000

Fin Tip Type = Round

Upgraded Alpha From Gregorek

(Temperature is 68.000000 degrees Fahrenheit)

Cd at 100 ft/sec = 0.450968

Nose-BodyCd= 0.253296 (BaseCd= 0.026090), Fin Cd= 0.177871, Lug Cd = 0.019800

Cd at 150 ft/sec = 0.408029

Nose-BodyCd= 0.236367 (BaseCd= 0.027191), Fin Cd= 0.153092, Lug Cd = 0.018570

Cd at 200 ft/sec = 0.381557

Nose-BodyCd= 0.225626 (BaseCd= 0.027972), Fin Cd= 0.138120, Lug Cd = 0.017810

Cd at 250 ft/sec = 0.362928

Nose-BodyCd= 0.217938 (BaseCd= 0.028578), Fin Cd= 0.127713, Lug Cd = 0.017277

Overall CP = 9.582093 Inches From Nose Tip

Predicted Cd 38.8 grams with catalog spec motor (to match Gregorek's

experiment in TR11:  Cd = .4130. Gregorek's Prediction: .391.

Gregorek's measurement was .421

Big Bertha

(Temperature is 68.000000 degrees Fahrenheit, Input Units are in Centimeters)

Total Length = 52.450001, Max Diameter = 4.160000, # of Sections = 1

Nose Type= Ellipsoid, Blunt Nose Length = 6.750000, NoseDiameter = 4.160000

For Section #1:

Diameter = 4.160000, Length = 52.450001, Surface Type = Polished

Number of Lugs = 1Lug OD = 0.431800, Lug ID = 0.381000, Lug Length = 6.000000

Number of Fin Sets for Section #1 is 1

4 Fins of Type Unspecified, with Polished Surface

Root = 9.500000, Tip = 9.000000, Mean Chord = 10.408125, Semi-Span = 4.000000

Thickness = 0.317600, SweepBack = 8.000000 Dist From Nose = 42.950001

Fin Tip Type = Round

Big Bertha

(Temperature is 68.000000 degrees Fahrenheit)

Cd at  25 m/sec = 0.449151

Nose-BodyCd= 0.285301 (BaseCd= 0.061269), Fin Cd= 0.152743, Lug Cd = 0.011107

Cd at  50 m/sec = 0.424118

Nose-BodyCd= 0.261341 (BaseCd= 0.065537), Fin Cd= 0.152963, Lug Cd = 0.009814

Cd at  75 m/sec = 0.411039

Nose-BodyCd= 0.249731 (BaseCd= 0.068034), Fin Cd= 0.152114, Lug Cd = 0.009194

Cd at 100 m/sec = 0.401328

Nose-BodyCd= 0.242398 (BaseCd= 0.069805), Fin Cd= 0.150127, Lug Cd = 0.008803

Overall CP = 38.395687 Centimeters From Nose Tip

Cd Predicted with profile from DIGITRAK = .437 (C6 motor, 119 grams)

Measured Cd = .477  (from Temporal and Optical Backtracking)

Cd error = 11%;    Altitude error = 2%

(Note that Polished Values are lower bounds. YMMV!)

STAGED ROCKET EXAMPLE - PART 1: LOWER STAGE

Arcon Hi From Barrowman Lower Stage - Entire Configuration

(Temperature is 68.000000 degrees Fahrenheit, Input Units are in Inches)

Total Length = 24.000000, Max Diameter = 1.040000, # of Sections = 2

Nose Type= Ogive, Sharp Nose Length = 5.250000, NoseDiameter = 1.040000

For Section #2:

Diameter = 1.040000, Length = 20.000000, Surface Type = Polished

Number of Lugs = 0Number of Fin Sets for Section #2 is 1

4 Fins of Type Trapezoidal, with Polished Surface

Root = 1.900000, Tip = 1.050000, Mean Chord = 1.475000, Semi-Span = 2.200000

Thickness = 0.125000, SweepBack = 0.850000 Dist From Nose = 18.100000

Fin Tip Type = Round

For Section #1:

Diameter = 1.040000, Length = 4.000000, Surface Type = Polished

Number of Lugs = 1Lug OD = 0.170000, Lug ID = 0.150000, Lug Length = 2.000000

Number of Fin Sets for Section #1 is 1

4 Fins of Type Trapezoidal, with Polished Surface

Root = 2.450000, Tip = 1.250000, Mean Chord = 1.850000, Semi-Span = 3.450000

Thickness = 0.125000, SweepBack = 0.600000 Dist From Nose = 21.549999

Fin Tip Type = Round

Arcon Hi From Barrowman Lower Stage - Entire Configuration

(Temperature is 68.000000 degrees Fahrenheit)

Cd at 100 ft/sec = 1.288957

Nose-BodyCd= 0.417446 (BaseCd= 0.047692), Fin Cd= 0.847240, Lug Cd = 0.024271

Cd at 150 ft/sec = 1.136756

Nose-BodyCd= 0.391844 (BaseCd= 0.049569), Fin Cd= 0.722293, Lug Cd = 0.022619

Cd at 200 ft/sec = 1.049608

Nose-BodyCd= 0.375496 (BaseCd= 0.050901), Fin Cd= 0.652525, Lug Cd = 0.021587

Cd at 250 ft/sec = 1.007711

Nose-BodyCd= 0.363742 (BaseCd= 0.051934), Fin Cd= 0.623113, Lug Cd = 0.020856

Overall CP = 20.803230 Inches From Nose Tip

(Barrowman's CP is 20.8)

STAGED ROCKET EXAMPLE PART 2 - UPPER STAGE

DATA SAVED FROM PART 1; MERELY CHANGED # OF SECTIONS TO 1

Arcon Hi From Barrowman - Upper Stage (Lug Left on 1st Stage)

(Temperature is 68.000000 degrees Fahrenheit, Input Units are in Inches)

Total Length = 20.000000, Max Diameter = 1.040000, # of Sections = 1

Nose Type= Ogive, Sharp Nose Length = 5.250000, NoseDiameter = 1.040000

For Section #1:

Diameter = 1.040000, Length = 20.000000, Surface Type = Polished

Number of Lugs = 0Number of Fin Sets for Section #1 is 1

4 Fins of Type Trapezoidal, with Polished Surface

Root = 1.900000, Tip = 1.050000, Mean Chord = 1.475000, Semi-Span = 2.200000

Thickness = 0.125000, SweepBack = 0.850000 Dist From Nose = 18.100000

Fin Tip Type = Round

Arcon Hi From Barrowman - Upper Stage (Lug Left on 1st Stage)

(Temperature is 68.000000 degrees Fahrenheit)

Cd at 100 ft/sec = 0.719100

Nose-BodyCd= 0.367379 (BaseCd= 0.051607), Fin Cd= 0.351721, Lug Cd = 0.000000

Cd at 150 ft/sec = 0.649785

Nose-BodyCd= 0.345582 (BaseCd= 0.053676), Fin Cd= 0.304203, Lug Cd = 0.000000

Cd at 200 ft/sec = 0.595528

Nose-BodyCd= 0.331720 (BaseCd= 0.055143), Fin Cd= 0.263808, Lug Cd = 0.000000

Cd at 250 ft/sec = 0.568476

Nose-BodyCd= 0.321784 (BaseCd= 0.056281), Fin Cd= 0.246692, Lug Cd = 0.000000

Overall CP = 17.845272 Inches From Nose Tip

(Barrowman's Cp is 17.9)

TRANSITIONED ROCKET EXAMPLE

Recruiter From Barrowman

(Temperature is 68.000000 degrees Fahrenheit, Input Units are in Inches)

Total Length = 18.000000, Max Diameter = 1.340000, # of Sections = 3

Nose Type= Ogive, Sharp Nose Length = 1.800000, NoseDiameter = 0.550000

For Section #3:

Diameter = 0.550000, Length = 3.350000, Surface Type = Polished

TransitionLength = 0.800000, Transition Base = 0.908000

Number of Lugs = 0Number of Fin Sets for Section #3 is 0

For Section #2:

Diameter = 0.908000, Length = 6.250000, Surface Type = Polished

TransitionLength = 1.000000, Transition Base = 1.340000

Number of Lugs = 0Number of Fin Sets for Section #2 is 0

For Section #1:

Diameter = 1.340000, Length = 8.400000, Surface Type = Polished

Number of Lugs = 1Lug OD = 0.170000, Lug ID = 0.150000, Lug Length = 2.000000

Number of Fin Sets for Section #1 is 1

6 Fins of Type Trapezoidal, with Polished Surface

Root = 1.750000, Tip = 0.900000, Mean Chord = 1.325000, Semi-Span = 1.560000

Thickness = 0.125000, SweepBack = 0.950000 Dist From Nose = 16.250000

Fin Tip Type = Round

Recruiter From Barrowman

(Temperature is 68.000000 degrees Fahrenheit)

Cd at 100 ft/sec = 0.551576

Nose-BodyCd= 0.260785 (BaseCd= 0.065649), Fin Cd= 0.276171, Lug Cd = 0.014620

Cd at 150 ft/sec = 0.506958

Nose-BodyCd= 0.248549 (BaseCd= 0.068308), Fin Cd= 0.244784, Lug Cd = 0.013625

Cd at 200 ft/sec = 0.465900

Nose-BodyCd= 0.240878 (BaseCd= 0.070194), Fin Cd= 0.212019, Lug Cd = 0.013003

Cd at 250 ft/sec = 0.439159

Nose-BodyCd= 0.235442 (BaseCd= 0.071658), Fin Cd= 0.191154, Lug Cd = 0.012563

Overall CP = 15.590468 Inches From Nose Tip

(Barrowman's CP is 15.6)

BIBLIOGRAPHY

Hoerner, S. F. _Fluid Dynamic Drag_ (Theoretical, Experimental and Statistical

Information) 1965 Hoerner Fluid Dynamics, Vancouver, WA

Gregorek, Gerald M. _Aerodynamic Drag of Model Rockets_ 1970 Estes Industries,

Inc, Colorado (Technical Report TR11)

Barrowman, J. _Calculating the Center of Pressure of a Model Rocket_ 1970

Century Engineering Company, Phoenix, AZ

Mandell, G.K.; Caporaso, G.J.; and Bengen, W.P. _Topics in Advanced Model

Rocketry_ 1973 MIT Press, Canbridge, MASS.

Curcio, L.M. _Measuring and Predicting Rocket Performance from Ascent Time Data_

Paper presented at NARAM 35 R&D competition

Kallend, J. ROCKET4A.BAS - computer program

Catt, Martin CPRESS.BAS - computer program