Small Altimeter for Model Rockets

by Hervé Cousin

Introduction

This altimeter is based on the altimeter published in RADIO-ELECTRONICS, October 1990 on page 37.  However, the size and the  weight of the device seemed quite clumsy. Therefore, the following modification were carried out:

-	The amplified signal from the pressure sensor (SCX15AN from Sensym, Sunnyvale, CA 94089) is converted by the means of a AD converter (ADC) instead of  reading a frequency of the VCO.

-	The "brain" of the device is the "BASIC-Stamp" of  Parallax, Rocklin, CA 95765 which is a BASIC-programmable microcontroller with 8 I/O ports ("The BASIC Stamp", Manual version 1.1, Parallax, Inc., 1994) instead of the RCA 1802 microprocessor.

-	The altimeter program is edited on a PC, and subsequently downloaded to the program memory (EEPROM) of the "BASIC-Stamp".

-	The data-memory is a EEPROM with 2048 x 8 bit instead of a RAM with the same memory size.

In order to take full advantage of  the ADC, the offset of the analog sensor signal and the reference input of the ADC are adjustable. Depending upon the altimeter program, the acquisition rate is about 10 8bit values per second, i.e. a total acquisition time of about 205 seconds.

The acquired values are read out by a small LCD terminal, which is controlled by another "BASIC-Stamp". Alternatively, a portable computer may be used as data-viewer.

The resulting altimeter weighs ca. 45g, without battery, and has the following dimensions, 13.5 cm x 4.1 cm x 1.5 cm (without battery). Test runs showed a resolution of about 5 m, up to ca. 1250 m. The accuracy and precision are to be determined in further experiments.

Theoretical Considerations (short)

The international altitude-formula is defined as:

p=1.013⋅1−0.0065⋅h2885.255bar

whereas p is the resulting pressure in bar (1 bar = 100 kPa= 105Nm-2) at the altitude h in meters (the pressure at sea-level (0 m) has been defined as 1.013 bar). The curve of this function has a negative slope and is not linear. However, the maximal error compared to the linearized values of the function (up to about 1250 m) is better than ± 5 m, and therefore equal to or smaller than the resolution. In this case one may admit the pressure as a linear function of the altitude.

Circuit description

The complete system consists of the altimeter board (containing the first Basic-Stamp) that is fit into the rocket and a LCD terminal (containing the second Basic-Stamp) which stays on the ground. The LCD terminal board is connected to the altimeter board, after the recovery of the rocket. The acquired values are then read out the EEPROM and displayed on the LCD display. The PC programming port of the BASIC-stamp can be alternatively used to read out the values on a PC.

The altimeter board (Figure 1):

The pressure sensor is a piezoresistive IC in a strain-gauge bridge configuration. This sensor has a vacuum reference cavity and measures, therefore, the absolute barometric pressure. The output voltage of the sensor is amplified with operational amplifiers IC2a-c resulting in a positive voltage inversely proportional to the barometric pressure res., to the altitude. The reference voltage is adjusted by a voltage divider and a unity-gain follower (IC2d). When connected to the LCD-terminal, the value are read out of the EEPROM, one-by-one by depressing (SW1). The operation mode is set by (SW2), i.e. ON = read mode, OFF = acquisition mode. The Basic-stamp module is connected via a 12 pin (inline) connector to the altimeter board.

The I/O-lines are connected as follows:

Basic-Stamp		-	Altimeter Board			(Designator on Figure 1)

Pin 1			-	Serial Output 			(S_OUT)

Pin 2			-	Mode Switch 			(Mode)

Pin 3			-	Step Button 				(Step)

Pin 4			-	A/D-converter data 		(ADCData)

Pin 5			-	A/D-converter clock 		(ADCCLK)

Pin 6			-	A/D-converter chip select 	(ADCCS)

Pin 7			-	EEPROM data 			(EEData)

Pin 8			-	EEPROM clock 			(EECLK)

Pin 9			-	not connected (nc)

Pin 10			-	Ground 				(GND)

Pin 11			-	+ 5V output 				(+5V)

Pin 12			-	+V Battery 				(+V_IN)

Figure 1	Altimeter Board (Consult Table 1 for the components)

The LCD Terminal (Figure 2):

This Board basically connects the other Basic-stamp with the LCD Display unit. The variable resistor sets the LCD voltage to obtain the best contrast and viewing angle. The Display unit is set to 4-bit mode, and therefore only the 4 upper bits are to be connected to the Basic-stamp. Because there will be only writing operations to the display, the R/W pin is connected to Ground (GND).

The PCBs have been designed with the freeware EASYTRAX version 2.06 for DOS from Protel Technology Pty. Ltd. The package of this doc-file includes the following files:

ALTI.PCB		Easytrax *.PCB file of the altimeter board

LCD.PCB		Easytrax *.PCB file of the LCD terminal board

ALTI.PS		Postscript file of the altimeter board

LCD.PS		Postscript file of the LCD terminal board

The labels on the PCB (Copper-layer side) should appear flipped (mirror image). If somebody may improve the design, I will be glad to get some feedback. The best result one may get with a laser printer with a new toner cartridge on foil with protective paper-sheet.

Figure 2	LCD Terminal Board (Consult Table 1 for the components). The number at the module represents the pin-number of the connector (see manual or data sheet).

Construction

The assembly of the two boards are straight forward. Please note the 2 bridges on the altimeter and 1 bridge on the LCD terminal board.

The sensor is screwed on the PCB. The BASIC-Stamp has to be carefully fixed on the PCB, in order to withstand the high acceleration of the rocket. Furthermore, the solder side of the BASIC-Stamp should not make any contact to one of the bridges on the main board (an adhesive tape should cure that problem). Figure 4 shows a possible solution. The connector is slightly bent upwards, and the Basic-Stamp is hold back with 2 hooks made of 0.2 mm Copper-sheet (glued with Epoxy-glue to the altimeter board).

The resistors R13, R14, and R15 are soldered on the solder-side; there is no holes for R15, has to be therefore soldered directly on the tracks (EEPROM data-line to +5 V), near the 12 pin connector of the Basic-Stamp.

Figure 3	Construction Hints

Testing and Calibration

The LCD terminal may be simply tested by downloading the LCD-program (Listing 2) to the BASIC-Stamp. The battery should be connected to the +Vin (NOT to +5V), and ground. Then one need just to send any ASCII characters to the LCD-terminal (from a simple terminal program on a PC, with the following communication parameters: 1200 Baud, 8 bit, no parity, 1 stop-bit). A 22 kW resistor should be connected between the serial input of the LCD terminal board and the serial output of the computer. the first value will appear as "Ground = XXXXm", the second "Apogee = XXXXm", and the following values as "YYYY = XXXXm" (YYYY = 0002 to 2047).

The altimeter board needs some more attention. Connect the battery to the +Vin (NOT to +5V!) and ground, and set the output of the IC2-c to a value of about 0.8 V and the value of the output of IC2-d (VRef) to about 0.9 V. The next step is to download the program AD_TEST.BAS and connect the serial output of the altimeter to the serial input of the LCD terminal.

The terminal will immediately start to display the values (about 150 m, by steps of 5 m) from the altimeter. Move upwards a specific number of meters (e.g. in a tall building). If the difference in altitude is not correct, modify the incremental value (in this program set to 5 m), whereas the maximum altitude is given by 1 byte for the A/D converter, i.e. 255 steps.

If you wish to fly higher than about 1250 m, then increase the above values on the IC2 in consequence. Calibrate again and set the correct increment.

Operating Instructions

The installation of the altimeter in a rocket and the launching of the rocket has to be done applying all standard safety rules.

Since I have done no launch carrying the altimeter as a payload, I only listed some general considerations (again, any feedback is highly appreciated):

My first test will be:

-	Carrier: "The Shadow" from Estes (D - E powered).

-	Altimeter installed in the nose-cone in upright position, sensor placed up.

- 	The altimeter is turned on with a miniature-reed contact, closing (= ON) when removing the magnet (Figure 4).

Figure 4	On/Off Switch and installation in the nose-cone.

Operation:

1.	Set Mode-switch to OFF (altimeter board).

2.	Tape  the magnet to the nose-cone and insert the battery.

3.	Prepare rocket for launch (including engine and ignitor).

4.	Remove the magnet.

5.	Start with the count-down and ignite.

6.	Recover the rocket.

7.	Connect to the LCD terminal.

8.	Set Mode-switch to ON (altimeter board).

9.	Turn ON LCD terminal.

10.	Depress Step-pulse-switch to get the values.

Conclusion

The weigh of the altimeter may be lowered by improved construction and layout of the PCB. The Basic-Stamp may be included on the main-board (the chips are separately available).

This type of A/D converter is available with up to 8 analog input channels (ADC0831, 0832, 0834, 0838); therefore, also other analog-values may be read and stored in the EEPROM. A new Basic-Stamp with 4x more program memory has been announced, thus many more possibilities.

Nowadays, many new chips are available, e.g. intelligent digitizing signal conditioner chip (e.g. AD1B60), communication chips, coder and decoder, etc., resulting in a huge amount of possible configurations (multichannel data logger or transmitter, remote control, etc.).

Any comments/suggestions:

e-mail:		cousin@inorg.chem.ethz.ch

fax:			++41-1-632-1090

phone:		++41-1-632-4472

ordinary mail:	Hervé COUSIN

ETHZ

Anorganische Chemie

Universitätstrasse 6

CH-8092 Zürich

Switzerland

Disclaimer:

Since I have no control over the use to which the information in this paper is put, I assume no liability for any damages resulting from its use and no responsibility for the suitability of this device  for any application. The different devices in this paper may include any patents, copyrights or other rights.

Table 1	Part List

Altimeter Board:

|  |  | Resistors (1/4 watt, 5%, unless otherwise noted) |
| --- | --- | --- |
| R1 | 100kW | 25 turn variable |
| R2 | 2kW | 25 turn variable |
| R3, R4 | 10kW | 1/8 watt, 1%, metalfilm |
| R5 | 8.06kW | 1/8 watt, 1%, metalfilm |
| R6 | 2kW | 1/8 watt, 1%, metalfilm |
| R7 | 270kW | 1/8 watt, 1%, metalfilm |
| R8 | 1MW | 1/8 watt, 1%, metalfilm |
| R9 | 100kW | 1/8 watt, 1%, metalfilm |
| R10 | 20kW | 1/8 watt, 1%, metalfilm |
| R11 | 6.8kW | 1/8 watt, 1%, metalfilm |
| R12 | 1kW | 1/8 watt, 1%, metalfilm |
| R13, R14, R15 | 10kW |  |
|  |  | Condensator |
| C1 | 22mF | Tantalum |
| C2 | 0.1mF | Foil, monolithic |
|  |  | Integrated Circuits |
| IC1 | SCX15AN | P-Sensor (absolute pressure), Sensym, Sunnyvale, CA |
| IC2 | 24LC16B | EEPROM (16k), Microchip Technology, Inc., Chandler, AZ |
| IC3 | LM324N | Quad operational amplifier, different suppliers |
| IC4 | ADC0831 | A/D converter (serial I/O), National Semiconductors |
|  |  | Miscellaneous |
| 6 - 9V Battery, 12 pin inline connector (female, 90° angle), IC sockets, Battery clip, |  |  |
| SW1 = Pulse switch, SW2 = DIP-switch (2 pin), 2 x screw M3x15mm, solder, etc. |  |  |

BASIC-Stamp Module:	"German version", with 12 pin inline connector (male) from Wilke Technology GmbH, P.O.Box 1727, D-52018 Aachen, Germany, Fax: ++49 (241) 15 84 75

LCD Terminal Board:

|  |  | Resistor (1/4 watt, 5%) |
| --- | --- | --- |
| R1 | 5kW | 25 turn variable |
|  |  | Condensator |
| C1 | 22mF | Tantalum |
|  |  | Miscellaneous |
| 9V Battery, 2 x 14 pin inline connector (female), 14 pin inline connector (male), and 3 |  |  |
| pin inline connector (female), IC sockets, Battery clip, solder, etc. |  |  |

BASIC-Stamp Module: "Original version" from Parallax, Inc., 3805 Atherton Road, 102, Rocklin, CA 95765, USA (Representative in Europe: Wilke Technology GmbH) LCD Display Module:	Vikay Alphanumeric Dot Matrix Module VK2216 from VGI Inc., 7246 Sharon Drive, Suite E, San Jose, CA 95129, USA, Fax: (408) 446-3840 (Vikay Europe Ltd, Kwai Chung, N.T., Hong Kong, Fax: 0-29280)

Figure 5	Connection Diagrams. For additional Information, consult the respective databooks.

Program Listing 1	Altimeter

' PROGRAM: ALTIM1.BAS

'

'16. December 1994

'

'Altimeter program. Pressure values are read with an ADC 0831 AD converter and stored in 'a 24LC16B EEPROM (2Kx8). The values are transmitted, after the acquisition one-by-one to 'the LCD terminal.

'

symbol	S_OUT	=	0	'Serial output pin.

symbol	MODE	=	1	'Mode switch. 0 = transmit, 1 = acquire.

symbol	ST		=	2	'Step button (next value).

symbol	add		=	pin3	'AD converter data input pin.

symbol	ADCLK	=	4	'AD converter clock output pin.

symbol	ADCS		=	5	'AD converter chip select output pin.

symbol	EECLK	=	6	'EEPROM clock output pin.

symbol	EED_N	=	7	'EEPROM data I/O pin number.

symbol	eed		=	pin7	'EEPROM data I/O pin.

symbol	data	=	b0		'EEPROM data in/out register.

symbol	adata	=	b1		'AD converter input register.

symbol	addr	=	w1		'Address register.

symbol	bu	=	b4		'Button variable.

symbol	i	=	b5		'For-loop index.

symbol	mval	=	b6		'Maximum altitude value (lowest pressure).

symbol	rdbyte	=	%10100001	'EEPROM read code.

symbol	wrbyte	=	%10100000	'EEPROM write code.

setup:	dirs = %11110001			'Set MODE, ST, add as input.

pins = 255				'Set all pins to 1.

mval = 255				'Set start value for mval.

main1:button MODE,0,255,255,bu,1,main2'MODE switch turned on? main2.

for addr = 0 to 2047			'Increase memory location.

'Get a value from AD converter.

conv:	low ADCLK				'Put clock line in starting state.

low ADCS				'Select ADC.

pulsout ADCLK, 1			'10 us clock pulse.

adata = 0				'Clear data.

for i = 1 to 8				'Eight data bits.

adata = adata * 2			'Perform shift left.

pulsout ADCLK, 1			'10 us clock pulse.

bit8 = add				'Put bit in LSB of data.

next i					'Do it again.

high ADCS				'Deselect ADC when done.

gosub wr_ee				'Write this value to the EEPROM.

if mval < adata then m11		'If mval is bigger then set mval

mval = adata				'equal to adata.

m11:	next addr				'Next memory location.

addr = 1				'Store mval

adata = mval				'in memory

gosub wr_ee				'location = 1.

main2:button MODE,0,255,255,bu,0,main2'Waite until MODE switch turned on.

for addr = 0 to 2047			'Increase memory location.

main3:button ST,0,254,250,bu,0,main3	'Wait until ST button depressed.

'Get a value from EEPROM.

rd_ee:	low EED_N				'Set start

low EECLK				'condition.

data = b3*2|wrbyte			'Or write code with 3 MSBits of

gosub out				'the address, and send to EEPROM.

data = b2				'Send LSByte of the address

gosub out				'to the EEPROM.

high EECLK				'Set stop

pulsout EED_N,10			'and restart

low EECLK				'condition.

data = b3*2|rdbyte			'Or read code with 3 MSBits of

gosub out				'the address, and send to EEPROM.

data = 0				'Get the

gosub in				'value from the EEPROM

high EECLK				'Set stop

high EED_N				'condition.

serout s_out,T1200,(data)		'Transmit the value.

next addr				'Next memory location.

end					'Program end.

wr_ee:low EED_N				'Set start

low EECLK				'condition.

data=b3*2|wrbyte			'Or write code with 3 MSBits of

gosub out				'the address, and send to EEPROM.

data=b2				'Send LSByte of the address

gosub out				'the address, and send to EEPROM.

data=adata				'Send the AD converted value

gosub out				'to the EEPROM.

high EECLK				'Set stop

high EED_N				'condition.

return					'

out:	output EED_N			'Set EEPROM data pin to output.

for i = 1 to 8				'Send 8 bit.

EED = bit7				'Set EEPROM pin to 7. bit of data.

pulsout EECLK,10			'10 us clock pulse.

data = data*2				'Shift data left.

next i					'Next bit.

low EED_N				'Set EEPROM data pin low.

pulsout EECLK,10			'Send acknowledge 10 us clock pulse.

return					'

in:	input EED_N			'Set EEPROM data pin to input.

for i = 1 to 8				'Send 8 bit.

data = data*2				'Shift data left.

high EECLK				'Set clock pin high.

bit0 = EED				'Read EEPROM data pin to 1.bit of

low EECLK				'data and set clock pin low.

next i					'Next bit.

return					'

Program Listing 2	LCD Terminal

'PROGRAM: LCD.BAS

'

'December 8, 1994

'

'LCD terminal for the altimeter. Data from the altimeter are sent to this terminal and 'displayed on a LCD Display VIKAY VK2216.

'

symbol	s_in	= 	0	'Serial data input pin.

symbol	RS	=	2	'Register select pin, 0 = instruction.

symbol	E	=	3	'Enable pin, 1 = enabled.

symbol  	data	=	b0	'Data sent to the LCD terminal.

symbol	char	=	b1	'Character sent to LCD.

'					b2	 Reserved (buffer)

symbol	a	=	b3	'Offset.

symbol	i	=	b4	'Index.

symbol	addr	=	w3	'Number for the values.

symbol	dec	=	w4	'Decimal increment register.

symbol	d_reg	=	w5	'Data register.

symbol	DB4	=	pin4	'LCD data lines (4 bit bus).

Symbol	DB5	=	pin5

Symbol	DB6	=	pin6

Symbol	DB7	=	pin7

'Set up the Stamp's I/O lines and initialize the LCD values.

'

setup:	let dirs = %11111100		'2 inputs, 6 outputs.

'Store "Ground" and "Apogee" in EEPROM, starting at address 0.

EEPROM 0,("G","r","o","u","n","d","A","p","o","g","e","e")

pause 200				'Wait 200 ms for LCD to reset.

'Initialize the LCD in accordance with VIKAY instructions for 4-bit

'interface.

'

i_LCD:let pins = %00110000		'Set to 8-bit operation.

pulsout E,1				'Send data three times

pause 10				'to initialize LCD.

pulsout E,1

pause 10

pulsout E,1

pause 10

let pins = %00100000		'Set to 4-bit operation.

pulsout E,1				'Send above data three times.

pulsout E,1

pulsout E,1

let char = %00101000		'Set up LCD in accordance with

gosub wr_LCD			'VIKAY technical description.

let char = %00001100

gosub wr_LCD

let char = 1

gosub wr_LCD

high	RS				'Prepare to send characters.

'Main program loop: receive data and display data on LCD.

'

main:	for addr=0 to 2047

serin s_in,t1200,data	 	'Receive data.

low RS				'Clear display.

char = 1				'     "

gosub wr_LCD			'     "

high RS				'     "

a = 2					'Display increment.

disp:	branch addr,(alt0, altm)		'1. value is Ground, 2. value is

a = 0					'Apogee.

goto altn				'Other values (2 - 2047).

alt0:	for i=0 to 5				'Display "Ground",

read i, char				'stored in the EEPROM.

gosub wr_LCD

next i

goto calc				'Goto the calculation routine.

altm:	for i=6 to 11				'Display "Apogee".

read i, char				'stored in the EEPROM.

gosub wr_LCD

next i

goto calc				'Goto the calculation routine.

altn:	let dec=1000				'Display number of the value.

for i=0 to 3

let char=addr/dec//10+$30	'Get N*10Ei and convert it into

let dec=dec/10			'ASCII character; $30 = "0",

gosub wr_LCD			'and display N.

next i

'Calculate altitude from received data.

'

calc:	for i= a to 4				'Display space and "=".

lookup i,(" "," "," ","="," "),char

gosub wr_LCD

next i

data = 255 - data			'Inverse the value.

let d_reg=0+5*data			'Function for the altitude

'calculation, alt = a + b*data.

let dec=1000

for i=0 to 3

let char=d_reg/dec//10+$30	'Get N*10Ei and convert it into

let dec=dec/10			'ASCII character; $30 = "0".

gosub wr_LCD			'and display N.

next i

let char="m"				'Add "m" to the

gosub wr_LCD			'Display.

next addr

end

'Write the ASCII character in char to LCD.

'

wr_LCD:	let pins = pins & %00000100	'Clear 4-bit data bus.

let b2 = char & %11110000			'Put low nibble of b1 into b2.

let pins = pins | b2				'OR the contents of b2 into pins.

pulsout E,1					'Blip enable pin.

let b2 = char*16				'Put high nibble of b1 into b2.

let pins = pins & %00000100		'Clear 4-bit data bus.

let pins = pins | b2				'OR the contents of b2 into pins.

pulsout E,1					'Blip enable pin.

return

Program Listing 3	Test Programs (Poorly documented)

'PROGRAM: AD_TEST.BAS

'

symbol	AD	=	pin3

symbol	CLK	=	4

symbol	CS	=	5

symbol	S_O	=	0

symbol	data	=	b0

symbol	i	=	b2

setup:	let pins = 255			'Pins high (deselect ADC).

let dirs = %11110111	'CLK, CS outputs; AD input.

loop:	gosub conv			'Get the data.

debug %data,data

data=255-data

debug %data,data

serout S_O,t1200,(data)	'Send data.

pause 2000			'Wait 0.2 seconds

goto loop			'Do it forever.

conv:	low CLK			'Put clock line in starting state.

low CS			'Select ADC.

pulsout CLK, 1		'10 us clock pulse.

let data = 0			'Clear data.

for i = 1 to 8			'Eight data bits.

let data = data * 2		'Perform shift left.

pulsout CLK, 1		'10 us clock pulse.

let data = data + AD	'Put bit in LSB of data.

next				'Do it again.

high CS			'Deselect ADC when done.

return

'PROGRAM:	BUT_TEST.BAS

'

symbol	MODE	=	1

symbol	ST		=	2

b5 = 0

loop:

MD:	button MODE,0,255,255,b5,0,MD

debug "MODE depressed"

SE:	button ST,0,255,255,b5,0,SE

debug "ST depressed "

debug "-"

goto loop

'PROGRAM:	EE_TEST.BAS

'

Symbol	CLK 		= 6		' Clock line to pin 1.

Symbol	D		= pin7		' Destination of Shout; input to Shin

Symbol	D_N		= 7		' Pin # of D for "input" & "output"

Symbol	ReadEE	= %10100001	' EEPROM opcode for read.

Symbol	WriteEE	= %10100000	' EEPROM opcode for write.

Symbol	Dout		= b0		' Data for writes; data from reads.

Symbol	ctl		= b1

Symbol	i		= b10

symbol	data		= b9

Symbol	Addr		= w1		' 9-bit address for reads & writes.

Symbol	AddrReg	= w2		' Use w1 to shift out 12-bit sequences.

high CLK

high D_N

main:	for Addr=501 to 501

AddrReg=Addr

data=b4+1

debug data

gosub wr

next addr

data=0

for Addr=501 to 501

AddrReg=Addr

gosub rd

debug data

next Addr

end

wr:	low D_N

low CLK

b5=b5*2

Dout=WriteEE|b5

gosub out

Dout=b4

gosub out

Dout=data

gosub out

high CLK

high D_N

return

rd:	low D_N

low CLK

b5=b5*2

Dout=WriteEE|b5

gosub out

Dout=b4

gosub out

high CLK

pulsout D_N,10

low CLK

Dout=ReadEE|b5

gosub out

Dout=0

gosub in

data=Dout

high CLK

high D_N

return

out:	output D_N

for i=1 to 8

D=bit7

pulsout CLK,10

Dout=Dout*2

next i

low D_N

pulsout CLK,10

return

in:	input D_N

for i=1 to 8

Dout=Dout*2

high CLK

bit0=D

low CLK

next i

return