NEW RASP SERIES - VERSION 1.0

-Larry Curcio

This is the first version of an elaborate rasp program system,

based on drag coefficients updated at each meter-per-second of

speed. The present version is good for 1-stage, sub-sonic rockets

on vertical trajectories only; however I am working on off-

vertical trajectories, hypersonic drag coefficients, and multi-

stage rockets. The programs run on the IBM and on clones. They

may be modified to run on other systems.

The system has three parts: two programs and one set of files.

{===============================================================}

{===============================================================}

PART I: PROGRAM DATCOMFL.PAS (TURBO PASCAL 6.0 FOR IBM & CLONES)

Takes rocket size and shape characteristics, puts out a file (you

name it) with USAF DATCOM drag coefficients at every integral

velocity from 0 to 250 m/sec. All input length entries are in

centimeters. All input is written to the first five lines of the

output file as documentation. These lines must be discarded by

programs that use the file as input. Input proceeds as follows:

A) The program requests a title, which will be centered for

screen printing of input. The title will also appear in the first

line of the output file, to remind the user of the file's

identity and significance;

B) The program requests a critical Reynolds number (RCRIT). This

is the Reynolds number where flow over the body becomes

turbulent. Smaller numbers denote flow that becomes turbulent at

a lower speed; or, at a given intermediate speed, becomes

turbulent nearer the nose. RCRIT=0 denotes a uniformly turbulent

flow, which is a reasonable approximation for very large models,

for poorly trimmed models, and for most models with cone-shaped

noses that meet the body at a corner. In fact, RCRIT over the

body ( but not over the fins) is automatically set to zero when

either a cone-shaped nose cone is selected (see nose cone shape

below). An arbitrarily large RCRIT (say, 999999999) denotes

uniformly laminar flow. There is usually no way to determine

RCRIT, but typical values are between 100,000 and 500,000. Note

that the drag coefficient changes discontinuously when RCRIT is

reached;

C) Next, the program requests the body tube diameter. Enter the

figure in cm;

D) The program asks for the overall length of the rocket body.

This includes the nose cone, body tube and boat tail, but not

swept-back fins. Enter the figure in cm;

E) Nose Cone Length in cm;

F) Boat Tail Length in cm: if there is no boat tail, enter zero;

G) If a non-zero value is entered for the boat tail length, then

the Diameter At the Boat Tail Base is requested. If zero was

entered for boat tail length, this request is not made. A large

negative number appears in the corresponding position of the

input documentation record. Enter the figure in cm;

H) Average Fin Chord in cm: A chord is the length of a fin

parallel to the rocket body (or air flow). Average fin chord is

defined as that number which, when multiplied by the width of the

fin (see next section below), yields the fin's area on one side.

For trapezoidal fins this is the average of root and tip chord

lengths;

I) Vertical Fin Width in cm: This is the greatest width of the

fin, perpendicular to the rocket body. It does NOT include half

of the body diameter. That is, this is not a half span, merely

the width of the fin;

J) The length of the Root Chord in cm. This, in other words, is

the length of the fin-body joint;

K) Maximum Fin Thickness in cm: This is usually approximated as

the quoted thickness of the fin stock;

L) Number of fins;

M) Launch Lug Diameter in cm;

N) Lug Position: The lug may be positioned at the rocket Body

(B), or in a fin-body Joint (J). Specify one or the other;

O) Nose Cone Type: Specify one of the following: Ogive, Cone,

hemispher, or Ellipse. A parabolic nose cone or other blunt nose

is entered as an ellipse. Tangency (smooth contour at the nose

cone - body joint) is assumed in all except the cone, though.

Cones that are rounded near the body joint to create tangency are

entered as ogives. It is safe to assume that flow will become

turbulent at the sharp corner where a true cone joins the rocket

body. Rockets with conical nose cones are automatically assigned

RCRIT=0 over the body.

{===============================================================}

{===============================================================}

PART II: THRUST CURVE COLLECTION

These files all have a file type of .ENG. Those that begin with

'REAL' (as in REALB6.ENG) are actual thrust curves supplied by

Jack Kane of the NAR. Those that begin with NAR (as in NARC6.ENG)

are simulated curves with impulses and burning times from NAR

standard tables. ESTES curve simulations are calculated semi-

empirically from grain geometry measurements and propellant

characteristics, values for which were supplied by ESTES

Industries. Small adjustments were made to obtain exact NAR

statistics. Simulated curves for other manufacturers are splined

from illustrations and NAR statistics. The standard curves are

more likely representative of actual flight than the real curves,

since performance varies. All real thrust curves have at least

200 intervals, and all simulated curves have 500 intervals, so

that flight time calculations and other characteristics are

precise (if not accurate). The large number of intervals makes

the RASP program slow, however. Fast hardware is recommended.

{===============================================================}

{===============================================================}

PART III: PROGRAM RASPFL.BAS (Programmed in BASIC)

(Future versions will be in TURBO PASCAL).

This program operates as follows:

A) It accepts an output file name for interval-by-interval

altitude, velocity, and acceleration figures. A null entry

signifies no output file;

B) It accepts a launch mass in grams;

C) It accepts a body diameter in cm. If a diameter of 0 or less

is entered, then the drag figure (to be entered below) is assumed

to be a drag constant, or the drag file is assumed to contain

constants rather than coefficients. (No way to produce one of

these files is provided. Call it a quirk.) A drag CONSTANT is the

proportionality factor between drag force and squared velocity. A

drag COEFFICIENT is a dimensionless component of that factor;

D) It accepts a drag coefficient file, a drag coefficient, or a

drag constant. An entry beginning with an alpha character

signifies a file; an entry beginning with a numeric character

(including '.') signifies a coefficient or constant. Dragless

simulations may be performed by entering zero here;

E) It accepts a trim factor. Since your mileage may differ from

that predicted by the drag coefficient file, you may wish to use

the drag coefficient function's general shape, but increase its

value at each interval, compensating for deficits in trim. You

may also wish to change this factor in order to back-engineer

your simulation on the basis of flight time or measured altitude;

F) It accepts a thrust curve file name (without the .ENG

extension)

The program displays:

Cutoff Altitude;

Cutoff Velocity;

Maximum Altitude;

Flight Time;

Coast Time;

RASPFL.BAS uses a table constructed from the drag coefficient

file to supply its drag coefficient at each interval according to

speed. Speeds are rounded to the nearest integer m/sec for this

purpose. The program also depreciates air pressure with altitude.

The influence of drag is iterated 4 times at each interval, a

process that is virtually the equivalent of performing a Fehskens

- Malewicki integral at each interval. (The altitude correction

from the redundant lines is usually small, and the extra lines

may be commented out if speed is an issue.) If the rocket exceeds

the maximum speed in the drag coefficient table, then an abend

occurs. This prevents misinterpretation of trans-sonic

trajectories.

SPECULATIONS ON FUTURE UPDATES

I believe that RASPFL.BAS will be usable even after the trans-

sonic/hyper-sonic implementation. The size of the drag

coefficient array may have to be increased, however. That is the

parameter, DRAGSIZE, set early in the program.

I am looking into hypersonic DATCOM coefficients, and I am also

checking out the APAS aerodynamic estimation system, which was

used in the design of the SR71 Blackbird. That system may be

impractical for all I know. Stay tuned for further updates.

I intend to update my old SIMLABXL.PAS to use drag files and to

depreciate air density. Also, it is now compatible with current

versions of TURBO PASCAL. The resulting program will allow off-

vertical, multi-stage capability, and graphic output.

DISCLAIMER

This system should not be used to determine minimum engine

requirements for a rocket configuration. Computed altitudes are

based on the assumption of stability and no normal forces. A

rocket that rises to substantial altitudes in these simulations

may fly wildly, prang, or become a land shark in reality.

Simulations should never be relied on for decision making

involving potentially dangerous situations. ACTUAL PERFORMANCE

MAY DEVIATE RADICALLY FROM SIMULATED PREDICTIONS. No

responsibility is assumed for collateral damage.