PROGRAM DRAWCURV

Larry Curcio - 1989

lc2b+@andrew.cmu.edu

DRAWCURV is a general purpose program that draws a smooth curve

between data points, graphs the result, and allows data

modification. DRAWCURV can be used to interpolate values of

functions that are known by only a limited number of data points;

it will also estimate the areas under the functional curves, and

generate data files with interpolated values for up to 1000

intervals. These capabilities make DRAWCURV a good tool for

digitalizing printed line graphs.

This version runs in IBM PC BASIC or QUICK BASIC, and (I am told)

works fine in MAC BASIC with very minimal modification (E.G.; take

out the KEY OFF instruction).

For the model rocket enthusiast, DRAWCURV has at least two

noteworthy functions. First, it can be used to summarize exhaust

velocity/pressure curves for engine simulation programs.  Indeed,

the program was motivated by the need to digitalize the BARBER

team's exhaust velocity (actually specific impulse) curve for ESTES

black powder.  Second, the program makes it possible to digitalize

thrust-time curves for use in altitude programs. This latter

activity takes some practice, and hints are given below. The

results are far more accurate than line summaries, and DRAWCURV's

area-finding (integrating) capability assures that the total

impulse is correct.

The program works by the method of cubic splining. It fits separate

polynomials to the individual intervals such that the functions for

adjoining intervals have the same values, slopes and curvatures at

the known data points.

This version has two limitations:

1) The curve MUST begin at point 0,0.  <===========********************

2) Since curvatures are not supplied by the user, several points must

be included around areas of great curvature; otherwise the program

may produce, say, a convex curve when a concave curve was desired.

Sometimes the data must be augmented to eliminate ambiguities.

DRAWCURV uses an ASCII input file with two columns. The first column

has the y (Thrust) values. The second column has the corresponding

x (time) values). Although data must be entered outside the program,

DRAWCURV is easily integrated with commonly-used text editors so that

data may be manipulated on the fly.

INSTALLING DRAWCURV

The process of installing DRAWCURV is that of integrating it with

a text editor. When the user wishes to manipulate the input data

(s)he is allowed to drop back into DOS. (S)he does this by entering

the letter, "X," for an input file name. (S)he is then said to

enter the "DOS shell."  The program simultaneously writes a small

file onto the default drive, called, "X.BAT." X.BAT contains one

line: "SPLINBAT fn" where fn is the name of the last input file

that was read. If the user enters the letter, "X" again, the X.BAT

file will be executed. That will in turn execute the batfile,

"SPLINBAT.BAT," with the file name as a parameter.  It is up to the

user to supply a SPLINBAT.BAT file that will call a preferred text

editor and pick up the data file. For example, if the user prefers

EDLIN as an editor, SPLINBAT.BAT might contain the folowing lines:

EDLIN %1

ERASE X.BAT

EXIT

When the user enters the DOS shell, DRAWCURV will write out the

X.BAT file. When X is entered again, the system executes the first

line of SPLINBAT.BAT, with the file name

substituted for %1. This allows text manipulation with all the

facilities of EDLIN. After EDLIN is terminated, the second line of

SPLINBAT.BAT is executed. This erases the X.BAT file written by

DRAWCURV. Finally, the third line is executed: EXIT.  This command

takes the user immediately back to DRAWCURV at the file name entry

line. If the user then enters a "*" for a file name, the new

version of the data file will be read, and the program will begin

to generate the curve immediately.  The user will not be asked

about intervals. The last values will be used.

Of course the user can do other things in the DOS shell. (S)he can,

for example, look for data files or change directories. When it

comes time to return, to the program the user can enter the command,

"EXIT," manually. Also of course, the user may tailor the SPLINBAT.BAT

file to his or her specific needs. For instance, a text editor other

than EDLIN may be called, and the user may not wish to

automatically erase the X.BAT file, or to automatically return to

DRAWCURV.

SAVING FILES

To save a data file, answer 'y' to the file saving prompt, and 'y'

to the data file prompt. In other words, if you have splined a

thrust curve, the program will save the graphed curve to the

specified number of intervals.

To save a parameter file for the SPLINE program, answer 'y' to the

file saving prompt, and 'n' to the data file prompt.

SAMPLE INPUT FILE

0       0

0.048   107

0.20   111

0.50   85

1.509   34

2.182   0

This is a replica of an AEROTECH G55 curve. Save it as, DRAWTST.DAT. (Or

something else). Specify that name as the input file name.

Choose "Number of intervals"

Specify, say, 500 intervals (1000 is tops)

Note how total impulse (area under curve) varies with number of intervals

If you have the SPLINTAB.BAT file, come around to INPUT FILE entry again

Enter X for the input file - you will fall into the DOS shell

Enter X again - if SPLINTAB.BAT is working, you will find yourself in the

text editor. Play with the data.

Terminate the editor - If SPLINTAB.BAT has EXIT as the last line, you

are back in DRAWCURV - otherwise enter EXIT manually.

You should find yourself at the INPUT FILE prompt. Enter *, and you

will get the new version of your file, and the program will automatically

calculate the curve at the same number of intervals as befor.

If instead of *, you simply press <RETURN>, then the program keeps the

old version of the file, but allows you to respecify the number of

intervals.

A TECHNIQUE FOR LIFTING THRUST CURVES FROM ILLUSTRATIONS

This takes practice, but after a little frustration, you pick it up.

1) XEROX the illustration

2) Affix a piece of fine-grain graph paper

3) Draw axes on graph paper around illustration

4) Draw points on curve, remembering to make several CLOSE TOGETHER

around points of great curvature (e.g.; at thrust peaks and at the

end of the curve.) If the rear end of the curve ends discontinuously,

you may have to end the curve at the end of the sustaining region. The

next point, 0,0 can be added to the output file if desired. (Since the

function of a spline program is to draw a continuous curve, a

discontinuity is flummoxing.)

5) Enter the points, Y,X in a spreadsheet in any convenient units (e.g.;

the number of tics on the graph paper). Do the conversion to useful

units in the spreadsheet. It will save you A LOT of trouble.

6) Output the data from the spreadsheet as an ASCII file

7) Read into DRAWCURV and graph

8) Make adjustments (extra points around curves, usually), until image

and total impulse are correct

REMEMBER: A small error in thrust curve shape makes very little difference

altitude-wise (There are very good closed-form approximations that assume

CONSTANT AVERAGE thrust.). Worry more about errors in total impulse.