PROGRAMS BSAFEX.BAS, BSAFE-BX.BAS, SIMLAB-F.BAS

By Larry Curcio 1988

These programs produce burning surface/time curves and

thrust /time curves for seven (related) kinds of charge

(grain) configurations. The seven are:

1) Unbored charge, ignited in nozzle throat;

2) Hemispherical dimple;

3) Flat-topped cylindrical perforation

4) Round-topped cylindrical perforation;

5) Conic frustum perforation;

6) Round-topped frustum perforation; and

7) Conic perforation

The rounded tops are assumed to be spherical segments,

tangent to the walls of the perforation (bore).

In the programs, TYPE OF CAP, refers to the top of the

bore (rounded or flat).

The nozzle throat is assumed to have the same dimensions

as the bottom of the perforation. A bore with the same

bottom and top radii is a cylinder. A bore with a nozzle

throat (bottom) radius larger than the radius at the top

of the bore is a frustum. If the top of the bore has a

radius of zero, the configuration is a cone. A

configuration with equal top & bottom radii, and no bore

length is uncored. The radius of the throat must equal or

exceed that of the top of the bore.

Uncored configurations will display a peak (surprise

surprise). Constant thrust would result only if the uncored

charge were ignited along the entire cross-section of the

combustion chamber. When the charge is ignited only along

the nozzle throat area, it recedes to a dug-out shape that

resembles the top half of a jelly doughnut. The rounded

sides grow radially toward the walls of the chamber. (The

model of burning in these programs corresponds to the

propagation of a Huygens wave front.)

BSAFE stands for Burning Surface Area Thrust (Force)

Estimator. The BSAFE programs are just this. You enter

the configuration, burning time, mass and total impulse.

The program assumes constant burning rate and exhaust

velocity. The initial thrusts from these models are non-

zero, because the initial surface areas are non-zero.

It is assumed that the charge is molded to the inside of

the nozzle. If the nozzle has a convergent section, then

the rear end of the charge is affected. BSAFEX takes care

of this sort of thing. Most ESTES engines have flat

nozzles with a variable amount of bevel where clay meets

wall. The A8 and 1/2A6 engines have a large and rounded

bevel, so that the nozzle resembles a flat-bottom bowl.

BSAFE-BX takes care of this sort of nozzle. The radius

of the flat surface, referred to in the input routine is

the radius of the flat bowl bottom. This should equal or

exceed the throat radius, but the program will kludge it

if it doesn't. These nozzles are represented as though

the rounded sides of the bowl-shaped nozzles have quarter

circles for cross-sections. Actually, they do not.

Actually, the real nozzles are uneven, but this is a good

approximation.

SIMLAB-F is a full-blown thrust simulator. It is,

essentially, BSAFE-BX, with the black powder parameters

measured by Ed Brown at ESTES. The exhaust velocity curve

is from specific impulse measures done at ESTES. Exhaust

velocity is represented as a cubic spline in file

SPLINTAB.DAT. The file is a parameter file from program

DRAWCURV.DAT. Burning rate is represented as an exponential

function of pressure. Pressure is represented as an exponential

function of burning surface/nozzle throat area ratio. SIMLAB-F

computes its own burning time. It will use a default

density to compute propellant mass if you enter 999 for

this last quantity.

The simulations are not perfect, especially since they

do not capture erosive burning, but they are quite

close. Actual curves tend to vary in two ways. First,

since maximum thrust realization requires a perfectly

centered, perfectly straight bore, in a perfectly even

nozzle, actual maximum thrusts are variable. Real peaks

are often (though not always) ragged. Simulated peaks

are rounded, and they always peak at the theoretical

maximum. Secondly, engines with long bores often burn as

though the frustums were more oblique than they actually

are. Whether this is a sign of erosive burning or ignition

anomaly, the discrepancy disappears when a small extra amount

of angle is introduced into the simulated bore. Simulated

curves usually hug actual curves very closely.

SIMLAB-F requires the file, PROPPARM.SPN and SPLINTAB.DAT.

PROPPARM.SPN contains burning rate parameters, and default

propellant density (used when default propellant mass of 999

is chosen - recommended procedure). It also points to file

SPLINTAB.DAT, the exhaust velocity/pressure curve file, saved

as a parameter data file from program DRAWCURV.BAS.

The parameter files may be updated to represent any propellant

with an exponential burning rate function. Some propellants

require more than one such function - that is, there is sometimes

a kink in the burning-rate/pressure curve. This program allows for

one such kink (two such functions). Rules for coding parameters are

printed in PROPPARM.SPN. The limitation, here, is that only the

described configurations are allowed, and only flat or bowl-shaped

nozzles are allowed.

CAVEAT

These programs are not bullet-proof by any means. When used

properly, they yield good results; however they do not edit

for input that they are not designed to handle. Examples:

These programs are designed such that the configurations must

finish their radial burning before they burn through at the top

of the charge. IF THERE IS TOO LITTLE PROPELLANT AT THE TOP OF

THE CHARGE, TOTAL IMPULSE WILL BE INCORRECT. OTHER DISCREPANCIES

MAY ALSO RESULT.

The radius at the bottom of the bore (nozzle throat radius) MUST

be no larger than that at the top of the bore.

SAMPLE INPUT: (Approximate C-6)

COMBUSTION CHAMBER RADIUS   .635

RADIUS OF FLAT BOTTOM       .485

TYPE OF CAP                 1

NOZZLE THROAT RADIUS        .17

RADIUS AT TOP OF BORE       .16

LENGTH OF CHARGE            4.5

PROPELLANT MASS             999 (DEFAULT)

COMPLETE SET:

Set #1 (Full simulation):

SIMLAB-F.BAS

PROPPARM.SPN (Must be present in default directory)

SPLINTAB.DAT (Must be present in default directory)

DRAWCURV.BAS (Optional)

Set #2 :

BSAFE-BX.BAS (Flat and bowl-shaped nozzles)

BSAFEX.BAS (Flat and tapered nozzles)

Here are some dimensions for ESTES rocket engines

=========================================================================

CHARGE DIMENSIONS

ID       CHAMBER CHARGE      THROAT    BORE      FLAT BOTTOM

RADIUS  LENGTH      RADIUS    LENGTH    RADIUS

1/2A3    .508     1.2        .14224    0.46990   .40

1/2A6    .635     1.0        .19812    0.47752   .24

A3       .508     2.15       .14224    0.46990   .40

A8       .635     1.3        .19812    0.56388   .24

A10      .508     2.4        .17780    0.85344   .45

B4       .635     3.1        .19812    0.56388   .40

B6       .635     2.1        .16891    0.49530   .485

B8       .635     2.274      .1778     1.016     .60

C5       .635     4.5        .1778     1.016     .55

C6       .635     4.5        .16891    0.49530   .485

D12      .9144    4.1        .24130    0.71628   .80

Bores are all frustum-shaped, with an angle of about 1/2 degree.

Chamber Radius, throat and bore dimensions are courtesy of Ed Brown, an

engineer at ESTES Industries. According to Mr. Brown, charge lengths

are adjusted to yield the rated impulse, according to the variations

in performance with different shipments of black powder. Charge lengths

and burning times will vary accordingly. The charge lengths supplied

were obtained by direct measurement from dissected engines. Radii of

flat bottoms were also measured, but as they are uneven, these are only

approximate. The rounded parts of bowl nozzles are represented as having

quarter-circle cross-sections:

Cross-section of ESTES nozzle: (_ _) <- sides are represented as 1/4 circles.

flat bottom ^

The radius of the flat bottom is derived by taking the height of the measured

curved surface and subtracting it from the radius of the chamber. After

measurement, the charges were mixed with water and discarded. It is not clear,

unfortunately, that the personally measured dimensions are representative.

The curves produced by this program can be used with SIMLAB-XL. In this way,

one can design a rocket engine, using black powder, or a propellant with known

parameters, and try out various grain configurations. One can design an air

frame, check the CP/CG relationship, and with SIMLAB, derive a drag coefficient

(SIMLAB also uses the thrust curves). With SIMLAB-XL, one can effectively

fly the engine/air frame configuration with various payload weights, at

various angles.

The SIMLAB series is not intended as an aid to the construction of scratch

rockets (It wouldn't significantly reduce the myriad dangers of such a

foolish enterprise); rather it is intended as a riskless SUBSTITUTE. Note

that these programs have bombed many times, without a single casualty!

=========================================================================

Questions?

Send e-mail to lc2b+@andrew.cmu.edu

-Larry Curcio