Include:  <bios.h>

Syntax:   unsigned _bios_disk(unsigned service,

struct diskinfo_t *diskinfo);

service: _DISK_ALTERNATE	_DISK_BADTRACK	    _DISK_DIAGNOSTIC

_DISK_FORMATDRIVE	_DISK_FORMATTRACK   _DISK_INITIALIZE

_DISK_OPMODE	_DISK_RECALIBRATE   _DISK_READ

_DISK_READDDAM	_DISK_READID	    _DISK_RETRACT

_DISK_SEEK 	_DISK_SENSE	    _DISK_VERIFY

_DISK_WRITE	_DISK_WRITEDDAM

<sector> etc. means the field of the structure diskinfo_t in the

description below.

Constant		    The function of _bios_disk

_DISK_ALTERNATE	It set s alternative track. The alternative track

address is stored in the area pointed to by <buffer>.

It can be used when _CMD_HD is in <command>.

If one of the other symbol constants is in <command>,

it is error, but it does not return the value which shows

errors.

_DISK_BADTRACK	It formats bad tracks. Befer this you should set

alternative tracks by _DISK_ALTERNATE.

It can be used when _CMD_HD is in <command>.

_DISK_DIAGNOSTIC	It reads data into <buffer> for diagnostis.

It can be used when  _CMD_2HD or _CMD_2DD is

in <command>.

The result status is stored in the area pointed to

by <result>.

_DISK_FORMATDRIVE	It formats a disk. Interleave facter is set in

<data_len>. it can be used when _CMD_2D

or _CMD_HD is in <command>.

_DISK_FORMATTRACK	It formats one track.

The sector ID information written in the track is set

in the area pointed to by <buffer>. It can be used

when one of _CMD_2HD, _CMD_2DD or _CMD_HD

is in <command>.

_DISK_INITIALIZE	It initializes <drive>. It does not use the field <drive>

and <command> only. It can be used when one

of _CMD_2HD, _CMD_2DD, _CMD_2D or _CMD_HD

is in <command>. It is different from the case the

other symbol constant is specified, it does not return

the status information when it is error.

_DISK_OPMODE	It specifies the action when 640 K bytes of

floppy disk is accessed under 1 mega bytes interface

mode. It can be used when _CMD_2HD, _CMD_2DD

or _CMD_2D is in <command>.

_DISK_READ		It reads data from the <sector> of the <drive>

into <buffer> by <nsectors> sectors. It can be used

when _CMD_2HD, _CMD_2DD, _CMD_2D, _CMD_HD

is in <command>.

The result status information is in the area pointed

by <result>.

_DISK_READDDAM	It reads Deleted Data Address Marked data

into <buffer>. It can be used when _CMD_2HD or

_CMD_2DD is in <command>.

The result status information is in the area pointed to

by <result>.

_DISK_READID	It reads ID information into <result>. It can be used

when _CMD_2HD or _CMD_2DD is in <command>.

_DISK_RECALIBRATE	It seeks cylinder 0 of <drive>. It can be used when

_CMD_2HD, _CMD_2DD or _CMD_HD is in <command>.

_DISK_RETRACT	It retracts the head of disk. It canbe used

when _CMD_HD is in <command>.

_DISK_SEEK		It seeks the disk. It can be used when _CMD_2HD

or _CMD_2DD is in <command>.

The result status informaion is stored in the area pointed

to by <result>.

_DISK_SENSE 	It checks the status of <drive>. It uses the fields

<drive> and <command> only.

It can be used when _CMD_2HD, _CMD_2DD,

_CMD_2D or _CMD_HD is in <command>.

_DISK_VERIFY	It reads data from <sector> of <drive> by <nsectors>

sectors. It does not store the data into memory.

It can be used when _CMD_2HD, _CMD_2DD,

_CMD_2D or _CMD_HD is in <command>.

The result status information is stored in the area

pointed to by <result>.

_DISK_WRITE 	It writes data stored in the area pointed to

by <buffer> to <sector> of <drive> by <nsectors>

sectors. It can be used when CMD_2HD, _CMD_2DD,

_CMD_2D or _CMD_HD is in <command>

The result status information is stored in the area

pointed to by <result>.

_DISK_WRITEDDAM	It writes Deleted Data Address Marked data.

It can be used when _CMD_2HD or _CMD_2DD is

in <command>.

The result status information is stored in the area

pointed to by <result>.

Structure:

struct diskinfo_t

{

unsigned command;	/* Command	   */

unsigned drive; 	/* The drive number	*/

unsigned head;		/* The head number	 */

unsigned cylinder;	/* The cylinder number	   */

unsigned data_len;	/* The data length	   */

unsigned sector_len;	/* The sector length	     */

unsigned sector;	/* The start sector number   */

unsigned nsectors;	/* The number of sectors	 */

void _far *buffer;	/* The data buffer   */

void _far *result;	/* The result buffer */

};

Returns: The low byte is status information. The upper byte is zero if

no error, non zero if error.

_bios_disk routine makes disk access operations specified in

<service>. You must specify the information needed for access in

structure diskinfo_t type of <diskinfo>.

Please make caution for use of this function because they access disks

directly.

The argument <service> has one of the following symbol constants.

_DISK_ALTERNATE	_DISK_BADTRACK	    _DISK_DIAGNOSTIC

_DISK_FORMATDRIVE	_DISK_FORMATTRACK   _DISK_INITIALIZE

_DISK_OPMODE	_DISK_RECALIBRATE   _DISK_READ

_DISK_READDDAM	_DISK_READID	    _DISK_RETRACT

_DISK_SEEK		_DISK_SENSE	    _DISK_VERIFY

_DISK_WRITE 	_DISK_WRITEDDAM

Return Value

If this function finishes normally, it returns zero in the upper byte

and the status information in the lower byte. If error, the upper byte

is not zero and the lower byte is the status information. When

_DISK_INITIALIZE is specified in <service> and error occurs, the

lower byte is zero. When it is _DISK_ALTERNATE, error value is not

returned.

The contents of status information are different depending the

argument <service>. It depends on some of symbol constants in

the argument <service> if the result status is stored in the field

result of structure diskinfo_t type or not. The meaning of the status

or result information is the below.

Value     Status information or

result status information

0x00    Normal end/Ready status

0x10    Control mark/Inhibit to write

0x20    DMA boundary

0x30    End of cylinder

0x40    Check device

0x50    Over run

0x60    Not ready

0x70    Disable to write

0x80    error

0x90    Time out

0xA0    Data error ( Control information )

0xB0    Data error(Data)

0xC0    No data

0xD0    Bad Cylinder

0xE0    Cannot find address mark(Control information)

0xF0    Cannot find address mark(Data)

0x01    Double side diskette is set

0x08    Normal data

0x78    Invalid disk address

0x88    Direct access to alternative track

0xB8    Data error

0xC8    Seek error

0xD8    Cannot read alternative track

/* DISK.C illustrates low-level disk access using functions:

*	_bios_disk					     (DOS-only)

*

*/

#include <stdio.h>

#include <conio.h>

#include <bios.h>

#include <dos.h>

#include <stdlib.h>

char _far diskbuf[ 2048 ], result[ 16 ];

void main( int argc, char *argv[] )

{

unsigned status = 0, i;

struct diskinfo_t di;

unsigned char _far *p, linebuf[ 17 ];

if ( argc != 5 ) {

printf( "  SYNTAX: DISK <drivenumber> <head> <track> <sector>" );

exit( 1 );

}

di.command	  = _CMD_SEEK | _CMD_MF | _CMD_MT | _CMD_RETRY

| _CMD_2HD;

di.drive	  = atoi( argv[ 1 ] );

di.head	  = atoi( argv[ 2 ] );

di.cylinder   = atoi( argv[ 3 ] );

di.sector	  = atoi( argv[ 4 ] );

di.data_len   = 2048;

di.sector_len = 3;

di.buffer	  = diskbuf;

di.result	  = result;

/* Read disk */

status = _bios_disk( _DISK_READ, &di );

/* Display one sector. */

if ( status &= 0x00ff )

printf( "Error: 0x%.2x\n", status );

else {

for ( p = diskbuf, i = 0;

p < ( diskbuf + ( 128 << di.sector_len ) ); p++ ) {

linebuf[ i++ ] = ( *p >= 0x20 && *p <= 0x7f ) ? *p : '.';

printf( "%.2x ", *p );

if ( i == 16 ) {

linebuf[ i ] = '\0';

printf( " %16s\n", linebuf );

i = 0;

}

}

}

exit( 1 );

}

------------------------------------------------------------------------------

Include:  <bios.h>

Syntax:   unsigned _bios_equiplist(void);

Returns:  The 16-bit status word which indicate installed devices installed

_bios_equiplist  routine checks current installed hardware and peripheral devices.

Return value

It returns bit sequence like the following which shows installed devices.

Bit  Meaning

0	    Reserved

1	    If true (1), math co-processor is installed.

2	    Reserved

3-7     The number of Installed disk drive

(00000=0 drives, 00001=1 drive etc).

8	    If true ( 1 ), a mouse is installed.

9-11    The number of installed RS-232C serial port.

12	    If true ( 1 ), GP-IB is installed.

13	    If true ( 1 ), a sound board is installed.

14-15   The number of installed printers

/* SYSINFO.C illustrates miscellaneous DOS and BIOS status functions

* including:

*	_dos_getdrive	    _dos_setdrive	_dos_getdiskfree

*	_bios_memsize	    _bios_equiplist	_bios_printer

*							       (DOS-only)

* See DISK.C for another example of _dos_getdiskfree.

*

* Also illustrated:

*	union		    bitfield struct

*/

#include <dos.h>

#include <bios.h>

#include <conio.h>

#include <stdio.h>

#define LPT1 0

void main()

{

struct diskfree_t drvinfo;

unsigned drive, drivecount, memory, pstatus;

union {			     /* Access equipment either as:  */

unsigned u;		     /* unsigned or		    */

struct {		     /* bit fields		    */

unsigned reserved1:1;    /* Reserved		    */

unsigned coprocessor:1;  /* Coprocessor?		    */

unsigned reserved2:1;    /* Reserved		    */

unsigned disks:5;	     /* Drives			    */

unsigned mouse:1;	     /* Mouse			    */

unsigned comports:3;     /* Serial ports		    */

unsigned gpib:1;	     /* GP-IB board		    */

unsigned sound:1;	     /* Sound board		    */

unsigned printers:2;     /* Number of printers	    */

} bits;

} equip;

char    y[] = "YES", n[] = "NO";

/* Get current drive. */

_dos_getdrive( &drive );

printf( "Current drive:\t\t\t%c:\n", 'A' + drive - 1 );

/* Set drive to current drive in order to get number of drives. */

_dos_setdrive( drive, &drivecount );

_dos_getdiskfree( drive, &drvinfo );

printf( "Disk space free:\t\t%ld\n",

( long )drvinfo.avail_clusters *

drvinfo.sectors_per_cluster *

drvinfo.bytes_per_sector );

/* Get new drive and number of logical drives in system. */

_dos_getdrive( &drive );

printf( "Number of logical drives:\t%d\n", drivecount );

memory = _bios_memsize();

printf( "Memory:\t\t\t\t%dK\n", memory );

equip.u = _bios_equiplist();

printf( "Disk drives:\t\t\t%d\n", equip.bits.disks );

printf( "Coprocessor:\t\t\t%s\n", equip.bits.coprocessor ? y : n );

printf( "Mouse driver:\t\t\t%s\n", equip.bits.mouse ? y : n );

printf( "Serial ports:\t\t\t%d\n", equip.bits.comports );

printf( "GP-IB interface:\t\t%s\n", equip.bits.gpib ? y : n );

printf( "FM-sound:\t\t\t%s\n", equip.bits.sound ? y : n );

printf( "Number of printers:\t\t%d\n", equip.bits.printers );

/* Fail if any error bit is on, or if either operation bit is off. */

pstatus = _bios_printer( _PRN_STATUS, 0 );

if ( pstatus & 0x01 )

pstatus = 0;

else

pstatus = 1;

printf( "Printer available:\t\t%s\n", pstatus ? y : n );

}

------------------------------------------------------------------------------

Include:  <bios.h>

Syntax:   unsigned _bios_keybrd(unsigned service,

unsigned char *keytable);

service: _KEYBRD_INITIALIZE  _KEYBRD_READ	     _KEYBRD_READY

_KEYBRD_SENSE	 _KEYBRD_SHIFTSTATUS

Returns: The return value is different depending on specified the

value in <service>

_bios_keybrd	routine  executes keyboard operation specified in <service>

The argument  <service>  indicates one of the below symbol constants.

_KEYBRD_READ	   _KEYBRD_READY

_KEYBRD_SHIFTSTATUS    _KEYBRD_INITIALIZE

_KEYBRD_SENSE

Returned value

The return value is different depending on specified operation in

<service>. Please refer topics of each symbol constants.

Constant: _KEYBRD_INITIALIZE  _KEYBRD_READ	   _KEYBRD_READY

_KEYBRD_SENSE	_KEYBRD_SHIFTSTATUS

Summary: Constants of the argument <service> of _bios_keybrd.

One of the below symbol constants is specified.

Constants		     The function of _bios_keybrd

_KEYBRD_INITIALIZE	It initializes keyboard interface.

It initializes the memory area and the controller.

It enable to  input from keyboard. It returns no values.

_KEYBRD_READ	It reads key data code in the top of the key data buffer.

It waits for input if no data is in the key data buffer.

The lower byte of the read data is ASCII code

and the upper byte is the scan code

_KEYBRD_READY	It checks whether there are key code in key data buffer.

If key data is in the key data buffer, it operates as the same function as

_KEYBRD_READ. But the status of the buffer is not changed.

It returns zero if no data.

_KEYBRD_SENSE	It checks the status of key input and returns it

in the 16 bytes of area pointed to by the argument <keytable>.

The each byte respond to the code group.

_KEYBRD_SHIFTSTATUS It checks whether SHIFT, CAPS, KANA, GRPH,

CTRL keys are pressed.

The lower byte of the return value shows the pressed key.

Bit  Pressed key

0     SHIFT

1     CAPS

2     KANA

3     GRPH

4     CTRL

5-7   Not used

/* KEYBRD.C illustrates:

*	_bios_keybrd						(DOS-only)

*/

#include <bios.h>

#include <stdio.h>

#include <ctype.h>

void main()

{

unsigned key, shift, scan, ascii = 0;

int     kread = _KEYBRD_READ;

int     kready = _KEYBRD_READY;

int     kshiftstatus = _KEYBRD_SHIFTSTATUS;

/* Read and display keys until ESC is pressed. */

while ( ascii != 27 ) {

/*

* Drain any keys in the keyboard type-ahead buffer, then get

* the current key. If you want the last key typed rather

* than the key currently being typed, omit the initial loop.

*/

while ( _bios_keybrd( kready, 0 ) )

_bios_keybrd( kread, 0 );

key = _bios_keybrd( kread, 0 );

/* Get shift state. */

shift = _bios_keybrd( kshiftstatus, 0 );

/* Split key into scan and ascii parts. */

scan = key >> 8;

ascii = key & 0x00ff;

/* Categorize key. */

if ( ( ascii == 0 ) || ( ascii == 0xE0 ) )

printf( "ASCII: no\tChar: NA\t" );

else if ( ascii < 32 )

printf( "ASCII: yes\tChar: ^%c\t", ascii + '@' );

else

printf( "ASCII: yes\tChar: %c \t", ascii );

printf( "Code: %.2X\tScan: %.2X\t Shift: %.4X\n",

ascii, scan, shift );

}

}

------------------------------------------------------------------------------

Include:  <bios.h>

Syntax:   unsigned _bios_memsize(void);

Returns:  Memory block size (unit:1K bytes)

_bios_memsize checks the size of total main memory

Return value

It returns the size of the installed total main memory by 1 K block

unit. The maximum size is 640, which means 640 K bytes.

/* SYSINFO.C illustrates miscellaneous DOS and BIOS status functions

* including:

*	_dos_getdrive	    _dos_setdrive	_dos_getdiskfree

*	_bios_memsize	    _bios_equiplist	_bios_printer

*							       (DOS-only)

* See DISK.C for another example of _dos_getdiskfree.

*

* Also illustrated:

*	union		    bitfield struct

*/

#include <dos.h>

#include <bios.h>

#include <conio.h>

#include <stdio.h>

#define LPT1 0

void main()

{

struct diskfree_t drvinfo;

unsigned drive, drivecount, memory, pstatus;

union {			     /* Access equipment either as:  */

unsigned u;		     /* unsigned or		    */

struct {		     /* bit fields		    */

unsigned reserved1:1;    /* Reserved		    */

unsigned coprocessor:1;  /* Coprocessor?		    */

unsigned reserved2:1;    /* Reserved		    */

unsigned disks:5;	     /* Drives			    */

unsigned mouse:1;	     /* Mouse			    */

unsigned comports:3;     /* Serial ports		    */

unsigned gpib:1;	     /* GP-IB board		    */

unsigned sound:1;	     /* Sound board		    */

unsigned printers:2;     /* Number of printers	    */

} bits;

} equip;

char    y[] = "YES", n[] = "NO";

/* Get current drive. */

_dos_getdrive( &drive );

printf( "Current drive:\t\t\t%c:\n", 'A' + drive - 1 );

/* Set drive to current drive in order to get number of drives. */

_dos_setdrive( drive, &drivecount );

_dos_getdiskfree( drive, &drvinfo );

printf( "Disk space free:\t\t%ld\n",

( long )drvinfo.avail_clusters *

drvinfo.sectors_per_cluster *

drvinfo.bytes_per_sector );

/* Get new drive and number of logical drives in system. */

_dos_getdrive( &drive );

printf( "Number of logical drives:\t%d\n", drivecount );

memory = _bios_memsize();

printf( "Memory:\t\t\t\t%dK\n", memory );

equip.u = _bios_equiplist();

printf( "Disk drives:\t\t\t%d\n", equip.bits.disks );

printf( "Coprocessor:\t\t\t%s\n", equip.bits.coprocessor ? y : n );

printf( "Mouse driver:\t\t\t%s\n", equip.bits.mouse ? y : n );

printf( "Serial ports:\t\t\t%d\n", equip.bits.comports );

printf( "GP-IB interface:\t\t%s\n", equip.bits.gpib ? y : n );

printf( "FM-sound:\t\t\t%s\n", equip.bits.sound ? y : n );

printf( "Number of printers:\t\t%d\n", equip.bits.printers );

/* Fail if any error bit is on, or if either operation bit is off. */

pstatus = _bios_printer( _PRN_STATUS, 0 );

if ( pstatus & 0x01 )

pstatus = 0;

else

pstatus = 1;

printf( "Printer available:\t\t%s\n", pstatus ? y : n );

}

------------------------------------------------------------------------------

Include:  <bios.h>

Syntax:   unsigned _bios_printer(unsigned service,

unsigned char *databyte);

service: _PRN_INIT	_PRN_WRITE  _PRN_STRING  _PRN_STATUS

Returns:  It is different depending on <service>.

_bios_printer routine operates things specified by <service>.

One of the following symbol constants is specified in the argument  <service>.

_PRN_INIT	    _PRN_STATUS     _PRN_WRITE	    _PRN_STRING

Return value

It is different depending on <service>. Please refer topics of each symbol constants

Constant: _PRN_INIT	   _PRN_STATUS	   _PRN_STRING	   _PRN_WRITE

Summary:  Constants of the argument <service> of _bios_printer

One of the below symbol constants is specified.

Constants		     The function of _bios_printer

_PRN_INIT		It initializes the status area and the controller.

The bit 0 of the return value shows the following contents.

Value  Meaning

1   Enable to output data

0   Disable to output data

_PRN_WRITE		It putout one byte of data from the area pointed to

by the argument <data>.

The bit 0 of the return value shows the following contents.

Value  Meaning

1   Complete to output data / Enable to output data

0   Data is not output.

The bit 0 of the return value shows the following contents.

Value  Meaning

1   Time out / The data is not output.

0   Complete to output data

_PRN_STRING 	It output the data in the area pointed to

by the argument <data> till the first null character.

It returns zero if no errors and the number of characters

which is not output if errors.

_PRN_STATUS 	It gets the status information of the printer.

The bit 0 of the return value shows the following contents.

Value  Meaning

1   Enable to output data

0   Disable to output data

/* SYSINFO.C illustrates miscellaneous DOS and BIOS status functions

* including:

*	_dos_getdrive	    _dos_setdrive	_dos_getdiskfree

*	_bios_memsize	    _bios_equiplist	_bios_printer

*							       (DOS-only)

* See DISK.C for another example of _dos_getdiskfree.

*

* Also illustrated:

*	union		    bitfield struct

*/

#include <dos.h>

#include <bios.h>

#include <conio.h>

#include <stdio.h>

#define LPT1 0

void main()

{

struct diskfree_t drvinfo;

unsigned drive, drivecount, memory, pstatus;

union {			     /* Access equipment either as:  */

unsigned u;		     /* unsigned or		    */

struct {		     /* bit fields		    */

unsigned reserved1:1;    /* Reserved		    */

unsigned coprocessor:1;  /* Coprocessor?		    */

unsigned reserved2:1;    /* Reserved		    */

unsigned disks:5;	     /* Drives			    */

unsigned mouse:1;	     /* Mouse			    */

unsigned comports:3;     /* Serial ports		    */

unsigned gpib:1;	     /* GP-IB board		    */

unsigned sound:1;	     /* Sound board		    */

unsigned printers:2;     /* Number of printers	    */

} bits;

} equip;

char    y[] = "YES", n[] = "NO";

/* Get current drive. */

_dos_getdrive( &drive );

printf( "Current drive:\t\t\t%c:\n", 'A' + drive - 1 );

/* Set drive to current drive in order to get number of drives. */

_dos_setdrive( drive, &drivecount );

_dos_getdiskfree( drive, &drvinfo );

printf( "Disk space free:\t\t%ld\n",

( long )drvinfo.avail_clusters *

drvinfo.sectors_per_cluster *

drvinfo.bytes_per_sector );

/* Get new drive and number of logical drives in system. */

_dos_getdrive( &drive );

printf( "Number of logical drives:\t%d\n", drivecount );

memory = _bios_memsize();

printf( "Memory:\t\t\t\t%dK\n", memory );

equip.u = _bios_equiplist();

printf( "Disk drives:\t\t\t%d\n", equip.bits.disks );

printf( "Coprocessor:\t\t\t%s\n", equip.bits.coprocessor ? y : n );

printf( "Mouse driver:\t\t\t%s\n", equip.bits.mouse ? y : n );

printf( "Serial ports:\t\t\t%d\n", equip.bits.comports );

printf( "GP-IB interface:\t\t%s\n", equip.bits.gpib ? y : n );

printf( "FM-sound:\t\t\t%s\n", equip.bits.sound ? y : n );

printf( "Number of printers:\t\t%d\n", equip.bits.printers );

/* Fail if any error bit is on, or if either operation bit is off. */

pstatus = _bios_printer( _PRN_STATUS, 0 );

if ( pstatus & 0x01 )

pstatus = 0;

else

pstatus = 1;

printf( "Printer available:\t\t%s\n", pstatus ? y : n );

}

------------------------------------------------------------------------------

Include:  <bios.h>

Syntax:   unsigned _bios_serialcom(unsigned service,

unsigned port, struct com_t *serialcom);

service: _COM_COMMAND   _COM_GETDTL   _COM_INIT    _COM_INITX

_COM_RECEIVE   _COM_SEND	  _COM_STATUS

port:    _COM_CH1	_COM_CH2   _COM_CH3

Returns:  It returns zero if no error, non-zero if error.

It is different in the structure com_t depending on <service>.

_bios_serialcom routine operates serial ports by operations specified

in <service>.  The argument <port> shows the operated serial port which

is one of _COM_CH1,_COM_CH2 or _COM_CH3. The information for operation is

specified in <serialcom> of the structure com_t.

One of the following symbol constants is specified in <service>.

_COM_COMMAND   _COM_GETDTL	 _COM_INIT    _COM_INITX

_COM_RECEIVE   _COM_SEND	 _COM_STATUS

Return value

It returns zero if no errors, non-zero if errors. The value stored in

the structure com_t is different depending on <service>. Please refer

topics about symbol constants.

Constant: _COM_COMMAND    _COM_GETDTL	   _COM_INIT	   _COM_INITX

_COM_RECEIVE    _COM_SEND	    _COM_STATUS

Summary:  Constants of the argument <service> of _bios_serialcom

One of the below symbol constants is specified.

Constants		     The function of _bios_serialcom

_COM_COMMAND    It outputs the command information.

It uses the field command of the structure com_t only.

_COM_GETDTL     It gets the data length of the valid data in the receive buffer.

The received data length is stored in the field size of the structure com_t.

_COM_INIT	    It initializes the serial port specified in the argument <port>.

It uses all fields of the structure com_t.

When _COM_CH2 or _COM_CH3 is specified in the argument <port>,

the field baud of the structure com_t is ignored.

_COM_INITX	    It is the same function as _COM_INIT

except it can set X parameter ( flow control ).

_COM_RECEIVE    It received data. It stores the received data

in the field buffer+0, the status in buffer+1 if no errors.

_COM_SEND	    It sends one byte of data in the field buffer

of the structure com_t

_COM_STATUS     It gets the status of the controller and the system port.

It stores the information of the controller information

in the field buffer+0 and the system port information

in the field buffer+1 of the structure com_t.

The controller information is the following.

Bit  Meaning  In the case of 1  In the case of  0

0	 Send status	      Ready	  Busy

1	 Receive status       Ready	  Busy

2	 Send buffer	      Empty	  Full

3	 Parity error	      Error	  No error

4	 Overrun error	      Error	  No error

5	 Framing error	      Error	  No error

6	 Detect break status  Detect	  Not detect

7	 Data set ready       On	  Off

The system port information is is the following.

Bit  Meaning		   1	       0

0-4  Not used

5	 Received carrier detect  Not detect  Detect

6	 Send			  Disable     Enable

7	 Receive		  No	      Yes

Structure:

struct com_t

{

unsigned baud; 	 /* Baud rate		*/

unsigned mode; 	 /* Mode set	       */

unsigned command;	 /* Command specified	      */

unsigned tx_time;	 /* The period od time out to send */

unsigned rx_time;	 /* The period od time out to receive */

unsigned size; 	 /* Buffer size to receive   */

void _far *buffer;	 /* Pointer to receive buffer	      */

};

/* COM.C illustrates serial port access using function:

*	_bios_serialcom 					(DOS-only)

*/

#include <bios.h>

#include <stdio.h>

void main()

{

unsigned  status;

status = _bios_serialcom( _COM_STATUS, _COM_CH1, 0 );

/*

* Report status of serial port and test whether there is a

* responding device (such as a modem) for each. If data-set-ready

* and clear-to-send bits are set, a device is responding.

*/

printf( "COM status: %.4X\tActive: %s\n",

status, ( status & 0x0003 ) ? "YES" : "NO" );

}

------------------------------------------------------------------------------

Include:  <bios.h>

Syntax:   unsigned _bios_timeofday(unsigned service, char *timeval);

service: _TIME_GETCLOCK  _TIME_SETCLOCK

Returns:  It returns zero if no errors, non-zero if errors

_bios_timeofday routine gets or sets system clocks. _TIME_GETCLOCK or

_TIME_SETCLOCK is specified in the argument <service>.

the area pointed by  the argument  <timeval> specifies to get or set

the system clock. This area is assigned like the below.

timeval+0	Year (BCD 00-99)

timeval+1	The upper 4 bit is month   ( 1-12 hex)

The lower 4 bits is day ( 0-6 hex)

timeval+2	date (BCD 1-31)

timeval+3	hours (BCD 0-23)

timeval+4	minutes (BCD 0-59)

timeval+5	seconds (BCD 0-59)

Return value

It returns zero if no errors, non-zero if errors.

/* SIEVE.C illustrates timing functions including:

*	clock		difftime	    _bios_timeofday

*

* In addition to the timing use shown here, these functions can be

* used for delay loops as shown for the clock function

* in BEEP.C.

*/

#include <time.h>

#include <stdio.h>

#include <bios.h>

#define TICKPERSEC	18.2

int	mark[ 10000 ];

void main()

{

time_t   tstart, tend;    /* For difftime	      */

clock_t  cstart, cend;    /* For clock	      */

long     bstart, bend;    /* For _bios_timeofday  */

register int i, loop;

int      n, num, step;

/* Start timing. */

printf( "Working...\n" );

time( &tstart );  /* Use time and difftime for timing to seconds   */

cstart = clock(); /* Use clock for timing to hundredths of seconds */

#if defined( DOS )    /* Define DOS to use _bios_timeofday	       */

_bios_timeofday( _TIME_GETCLOCK, &bstart );

#endif

/* Do timed Sieve of Erotosthenes. */

for ( loop = 0; loop < 250; ++loop )

for ( num = 0, n = 3; n < 10000; n += 2 )

if ( !mark[ n ] ) {

step = 2 * n;

for ( i = 3 * n; i < 10000; i += step )

mark[ i ] = -1;

++num;

}

/*

* End timing and print results. Note that _bios_timeofday doesn't

* handle midnight rollover.

*/

time( &tend );

printf( "\ndifftime:\t\t%4.2f seconds to find %d primes 50 times\n",

difftime( tend, tstart ), num );

cend = clock();

printf( "\nclock:\t\t\t%4.2f seconds to find %d primes 50 times\n",

( ( float )cend - cstart ) / CLK_TCK, num );

#if defined( DOS )

_bios_timeofday( _TIME_GETCLOCK, &bend );

printf( "\n_bios_timeofday:\t%4.2f seconds",

( ( float )bend - bstart ) / TICKPERSEC );

printf( " to find %d primes 50 times\n", num );

#endif

}