To use PC/NFS 4.0 with an NE2000 card we set up our system as follows:

In drive C:, in a directory named LANMAN, we have the following files:

NE2000   DOS     16720 11-28-90   4:22p

NETBIND  EXE     15639 02-13-91   3:50a

NFS-NDIS SYS      3376 02-13-91   3:50a

PROTMAN  SYS     10649 02-13-91   3:50a

PROTOCOL INI       196 01-15-92   3:33p

PROTOCOL.INI contains the following:

[protocol manager]

drivername = PROTMAN$

[NE2000]

IOBASE = 0x320

INTERRUPT = 3

DRIVERNAME = MS2000$

[NFS-NDIS]

drivername = NFSLINK1

bindings   = NE2000

CONFIG.SYS contains the following:

DEVICE=c:\LANMAN\PROTMAN.SYS

DEVICE=c:\LANMAN\NE2000.DOS

DEVICEHIGH=c:\LANMAN\NFS-NDIS.SYS

DEVICEHIGH=c:\NFS\PCNFS.SYS

DEVICEHIGH=c:\NFS\SOCKDRV.SYS

AUTOEXEC.BAT contains the following:

C:\LANMAN\NETBIND

SET NFSDRIVE=C

SET NFSPATH=C:\NFS

LOADHIGH C:\NFS\PRT *

C:\NFS\NET INIT